| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Neighbor Discovery among Associated HR-MS |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Choong Hock Mar Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Text proposal to Sections 17.2.7.1 and 17.3.7.1.1 of the AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Neighbor Discovery among Associated HR-MS

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Choong Hock Mar

Institute for Infocomm Research (I2R)

Introduction

In May 2011 meeting, 16n TG has agreed on the principle to allow HR-MSs under coverage of a common infrastructure station (HR-BS/RS) to discover the neighborhood relationship. The text has been included in Sections 17.2.7.1 and 17.3.7.1.1 of the AWD [1]. In this contribution, we:

- propose that ranging preambles be used as PSA (predefined self-advertizing) signals;

- define control messages for the neighbor discovering process.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0009r1.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify section 17.27.1.1 and 17.3.7.1.1]

Modify section 17.2.7.1.1 and 17.3.7.1.1 as indicated

17.2.7.1.1 Neighbor Discovery between among associated Registered HR-MSs (Use case 1)

For associated registered HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) signals so that other HR-MSs can try to receive and verify their neighbor relationship. Either rRanging preambles or frame preambles (FFS) can shall be used as PSA signals.

The process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS sends HR-DCV-CMD message to schedules one or multiple associated registered HR-MSs to broadcast PSA sequences in assigned channels. Multiple HR-MSs may share the same PSA signal or the same channel PSA ranging sequence or the same assigned channel, but not both.

- In the same HR-DCV-CMD message. Tthe serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for PSA ranging signals.

- Each HR-MS that is scheduled to receive PSA ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs may report their measurements to the serving HR-BS/HR-RS using HR-DCV-REP message. Whether a receiving HR-MS shall report its measurements or not may be based on a threshold.

[Informative text] The serving HR-BS/HR-RS can determine neighbor topology based on reported measurements of transmitted PSA signals. The HR-MS is also able to construct a one hop neighborhood map that may be used for different purposes. How HR-BS/HR-RS/HR-MS construct neighbor topology is outside of the scope of this standard.

The transmission of HR-DCV-CMD can be described as follows. The HR-BS unicasts HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the HR-DCV-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

17.3.7.1.1 Neighbor Discovery between among associated Registered HR-MSs (Use case 1)

For associated registered HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) signals so that other HR-MSs can try to receive and verify their neighbor relationship. Either rRanging preambles or frame preambles (FFS) can shall be used as PSA signals.

The process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS sends HR-DCV-CMD message to schedules one or multiple associated registered HR-MSs to broadcast PSA ranging sequences in assigned channels. Multiple HR-MSs may share the same PSA signal or the same channel PSA ranging sequence or the same assigned channel, but not both.

- In the same HR-DCV-CMD message. Tthe serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for PSA ranging signals.

- Each HR-MS that is scheduled to receive PSA ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs may report their measurements to the serving HR-BS/HR-RS using HR-DCV-REP message. Whether a receiving HR-MS shall report its measurements or not may be based on a threshold.

[Informative text] The serving HR-BS/HR-RS can determine neighbor topology based on reported measurements of transmitted PSA signals. The HR-MS is also able to construct a one hop neighborhood map that may be used for different purposes. How HR-BS/HR-RS/HR-MS construct neighbor topology is outside of the scope of this standard.

The transmission of HR-DCV-CMD can be described as follows. The HR-BS unicasts HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the HR-DCV-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

[Remedy2: Add the following text at the end of 16.2.3 of 802.16n AWD.]

16.2.3.xx HR-DCV-CMD message

Table xx1 – HR-DCV-CMD message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. |  |
| Action | 1 | 0b0: HR-MS to transmit the ranging signal as instructed. 0b1: HR-MS to receive the ranging signal as instructed. |  |
| If (Action == 0b0){ |  |  |  |
| Transmit power level | 5 | Unsigned integer from 1 to 64 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| } |  |  |  |
| Else{ |  |  |  |
| Report mode | 2 | Indicate if the report mode is exclusive (all receiving HR-MS should send HR-DCV-REP message) or triggered by threshold. 0b00: exclusive reporting 0b01: triggered-based reporting 0b10: for peer-to-peer synchronization and no reporting 0b00: reserved. |  |
| If (Report mode == 0b01){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.3.xx HR-DCV-REP message

Table xx2 – HR-DCV-REP message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i = 0; i < Number of ranging codes to be reported; i++){ |  |  |  |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Received SINR | 4 | Indicates the received SINR of the ranging code. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Timing offset | 15 | Time offset, in units of Fs, of the received ranging signal, with respect to the frame timing of the HR-MS. MSB 1 bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1…16384] that corresponds to 0x0000 ~ 0x3FFF, respectively. | Optional |
| Frequency offset | 9 | Frequency offset, in units of 2% of the subcarrier spacing (ƒ), of the received ranging signal, with respect to the frequency of the HR-MS. MSB 1bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively | Optional |
| } |  |  |  |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.