| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Synchronization between HR-MSs Involving in HR-MS DC/FTN |  |
| Date Submitted | 2011-07-12 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Choong Hock Mar Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Text proposal to Sections 17.2.2.3 and 17.3.2.3 of the AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Synchronization between HR-MSs Involving in HR-MS DC/FTN

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Choong Hock Mar

Institute for Infocomm Research (I2R)

Introduction

In May 2011 meeting, the group has agreed on the principle to support synchronization between 2 HR-MSs involving in DC and FTN. The text has been included in Sections 17.2.2.3 and 17.3.2.3 of the AWD [1]. This contribution proposes some more details to the existing text.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0009r1.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

Modify section 17.2.2.3 and 17.3.2.3 as indicated

#### Synchronization between HR-MSs involving in HR-MS DC/FTN

This section describes the process of maintaining synchronization between two HR-MSs that communicate directly with each other under HR-MS DC and FTN. The process is employed after HR-MS DC/FTN has been setup, and therefore should be differentiated from the discovery process described in 17.3.7.1.2. Synchronization between HR-MSs is classified into two levels:

- The frame-level (first level) should allow HR-MSs to share a common understanding of frame and/or superframe timing and configuration.

- The symbol-level (second level) should allow reliable (i.e. received within cyclic prefix the appropriate reception threshold) bi-directional transmissions between HR-MSs.

Synchronization mechanisms are specified for three different use cases as follows.

        - Use case 1: Both HR-MSs are within the coverage of HR-BS/RS

- The first level of synchronization shall be maintained by common DL signaling (i.e. preambles, FCH, MAP…) from HR-BS/RS.

- The second level of synchronization can be achieved by HR-MSs exchanging ranging signals.

The following synchronization mechanisms are specifically designed enable HR-MS DC/FTN in uplink area of a framefor the case when HR-MS DC and FTN are scheduled in UL area of a frame.

Frame-level Synchronization:

When both HR-MSs are able to receive preambles and DL control signals from a common serving  HR-BS/HR-RS, they shall use these to achieve frame-level synchronization (with respect to HR-BS/HR-RS and between themselves).  When both HR-MSs involved in DC or FTN are within the coverage of HR-BS/HR-RS, frame-level synchronization means the HR-MSs acquire DL synchronization with the serving HR-BS/HR-RS and are able to achieve system configuration and control messages.

Symbol-level Synchronization:

When the HR-MS/HR-MS direct link is scheduled in a UL area of a frame, the transmitting HR-MS shall follow the same timing advance as has been adjusted and agreed with the serving HR-BS/HR-RS. This means the transmitting HR-MS shall time its direct transmissions as if these are normal UL transmissions toward the serving HR-BS/HR-RS.

It is the responsibility of the receiving HR-MS to adjust its receive timing to match the time of arrival (TOA) of the signal transmitted by the other HR-MS. This time adjustment shall be achieved by the serving HR-BS/HR-RS scheduling the HR-MSs to transmit ranging sequences to each other. Based on a received ranging sequence, an HR-MS can estimate and correct its time offset with the transmitting HR-MS. To facilitate this process, the serving HR-BS/HR-RS shall assign dedicated ranging sequences and ranging channels in UL area of a frame for HR-MS/HR-MS direct ranging.

To enhance bi-directional communication between HR-MSs, the serving HR-BS/HR-RS can allocate ranging resources to both involved HR-MSs in a single assignment. This allows the receiving HR-MS to transmit back a ranging sequence right after successfully processing the ranging sequence transmitted by the other HR-MS.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

        - Use case 2: one HR-MS is inside and the other is outside the coverage of HR-BS/RS

- The first level of synchronization shall be achieved by the inside-of-coverage HR-MS transmitting preamble and in some cases network configuration information toward the outside-of-coverage HR-MS. The locations of these control signals are TBD. HR-MS that are associated with an HR-BS transmit preambles at known locations. The preamble location and conditions for transmission are TBD.

- The second level of synchronization can be achieved by HR-MSs exchanging ranging signals.

The following synchronization mechanisms are specifically designed enable HR-MS DC/FTN in uplink area of a framefor the case when HR-MS DC and FTN are scheduled in UL area of a frame.

Frame-level Synchronization:

When two HR-MSs need to achieve frame-level synchronization and only one of them is within the coverage of and registered with an the serving HR-BS/HR-RS, the registered  inside-of-coverage HR-MS shall first acquires DL synchronization with the serving HR-BS/HR-RS (based on preambles and control messages from the serving HR-BS/HR-RS). The registered  inside-of-coverage HR-MS shall subsequently broadcast preambles and possibly network configuration information (NCI) for the outside-of-coverage HR-MS to co-synchronize.

The registered HR-MS shall transmit preambles either at the first OFDMA symbol or the last OFDMA symbol of the frame. The NCI shall be transmitted in an UL area. The location of the NCI, relative to the transmitted preambles, shall be determinable by the outside-of-coverage HR-MS

Symbol-level Synchronization:

Using the preambles and NCI transmitted by the inside-of-coverage HR-MS, the outside-of-coverage HR-MS shall adjust its timing to receive messages transmitted from the inside-of-coverage HR-MS. To further improve synchronization in this direction, the inside-of-coverage HR-MS can transmit ranging signal toward the outside-of-coverage HR-MS so that this node can estimate and correct its time/frequency offsets. Symbol-level synchronization in the opposite direction, i.e., from the outside-of-coverage of HR-MS toward the inside-of-coverage HR-MS shall be achieved by the outside-of-coverage HR-MS transmitting ranging signal toward the inside-of-coverage HR-MS. Upon processing the received ranging signal, the inside-of-coverage HR-MS can either adjust its own receive timing or request the outside-of-coverage HR-MS to adjust the transmit timing.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

#### Synchronization between HR-MSs involving in HR-MS DC/FTN

This section describes the process of maintaining synchronization between two HR-MSs that communicate directly with each other under HR-MS DC and FTN. The process is employed after HR-MS DC/FTN has been setup, and therefore should be differentiated from the discovery process described in 17.3.7.1.2. Synchronization between HR-MSs is classified into two levels:

- The frame-level (first level) should allow HR-MSs to share a common understanding of frame and/or superframe timing and configuration.

- The symbol-level (second level) should allow reliable (i.e. received within cyclic prefix the appropriate reception threshold) bi-directional transmissions between HR-MSs.

Synchronization mechanisms are specified for three different use cases as follows.

        - Use case 1: Both HR-MSs are within the coverage of HR-BS/RS

- The first level of synchronization shall be maintained by common DL signaling (i.e. preambles, FCH, MAP…) from HR-BS/RS.

- The second level of synchronization can be achieved by HR-MSs exchanging ranging signals.

The following synchronization mechanisms are specifically designed enable HR-MS DC/FTN in uplink area of a framefor the case when HR-MS DC and FTN are scheduled in UL area of a frame.

Frame-level Synchronization:

When both HR-MSs are able to receive PA/SA-pPreambles and DL control signals from a common serving HR-BS/HR-RS, they shall use these to achieve frame-level synchronization (with respect to HR-BS/HR-RS and between themselves).  When both HR-MSs involved in DC or FTN are within the coverage of HR-BS/HR-RS, frame-level synchronization means the HR-MSs acquire DL synchronization with the serving HR-BS/HR-RS and are able to achieve system configuration and control messages.

Symbol-level Synchronization:

When the HR-MS/HR-MS direct link is scheduled in a UL area of a frame, the transmitting HR-MS shall follow the same timing advance as has been adjusted and agreed with the serving HR-BS/HR-RS. This means the transmitting HR-MS shall time its direct transmissions as if these are normal UL transmissions toward the serving HR-BS/HR-RS.

It is the responsibility of the receiving HR-MS to adjust its receive timing to match the time of arrival (TOA) of the signal transmitted by the other HR-MS. This time adjustment shall be achieved by the serving HR-BS/HR-RS scheduling the HR-MSs to transmit ranging sequences to each other. Based on a received ranging sequence, an HR-MS can estimate and correct its time offset with the transmitting HR-MS. To facilitate this process, the serving HR-BS/HR-RS shall assign dedicated ranging sequences and ranging channels in UL area of a frame for HR-MS/HR-MS direct ranging.

To enhance bi-directional communication between HR-MSs, the serving HR-BS/HR-RS can allocate ranging resources to both involved HR-MSs in a single assignment. This allows the receiving HR-MS to transmit back a ranging sequence right after successfully processing the ranging sequence transmitted by the other HR-MS.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

        - Use case 2: one HR-MS is inside and the other is outside the coverage of HR-BS/RS

- The first level of synchronization shall be achieved by the inside-of-coverage HR-MS transmitting preamble and in some cases network configuration information toward the outside-of-coverage HR-MS. The locations of these control signals are TBD. HR-MS that are associated with an HR-BS transmit preambles at known locations. For AAI baseline the PA-Preamble alone or PA-Preamble and SA-Preamble may be used. The preamble location and conditions for transmission are TBD.

- The second level of synchronization can be achieved by HR-MSs exchanging ranging signals.

The following synchronization mechanisms are specifically designed enable HR-MS DC/FTN in uplink area of a framefor the case when HR-MS DC and FTN are scheduled in UL area of a frame.

Frame-level Synchronization:

When two HR-MSs need to achieve frame-level synchronization and only one of them is within the coverage of and registered with an the serving HR-BS/HR-RS, the registered  inside-of-coverage HR-MS shall first acquires DL synchronization with the serving HR-BS/HR-RS (based on PA/SA-pPreambles and control messages from the serving HR-BS/HR-RS). The registered  inside-of-coverage HR-MS shall subsequently broadcast preambles and possibly network configuration information (NCI) for the outside-of-coverage HR-MS to co-synchronize.

For 16m baseline, t The inside-of-coverage HR-MS shall transmit PA/SA preambles at the first OFDMA symbols of 2nd and 3rd frames within each superframe. The NCI shall be transmitted in an UL area. The location of the NCI, relative to the transmitted preambles, shall be determinable by the outside-of-coverage HR-MS.

Symbol-level Synchronization:

Using the preambles and NCI transmitted by the inside-of-coverage HR-MS, the outside-of-coverage HR-MS shall adjust its timing to receive messages transmitted from the inside-of-coverage HR-MS. To further improve synchronization in this direction, the inside-of-coverage HR-MS can transmit ranging signal toward the outside-of-coverage HR-MS so that this node can estimate and correct its time/frequency offsets. Symbol-level synchronization in the opposite direction, i.e., from the outside-of-coverage of HR-MS toward the inside-of-coverage HR-MS shall be achieved by the outside-of-coverage HR-MS transmitting ranging signal toward the inside-of-coverage HR-MS. Upon processing the received ranging signal, the inside-of-coverage HR-MS can either adjust its own receive timing or request the outside-of-coverage HR-MS to adjust the transmit timing.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.