| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PHY Control Operations for Talk-around Direct Communications |  |
| Date Submitted | 2011-09-14 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Eunkyung Kim, Miyoung Yun, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16n Amendment Working Document |  |
| Abstract | This provides AWD text proposals of PHY control operations for talk-around direct communication in 16n |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# PHY Control Operations for Talk-around Direct Communications

Sungcheol Chang, Seokki Kim, Eunkyung Kim, Miyoung Yun, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

The following set of contributions provide ETRI’s proposals for talk-around direct communication:

- Physical channels including synchronization, dedicated, and supplementary channels [1-3,6].

- Physical control operations of synchronization, dedicated channel allocation, HARQ, etc. [4]

- MAC Control procedures including one of PTT based one-way communication, and two hop direct communication[5].

This contribution provides AWD text proposals of MAC control operations for talk-around direct communication. HR-MSs are synchronized in a distributed way and the reference timing from HR-BS or GPS is propagated into the other HR-MSs using synchronization preambles that HR-MSs send. HR-MS reserves a dedicated channel in a distributed way that interference is minimized. Supplementary channel carries feedbacks from receiving HR-MS to sending HR-MS including CQI report, broadcast indicator, ACK, NAK, MCS Change Confirm.

HR-MSs listen to a synchronization channel and receive synchronization preambles in the synchronization channel. When an HR-MS decides to send synchronization preamble in a direct communication frame, the HR-MS sends synchronization preamble periodically with a period Tsych. The HR-MS selects the direct communication frame in which it is expected that there is no synchronization preamble. The HR-MS sends it when the priority of synchronization preamble from the HR-MS is greater than priorities of received synchronization preambles. The HR-MS sends it periodically with a period Tsych. Figure 1 shows that synchronization preambles are sent periodically and in a distributed way.

Figure 1 Periodic synchronization preambles from selected HR-MSs

For distributed allocation of dedicated channel, an HR-MS selects a dedicated channel and reserves it using exchange of ‘request to send’ (RTS) and ‘clear to send’ (CTS) messages. After the reservation, the HR-MS uses the dedicated channel continuously. Red colored circle is a coverage of signal that Tx HR-MS transmits. Any signal from HR-MS within blue colored circle may affect Rx HR-MS. To avoid interferences each other, Only Tx and Rx HR-MSs out of HR-MSs within two circles use the dedicated channel. Only when Tx and Rx HR-MSs do not receive any signal on the dedicated channel, Tx HR-MS sends RTS message and Rx HR-MS sends CTS message in response to it. If the Tx HR-MS receives the CTS message successfully, the Tx HR-MS use the dedicated channel continuously.

Figure 2 Signal coverage by Tx and Rx HR-MSs

In addition, CTS and RTS messages can piggyback MAC control messages including DM-LEST-REQ, DM-LEST-REQ, etc. ‘piggyback message indicator’ field is in CTS and RTS messages and if the field is set to 1 a MAC control message is piggybacked by CTS and RTS messages.

Supplementary channel carries CQI reports and feedbacks from receiving HR-MS to sending HR-MS. The feedbacks include ACK, NAK, MCS Change Confirm, and etc. The receiving HR-MS of a dedicated channel reports channel quality information (CQI) on corresponding supplementary channel periodically. Resources of a supplementary channel excluding CQI reports are used for sending feedbacks.

Figure 3 An example of periodic CQI reports and feedbacks

Supplementary channel carries a ranging sequence from receiving HR-MS to sending HR-MS. The receiving HR-MS of a dedicated channel sends the ranging sequence on corresponding supplementary channel periodically. The sending HR-MS receives the periodic ranging sequences and detects timing and frequency offsets from the references. If either timing offset or frequency offset are greater than threshold values, the sending HR-MS sends an event driven ranging sequence on a resource of dedicated channel in a DC frame to the receiving HR-MS. The event driven ranging procedure is FFS.

Figure 4 An example of periodic ranging sequences and feedbacks

When receiving CQI reports on supplementary channel periodically, the sending HR-MS decides a MCS level of data signal on corresponding dedicated channel and informs the receiving HR-MS of the MCS level using an MCS Change Command message. The receiving HR-MS sends MCS Change Confirm code on feedback channel in response to the MCS Change Command message. After receiving the MCS Change Confirm code, the sending HR-MS changes MCS level of a data signal on dedicated channel. The receiving HR-MS shall try to decode the data signal with new MCS level. If fails, the receiving HR-MS shall decode the data signal with old MCS level.

The sending HR-MS sends a data packet on dedicated channel in a DC frame. If the receiving HR-MS fails to decode the data packet successfully, it send NAK feedback within four DC frames. There are four types of NAK feedbacks and each type of NAK feedbacks indicates a frame number that data packets corrupts.

If the receiving HR-MS has no feedback, ACK feedback are sent on the supplementary channel. It indicates that the receiving HR-MS receives all the packets on the dedicated channel.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Add the following text proposal before Section 17.3.2.7 in the 802.16n AWD]

[note: The contents of physical layer:

17.3.2.6.2 Physical layer

17.3.2.6.2.1 Frame structure

17.3.2.6.2.2 Synchronization channel

17.3.2.6.2.3 Dedicated channel

17.3.2.6.2.4 Supplementary channel

17.3.2.6.2.5 Distributed synchronization

17.3.2.6.2.6 Distributed resource reservation

17.3.2.6.2.7 CQI report and link adaptation

17.3.2.6.2.8 HARQ]

17.3.2.6.2.5 Distributed synchronization

An HR-MS listens to a synchronization channel and receives synchronization preambles in the synchronization channel. The HR-MS selects a reference time among candidate values including synchronization preambles, GPS, and HR-BS preambles. When deciding to send a synchronization preamble on synchronization channel, the HR-MS sends it periodically with a period Tsync.

An HR-MS follows a priority rule to select a reference time in descending order of priority as the followings:

1) HR-BS preamble

2) GPS

3) Synchronization preamble that has smaller value of the ‘hop counter’ field and smaller value of the ‘reference signal strength’ field in Synchronization channel message IE. The reference source is either HR-BS or GPS.

4) Synchronization preamble that has smaller value of the ‘hop counter’ field and smaller value of the ‘reference signal strength’ field in Synchronization channel message IE. The reference source is HR-MS local clock.

5) HR-MS local clock.

The HR-MS calculates a sending reference time from the selected reference time, in which hop counter increases by one. If signal strength of received synchronization preamble of the selected reference time, is smaller than value of the ‘reference signal strength’ field in received synchronization preamble with the hop counter value of the sending reference time, the HR-MS sends a synchronization preamble periodically.

17.3.2.6.2.6 Distributed resource reservation

An HR-MS listens to all the supplementary channels and all the dedicated channels. If signal strength on a dedicated channel and corresponding supplementary channel is greater than a threshold, the HR-MS shall avoid using the dedicated channel and the corresponding channel.

An HR-MS selects a dedicated channel and reserves it using ‘request to send’ (RTS) and ‘clear to send’ (CTS) messages. The HR-MS sends RTS message on the dedicated channel and the RTS message includes a sending DCTID and a receiving address – DCTID or DCGID. If the receiving address is DCTID, the receiving HR-MS sends CTS message on the dedicated channel in response to RTS message. If the receiving address is DCGID, the receiving HR-MSs sends ACK on corresponding supplementary channel in response to RTS message and the sending HR-MS detects ACK signal. After exchange of RTS message and its response, the HR-MS sends packets on the dedicated channel continuously.

In addition, CTS and RTS messages can piggyback MAC control messages including DM-LEST-REQ, DM-LEST-RSP, etc. The ‘piggyback message indicator’ field is in CTS and RTS messages and if the field is set to 1 a MAC control message is piggybacked by CTS and RTS messages.

17.3.2.6.2.7 CQI report and link adaptation

The receiving HR-MS of a dedicated channel reports channel quality information (CQI) on corresponding supplementary channel periodically. When receiving CQI reports, the sending HR-MS decides a MCS level of data signal on the dedicated channel and informs the receiving HR-MS of the MCS level using an MCS Change Command message. The receiving HR-MS sends MCS Change Confirm code on feedback channel in response to the MCS Change Command message. After receiving the MCS Change Confirm code, the sending HR-MS changes MCS level of a data signal on dedicated channel. The receiving HR-MS shall try to decode the data signal with new MCS level. If fails, the receiving HR-MS shall decode the data signal with old MCS level.

17.3.2.6.2.8 HARQ

The sending HR-MS sends a data packet on dedicated channel in a DC frame. If the receiving HR-MS fails to decode the data packet successfully, it sends NAK feedback within four DC frames. There are four types of NAK feedbacks and each type of NAK feedbacks indicates a frame number that data packet corrupts. The sending HR-MS retransmits the data packet with NAK indicated frame number next frame.

If the receiving HR-MS has no feedback, ACK feedback are sent on the supplementary channel. It indicates that the receiving HR-MS receives all the packets on the dedicated channel.

## [Remedy2: Modify the following text in Section 17.3.2.6.1.3 in the 802.16n AWD]

17.3.2.6.1.3 MAC control messages

The peer-to-peer protocols of MAC layers in two HR-MSs communicate using the MAC control messages to perform the control plane function. MAC control messages shall be carried in a MAC PDU to be transported in a unicast control connection. In addition, the point-to-multipoint protocols of MAC layers in a multicast group of HR-MSs communicate using MAC control messages to perform the control plane functions. Table xxx lists the MAC control messages that shall be defined in the ASN.1 format.

Table 1216 – MAC control messages

| No. | Functional Areas | Message names | Message description | Secuirty | Connection |
| --- | --- | --- | --- | --- | --- |
| n+1 | Link Establishment | DM-LEST-REQ | Link Establishment Request |  | Unicast |
| n+2 | Link Establishment | DM-LEST-RSP | Link Establishment Response |  | Unicast |
| n+3 | Link Release | DM-LREL-REQ | Link Release Request |  | Unicast |
| n+4 | Link Release | DM-LREL-RSP | Link Release Response |  | Unicast |
| n+5 | Flow Management | DM-DSA-REQ | Dynamic Service Addition Request |  | Unicast |
| n+6 | Flow Management | DM-DSA-RSP | Dynamic Service Addition Response |  | Unicast |
| n+7 | Flow Management | DM-DSA-ACK | Dynamic Service Addition Acknowledgement |  | Unicast |
| n+8 | Flow Management | DM-DSC-REQ | Dynamic Service Change Request |  | Unicast |
| n+9 | Flow Management | DM-DSC-RSP | Dynamic Service Change Response |  | Unicast |
| n+10 | Flow Management | DM-DSC-ACK | Dynamic Service Change Acknowledgement |  | Unicast |
| n+11 | Flow Management | DM-DSD-REQ | Dynamic Service Deletion Request |  | Unicast |
| n+12 | Flow Management | DM-DSD-RSP | Dynamic Service Deletion Response |  | Unicast |
| n+13 | Flow Management | DM-DSD-ACK | Dynamic Service Deletion Acknowledgement |  | Unicast |
| n+14 | Measurement | DM-MES-REQ | Measurement Request |  | Unicast |
| n+15 | Measurement | DM-MES-RSP | Measurement Response |  | Unicast |
| n+16 | Measurement | DM-MES-REP | Measurement Report |  | Unicast |
| n+17 | Resource Management | DM-RCHG-REQ | Resource Change Request |  | Unicast |
| n+18 | Resource Management | DM-RCHG-RSP | Resource Change Response |  | Unicast |
| n+19 | Token Management | DM-TKN-REQ | Token Request |  | Unicast |
| n+20 | Token Management | DM-TKN-RSP | Token Response |  | Unicast |
| n+21 | Token Management | DM-TKN-HO | Token Handover |  | Unicast or Multicast |
| n+22 | Link Establishment | DM-LEST-CMD | Link Establishment Command |  | Multicast |
| n+23 | Link Release | DM-LREL-CMD | Link Release Command |  | Multicast |
| n+24 | Flow Management | DM-DSA-CMD | Dynamic Service Addition Command |  | Multicast |
| n+25 | Flow Management | DM-DSC-CMD | Dynamic Service Change Command |  | Multicast |
| n+26 | Flow Management | DM-DSD-CMD | Dynamic Service Delete Command |  | Multicast |
| n+27 | Measurement | DM-MES-CMD | Measurement Command |  | Multicast |
| n+28 | Resource Management | DM-RCHG-CMD | Resource Change Command |  | Multicast |
| n+29 | Token Management | DM-TKN-ADV | Token Advertisement |  | Multicast |
| n+17 | Resource allocation | RTS | Request To Send |  | Unicast or Multicast or broadcast |
| n+18 | Resource allocation | CTS | Clear To Send |  | Unicast |
| N+19 | MCS Change | DM-MCHG-CMD | MCS Change Command |  | Unicast |

## [Remedy3: Add the following text after Section 17.3.2.6.1.3.29 in the 802.16n AWD]

17.3.2.6.1.3.17 RTS

An HR-MS transmits a RTS message to reserve a dedicated channel in a distributed way.

Table 1246 — RTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| DCTID | 12 | Indicates a sending HR-MS address |  |
| DCTID or DCGID | 12 | Indicates a receiving HR-MS (Group) address. |  |
| Target address type | 1 | Indicates type of target address. 0: DCTID 1: DCGID |  |
| Piggyback message indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 6 |  |  |
| MAC control message | variable | MAC control messages in Table 1216 except RTS and CTS messages. | Present if Piggyback message indicator is set to 1 |

17.3.2.6.1.3.18CTS

An HR-MS transmits a CTS message in response to RTS message.

Table 1247 — CTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| DCTID | 12 | Indicates a sending HR-MS address |  |
| DCTID | 12 | Indicates a receiving HR-MS address. |  |
| Piggyback message indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 7 |  |  |
| MAC control message | variable | MAC control messages in Table 1216 except RTS and CTS messages. | Present if Piggyback message indicator is set to 1 |

17.3.2.6.1.3.19 DM-MCHG-CMD

An HR-MS transmits a DM-MCHG-CMD (MCS Change Command) message to change MCS level of data signals on the dedicated channel.

Table 1247 — DM-MCHG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| New MCS Level | 8 | Indicates new MCS level |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE C802.16n-11/0152, “ Frame Structure for Talk-around Direct Communications.”

[2] IEEE C802.16n-11/0153, “ Changes on Synchronization Channel for Talk-around Direct Communications.”

[3] IEEE C802.16n-11/0154, “ Supplementary Channel for Talk-around Direct Communications.”

[4] IEEE C802.16n-11/0155, “ PHY Control Operations for Talk-around Direct Communications.”

[5] IEEE C802.16n- 11/0156, “ MAC Control Operations for Talk-around Direct Communications.”

[6] IEEE C802.16n- 11/0160, “ Dedicated Channel for Talk-around Direct Communications.”