| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Messages format for alternative path in Path Discovery and Management |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for contributions for the 802.16n Amendment Working Draft |  |
| Abstract | This provides a proposal of establishment and maintenance of alternative path in 802.16n networks (Original document in Session #72: C80216n-11_0026.doc) |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Message formats for determining alternative path in path management of 802.16n-GRIDMAN networks

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# In this proposal, we propose a detail message formats for establishment and maintenance of alternative paths to support fast recovery in the event of intermediate node failures.

# Scenarios

We concentrate on the scenario mentioned in session 6.1.3.4 of SRD:

6.1.3.4 Path Management

HR-Network shall support establishment and maintenance of alternative paths to support fast recovery in the event of disruption; for example, encountering intermediate HR-MS failure or movement...

Our target scenario is as follow:

# Detail explanation about Neighbor discovery (user case 1)

Current AWD text:

17.2.7.1 HR-MS Neighbor Discovery

HR-MS neighbor discovery is a key functionality to enable other 16n features such as path discovery and management, HR-MS direct communications (with or without presence of infrastructure), and HR-MS forwarding to network. HR-MS neighbor discovery procedures are specified for two scenarios: i) when HR-MSs associated with a common super-ordinate station (HR-BS/RS or a coordinating HR-MS) attempt to discovery each other and ii) when an out-of-coverage HR-MS attempts to discover an HR-MS in order to connect through it to network infrastructure.

17.2.7.1.1 Neighbor Discovery among associated HR-MSs (Use Case 1)

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall ...

Which is perfectly enough to describe the user case 1 when associated HR-MSs (even for HR-MS 2 hops away) discover each other.

# Example scenario

An example of a HR-Network in operation, all HR-MS associate with HR-BS

# Text Proposal in IEEE 802.16n AWD

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the following text into Section 17.2.7 and 17.3.7 Path Discovery and Management of 802.16n AWD]

17.3.7.1 HR-MS Neighbor Discovery

HR-MS neighbor discovery is a key functionality to enable other 16n features such as path discovery and management, HR-MS direct communications (with or without presence of infrastructure), and HR-MS forwarding to network. HR-MS neighbor discovery procedures are specified for two scenarios: i) when HR-MSs associated with a common super-ordinate station (HR-BS/RS or a coordinating HR-MS) attempt to discovery each other and ii) when an out-of-coverage HR-MS attempts to discover an HR-MS in order to connect through it to network infrastructure.

To enable neighbor discovery among directly associated HR-MSs (use case 1), the super-ordinate station shall instruct these directly associated HR-MSs to transmit and receive predefined signals.

Table xx1 – AP-NBR-REQ message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Discovery mode requested | 2 | 0b00: Normal neighbor discovery 0b01: Discovery to prepare for BS determine alternative path 0b10: Discovery to prepare for MS determine alternative path 0b11: reserve | In the case of 0b10, HR-BS shall reply HR-MS in the message AP-NBR-REP. |
| } |  |  |  |

Table xx2 – AP-NBR-REP message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i = 0; i < Number of neighbor HR-MSs; i ++){ |  |  |  |
| Index | 4 | Index of the neighbor HR-MSs |  |
| SINR | 4 | Indicates the received SINR of the ranging code from neighbor HR-MSs. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Hop | 2 | Number of hop to HR-BS 00 1 hop 01 2 hop 10 3 hop 11 4 hop |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.