| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control messages to support HR-MS mode change to HR-BS |  |
| Date Submitted | 2011-09-20 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Seokjoo Shin Chosun University | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr |
| Re: | “IEEE 802.16n-11/0015,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | This contribution proposes more detailed procedure and new MAC control messages to support HR-MS mode change to HR-BS. |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC control messages to support HR-MS mode change to HR-BS

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Seokjoo Shin

Chosun University

- Introductions

IEEE 802.16n AWD describes a simple procedure of proactive operation for BS function of HR-MS in Section 17.3.1.3.1. In this contribution, we propose more detailed procedure and new MAC control messages to effectively support proactive operation in BS function of HR-MS.

Figure 1. Procedure to transfer HR-BS role to multimode HR-MS

If a superordinate HR-BS selects a target HR-MS capable of role changing to HR-BS, it shall transmit AAI-MMBS-REQ message to the target HR-MS. The AAI-MMBS-REQ message may include the PHY layer operational parameters. When the target HR-MS receives the AAI-MMBS-REQ message, it may initiate HR-BS mode. In response to the AAI-MMBS-REQ message, the target HR-MS shall transmit AAI-MMBS-RSP message to the superordinate HR-BS. If the target HR-MS accepts the request to change the mode to HR-BS, the superordinate HR-BS shall transmit AAI-MMBS-CMD message to the target HR-MS to inform the action time for starting as HR-BS mode. Upon expiring the action time, the target HR-MS starts the HR-BS mode operation and the superordinate HR-BS may stop the HR-BS role simultaneously, in order to avoid potential interference between the superordinate HR-BS and the target HR-MS. The superordinate HR-BS may transmit MS context information before sending AAI-MMBS-CMD message to the target HR-MS so that subordinate MSs connected to the superordinate HR-BS perform handover to the target HR-MS acting as HR-BS.

In addition, this contribution proposes new MAC control messages described below as follows:

- (new) AAI-MMBS-REQ message

A superordinate HR-BS shall transmit AAI-MMBS-REQ message to a target (multimode) HR-MS for the purpose as follows:

  - to request to transfer HR-BS role to the target HR-MS

  - to response or reject the unsolicited AAI-MMBS-RSP message by the multimode HR-MS

In addition, the superordinate HR-BS may utilize the AAI-MMBS-REQ message to configure the multimode HR-MS acting as HR-BS by sending its recommended BS PHY parameters.

- (new) AAI-MMBS-RSP message

A target HR-MS shall transmit AAI-MMBS-RSP message to the superordinate HR-MS for the purpose as follows:

  - to response or reject the AAI-MMBS-REQ message by the superordinate HR-MS

  - to request change of its mode to HR-BS in an unsolicited manner

- (new) AAI-MMBS-CMD message

A superordinate HR-BS shall transmit AAI-MMBS-CMD message to a target HR-MS for the purpose as follows:

  - to inform the action time for starting as HR-BS mode

The target HR-MS shall start HR-BS mode at the action time expires.

- References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0015, “802.16n Amendment Working Draft,” August 2011.

- Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adopt the following change in Section 17.3.1 in the 802.16n AWD.]

17.3.1 Multi-mode operation

17.3.1.1 Base station function for HR-MS

17.3.1.3.1 Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS' status information such as the battery level. The superordinate HR-BS may transmit AAI-MM-ADV message with trigger condition for which the subordinate HR-MSs capable of role changing to HR-BS shall report its status information. When the trigger condition is met, the The subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP MAC control message and/or AMS Battery Level Report header as described in 16.2.2.1.3.5. The triggering condition for reporting status information may be configured by the superordinate HR-BS.

After selecting the target HR-MS, the superordinate HR-BS requests the target HR-MS to change its mode to HR-BS by exchanging HRBS-REQ/RSP transmitting AAI-MMBS-REQ message. The AAI-MMBS-REQ message may include the PHY layer operational parameters recommended by the superordinate HR-BS. In response to the AAI-MMBS-REQ message, the target HR-MS shall transmit AAI-MMBS-RSP message to the superordinate HR-BS. If the target HR-MS accepts the request from the superordinate HR-BS to change the mode to HR-BS, the superordinate HR-BS may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters and the trigger conditions for operating as HR-BS. the superordinate HR-BS shall transmit AAI-MMBS-CMD message to the target HR-MS to inform the action time for starting as HR-BS mode. At the action time expires, the target HR-MS shall start HR-BS mode operation and subordinate MSs connected to the superordinate HR-BS may perform network reentry or handover to the target HR-MS acting as HR-BS. If there is potential interference between the superordinate HR-BS and the target HR-MS, the superordinate HR-BS may stop HR-BS role upon starting HR-BS mode operation of the target HR-MS.

[Remedy2: Add the following rows in Table 683, section 16.2.3 of 802.16n AWD.]

| xx | Multimode | AAI-MMBS-REQ | Multimode Base station request | Encrypted/ICV | Unicast |
| --- | --- | --- | --- | --- | --- |
| xx+1 | Multimode | AAI-MMBS-RSP | Multimode Base station response | Encrypted/ICV | Unicast |
| xx+2 | Multimode | AAI-MMBS-CONFIG-CMD | Multimode Base station command | Encrypted/ICV | Unicast |

[Remedy3: Add the following text into section 16.2.3 of 802.16n AWD.]

16.2.3 MAC Control messages

16.2.3.x1 AAI-MMBS-REQ

To request to transfer HR-BS role to a multimode HR-MS, the AAI-MMBS-REQ message shall be sent from superordinate HR-BS to a multimode HR-MS.

Table xxx—AAI-MMBS-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 2 | Used to indicate the purpose of this message. 0b00: Request to transfer HR-BS role to the multimode HR-MS 0b01: Response for the unsolicited AAI-MMBS-RSP message by the multimode HR-MS. 0b10: Reject for the unsolicited AAI- MMBS-RSP message by the multimode HR-MS. 0b11: Reserved. |  |
| BS configuration indication | 2 | Indicates whether the superordinate HR-BS provides the recommended PHY parameters to multimode HR-MS or not. 0b00: There are no recommended PHY parameters. 0b01: There are no recommended PHY parameters. But, must be equal to the PHY parameters of the superordinate HR-BS 0b10: There are recommended PHY parameters. 0b11: Reserved |  |
| Configuration deadline | 8 | The duration in units of 100 ms to which the superordinate HR-BS set the HR-BS mode configuration deadline. The time for the superordinate HR-BS to confirm the successful HR-BS mode initiation by receiving the AAI-MMBS-RSP message before its expiration. The multimode HR-MS shall send the AAI-MMBS-RSP message to be received by the HR-BS before the expiry. |  |
| If (BS configuration indication == 0b10) { |  |  |  |
| BSID | 48 | HR-BS ID |  |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with IEEE Std 802.16-2009 definition, with new MAC protocol version defined for AAI or HR-AAI. |  |
| CP time | 2 | CP time of the BS 0b00: 1/8 0b01: 1/16 0b10: 1/4 |  |
| For(j=0; j<N-Carrier-Info; j++) { |  |  |  |
| SA-PREAMBLE index | 10 |  |  |
| Physical carrier index | 6 | Refer to the physical carrier index in AAI-Global-CFG message |  |
| Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in ABS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| PGID | 16 | Indicates paging group identifier where the carrier belongs |  |
| S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS |  |
| SFH encoding format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier that transmits this AAI-NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell ABS, the SFH encoding format shall be either 0b00 or 0b01 or 0b11. |  |
| If (SFH encoding format == 0b00) { |  |  |  |
| SFH Subpkt 1 | variable | See Table 839 in IEEE Std 802.16m-2011. |  |
| SFH Subpkt 2 | variable | See Table 840 in IEEE Std 802.16m-2011. |  |
| SFH Subpkt 3 | variable | See Table 841 in IEEE Std 802.16m-2011. |  |
| } else if (SFH encoding format == 0b01¦¦0b11) { |  |  |  |
| Delta information | variable | Indicates the delta encoding, between the reference carrier and the current carrier where this message is transmitted if SFH encoding format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format=0b11. Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 839, Table 840, and Table 841 of IEEE Std 802.16m-2011, respectively. Each delta information attribute is optional. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.3.x2 AAI-MMBS-RSP

After configuring HR-BS mode or expiring HR-BS mode configuration deadline, the AAI-MMBS-RSP message shall be sent from multimode HR-MS to a superordinate HR-BS in response to an AAI-MMBS-REQ message. The multimode HR-MS may send the AAI- MMBS-RSP message in unsolicited manner with Response_Code = 0b00 (i.e., Request by multimode HR-MS in Unsolicited manner).

Table xxx—AAI-MMBS-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Response code | 2 | 0b00: Request by multimode HR-MS in Unsolicited manner 0b01: in response to the AAI-MMBS-REQ message to accept the request 0b10: in response to the AAI-MMBS-REQ message to allow to transmit subordinate station initiated AAI-MMBS-REQ after HR-BS configuration time expires 0b11: in response to the AAI-MMRS-REQ message to reject the request |  |

16.2.3.x3 AAI-MMBS -CMD

To inform the time for starting HR-BS role, the AAI-MMBS -CMD message shall be sent from superordinate HR-BS to the multimode HR-MS.

Table xxx—AAI-MMBS -CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action time | 4 | This is the wait time in units of 100 ms before the HR-MS starts to perform HR-BS mode. The multimode HR-MS shall start HR-BS mode at the action time expires. |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]