| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control messages to support HR-MS mode change to HR-BS |  |
| Date Submitted | 2011-09-20 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Seokjoo Shin Chosun University Wai-Leong Yeow Institute for Infocomm Research | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr wlyeow@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16n-11/0015,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | This contribution proposes more detailed procedure and new MAC control messages to support HR-MS mode change to HR-BS. |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC control messages to support HR-MS mode change to HR-BS

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Seokjoo Shin

Chosun University

Wai-Leong Yeow

Institute for Infocomm Research

- Introductions

IEEE 802.16n AWD describes a simple procedure of proactive operation for BS function of HR-MS in Section 17.3.1.3.1. In this contribution, we propose more detailed procedure and new MAC control messages to effectively support proactive operation in BS function of HR-MS.

Figure 1. Procedure to transfer HR-BS role to multimode HR-MS without trigger

If a superordinate HR-BS selects a target HR-MS capable of role changing to HR-BS, it shall transmit AAI-MMBS-REQ message to the target HR-MS. The AAI-MMBS-REQ message may include recommended PHY operational parameters. In response to the AAI-MMBS-REQ message, the target HR-MS shall transmit AAI-MMBS-RSP message to the superordinate HR-BS.

If the target HR-MS accepts the request to change the mode to HR-BS, the superordinate HR-BS shall transmit AAI-MMBS-CMD message to the target HR-MS to inform the action time or trigger conditions for starting as HR-BS mode.

If the action time for mode change are included in the AAI-MMBS-CMD message (Trigger type == 0b0), the target HR-MS starts the HR-BS mode operation upon expiring the action time. At the same time, the superordinate HR-BS may stop the HR-BS role in order to avoid potential interference from the target HR-MS. If trigger conditions for mode change are included in the AAI-MMBS-CMD message (Trigger type == 0b1), the target HR-MS shall start HR-BS role upon reaching the trigger conditions, as shown in Figure 2.

If the target HR-MS is configured with the same PHY operational parameters (BS configuration index == 0b01), subordinate MSs connected to the superordinate HR-BS may be connected to the target HR-MS acting as HR-BS without initial access or handover. In this case, the superordinate HR-BS shall transmit AAI-L2-XFER message which contains MS context information.

If the target HR-MS is configured with the recommended PHY operational parameters (BS configuration index == 0b10), subordinate MSs connected to the superordinate HR-BS may perform handover to the target HR-MS acting as HR-BS.

If necessary, the superordinate HR-BS may transmit AAI-L2-XFER message which contains MS context information before sending AAI-MMBS-CMD message to the target HR-MS.

Figure 2. Procedure to transfer HR-BS role to multimode HR-MS with trigger

In summary, this contribution proposes new MAC control messages described below as follows:

- AAI-MMBS-REQ message

A superordinate HR-BS shall transmit AAI-MMBS-REQ message to a target (multimode) HR-MS for the purpose as follows:

  - to request to transfer HR-BS role to the target HR-MS

  - to response or reject the unsolicited AAI-MMBS-RSP message by the multimode HR-MS

In addition, the superordinate HR-BS may utilize the AAI-MMBS-REQ message to configure the multimode HR-MS acting as HR-BS by sending its recommended BS PHY operational parameters.

- AAI-MMBS-RSP message

A target HR-MS shall transmit AAI-MMBS-RSP message to the superordinate HR-BS for the purpose as follows:

  - to response or reject the AAI-MMBS-REQ message by the superordinate HR-BS

  - to request change of its mode to HR-BS in an unsolicited manner

- AAI-MMBS-CMD message

A superordinate HR-BS shall transmit AAI-MMBS-CMD message to a target HR-MS for the purpose as follows:

  - to inform the action time or trigger conditions for starting as HR-BS mode

The target HR-MS shall start HR-BS mode after expiry of the action time or upon reaching the trigger conditions.

- References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0015, “802.16n Amendment Working Draft,” August 2011.

- Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adopt the following change in Section 17.3.1 in the 802.16n AWD.]

17.3.1 Multi-mode operation

17.3.1.1 Base station function for HR-MS

17.3.1.3.1 Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS' status information such as the battery level. The superordinate HR-BS may transmit AAI-MM-ADV message with trigger condition for which the subordinate HR-MSs capable of role changing to HR-BS shall report its status information. When the trigger condition is met, the The subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP MAC control message and/or AMS Battery Level Report header as described in 16.2.2.1.3.5. The triggering condition for reporting status information may be configured by the superordinate HR-BS.

After selecting the target HR-MS, the superordinate HR-BS requests the target HR-MS to change its mode to HR-BS by exchanging HRBS-REQ/RSP transmitting AAI-MMBS-REQ message. The AAI-MMBS-REQ message may include PHY operational parameters recommended by the superordinate HR-BS. If the target HR-MS accepts the request from the superordinate HR-BS to change the mode to HR-BS, the superordinate HR-BS may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters and the trigger conditions for operating as HR-BS. it shall transmit AAI-MMBS-RSP message to the superordinate HR-BS when it is ready to start HR-BS role.

After receiving the AAI-MMBS-RSP message, the superordinate HR-BS shall transmit AAI-MMBS-CMD message to the target HR-MS to inform the action time or trigger conditions for starting as HR-BS mode. If the action time for mode change are included in the AAI-MMBS-CMD message, the target HR-MS starts the HR-BS mode operation upon expiring the action time. At the same time, the superordinate HR-BS may stop the HR-BS role in order to avoid potential interference from the target HR-MS. If the trigger type is 0b1, trigger conditions for mode change shall be included in the AAI-MMBS-CMD message and the mode change to HR-BS role starts after trigger event. One possible use is to prepare against an unforeseeable SPOF event, e.g., a HR-BS failure. The trigger shall be canceled if the target HR-MS performs a handover to other infrastructure stations.

If handover of subordinate MSs connected to the superordinate HR-BS is necessary, the superordinate HR-BS may transmit AAI-L2-XFER message which contains MS context information before sending AAI-MMBS-CMD message to the target HR-MS.

[Remedy2: Add the following rows in Table 683, section 16.2.3 of 802.16n AWD.]

| xx | Multimode | AAI-MMBS-REQ | Multimode Base station request | Encrypted/ICV | Unicast |
| --- | --- | --- | --- | --- | --- |
| xx+1 | Multimode | AAI-MMBS-RSP | Multimode Base station response | Encrypted/ICV | Unicast |
| xx+2 | Multimode | AAI-MMBS-CMD | Multimode Base station command | Encrypted/ICV | Unicast |

[Remedy3: Add the following text into section 16.2.3 of 802.16n AWD.]

16.2.3 MAC Control messages

16.2.3.x1 AAI-MMBS-REQ

To request to transfer HR-BS role to a multimode HR-MS, the AAI-MMBS-REQ message shall be sent from superordinate HR-BS to a multimode HR-MS.

Table xxx—AAI-MMBS-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 2 | Used to indicate the purpose of this message. 0b00: Request to transfer HR-BS role to the multimode HR-MS 0b01: Response for the unsolicited AAI-MMBS-RSP message by the multimode HR-MS. 0b10: Reject for the unsolicited AAI- MMBS-RSP message by the multimode HR-MS. 0b11: Reserved. |  |
| BS configuration indication | 2 | Indicates whether the superordinate HR-BS provides the recommended PHY parameters to multimode HR-MS or not. 0b00: There are no recommended PHY parameters. 0b01: There are no recommended PHY parameters. But, must be equal to the PHY operational parameters of the superordinate HR-BS. 0b10: There are recommended PHY parameters. 0b11: Reserved |  |
| If (BS configuration indication == 0b01) { |  |  |  |
| S-SFH change count | 4 | Indicates the value of S-SFH change count of this HR-BS |  |
| Global carrier configuration change count | 3 | Indicates the HR-MS’s last received value of Global carrier configuration change count of the network. If set to 0, it implies that HR-MS never received a Global carrier configuration information. |  |
| SCD count | 4 | Indicates Configuration Change Count that is associated with the system configuration of the AAI-SCD. |  |
| Multicarrier configuration change count | 4 | The value is incremented by 1 upon each update |  |
| } else if (BS configuration indication == 0b10) { |  |  |  |
| FA index | 8 | FA index |  |
| BSID | 48 | HR-BS ID |  |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with IEEE Std 802.16-2009 definition, with new MAC protocol version defined 10 for AAI. |  |
| CP time | 2 | CP time of the HR-BS 0b00: 1/8 0b01: 1/16 0b10: 1/4 |  |
| For(j=0; j<N-Carrier-Info; j++) { |  |  |  |
| SA-PREAMBLE index | 10 |  |  |
| Physical carrier index | 6 | Refer to the physical carrier index in AAI-Global-CFG message |  |
| Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in HR-BS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| PGID | 16 | Indicates paging group identifier where the carrier belongs |  |
| S-SFH change count | 4 | Indicates the value of S-SFH change count of this HR-BS |  |
| SFH encoding format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier that transmits this AAI-MMBS-REQ message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell HR-BS, the SFH encoding format shall be either 0b00 or 0b01 or 0b11. |  |
| If (SFH encoding format == 0b00) { |  |  |  |
| SFH Subpkt 1 | variable | See Table 839 in IEEE Std 802.16m-2011. |  |
| SFH Subpkt 2 | variable | See Table 840 in IEEE Std 802.16m-2011. |  |
| SFH Subpkt 3 | variable | See Table 841 in IEEE Std 802.16m-2011. |  |
| } else if (SFH encoding format == 0b01¦¦0b11) { |  |  |  |
| Delta information | variable | Indicates the delta encoding, between the reference carrier and the current carrier where this message is transmitted if SFH encoding format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format=0b11. Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 839, Table 840, and Table 841 of IEEE Std 802.16m-2011, respectively. Each delta information attribute is optional. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.3.x2 AAI-MMBS-RSP

After configuring HR-BS mode or expiring predefined BS configuration deadline, the AAI-MMBS-RSP message shall be sent from multimode HR-MS to a superordinate HR-BS in response to an AAI-MMBS-REQ message. The multimode HR-MS may send the AAI- MMBS-RSP message in unsolicited manner with Response_Code = 0b00 (i.e., Request by multimode HR-MS in Unsolicited manner).

Table xxx—AAI-MMBS-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Response code | 2 | This indicates response type of AAI-MMBS-RSP message. 0b00: Request by multimode HR-MS in Unsolicited manner 0b01: Approval of AAI-MMBS-REQ 0b10: Rejection of AAI-MMBS-REQ 0b11: Reserved |  |
| If (Response code == 0b10) { |  |  |  |
| Operation | 2 | 0b00: Retransmit AAI-MMBS-REQ message (HR-BS mode of multimode HR-MS is not yet completely configured.) 0b01: Retransmit AAI-MMBS-REQ message after applying the system configuration change count (System configuration change count of multimode HR-MS is out of date.) 0b10: Cancel BS configuration request 0b11: Reserved |  |
| } |  |  |  |

16.2.3.x3 AAI-MMBS -CMD

To inform the time for starting HR-BS role, the AAI-MMBS-CMD message shall be sent from superordinate HR-BS to the multimode HR-MS. The trigger condition indicates when a HR-MS shall initiate a mode change to BS. See Table 775.

Table xxx—AAI-MMBS -CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Trigger type | 1 | 0b0: Start HR-BS mode after expiring action time 0b1: Start HR-BS mode upon reaching trigger conditions |  |
| If (Trigger type == 0b0) |  |  |  |
| Action time | 4 | This is the wait time in units of 100 ms before the HR-MS starts to perform HR-BS mode. The multimode HR-MS shall start HR-BS mode at the action time expires. |  |
| } else { |  |  |  |
| Trigger condition | variable | Triggers defined in table 775. |  |
| } |  |  |  |

[Remedy4: Modify the following text in Table 776 of 802.16n AWD.]

Table 776— Description of the trigger type/function/action

| Name | Size (Bits) | Value | Description |
| --- | --- | --- | --- |
| Type | 3 (MSB) | Trigger metric type: 0x0: CINR 0x1: RSSI 0x2: RTD 0x3: Number of consecutive PSFHs missed 0x4: RD 0x5–0x7: Reserved |  |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor ABS is greater than absolute value 0x2: Metric of neighbor ABS is less than absolute value 0x3: Metric of neighbor ABS is greater than S-ABS metric by relative value 0x4: Metric of neighbor ABS is less than S-ABS metric by relative value 0x5: Metric of S-ABS greater than absolute value 0x6: Metric of S-ABS less than absolute value 0x7: (For AMS in CA mode): Number of neighbor ABS’s carriers (whose CINR/RSSI is greater than absolute value) is higher than threshold value. The threshold value for the “Number of neighbor ABS’s carriers” can be AMS specific or ABS specific. (For AMS specific, the threshold value is defined as the number of AMS’s active carriers. For ABS specific, the threshold value is configured by the S-ABS and signaled through AAI-MC-ADV.) | Function 0x1-0x4 not applicable for RTD/RD trigger metric When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from –32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5, 0x6, or 0x07, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as –103.75 dBm and 0xFF is interpreted as –40 dBm Type 0x3 can only be used together with function 0x5 or function 0x6. Function 0x7 can only be used with Action 0x1 and applicable for AMS in CA mode. Non-CA mode AMS shall ignore this trigger. |
| Action | 3 (LSB) | Action performed upon trigger condition is satisfied: 0x0: Reserved 0x1: Respond on trigger with AAI-SCN-REP 0x2: Respond on trigger with AAI-HO-REQ 0x3: Respond on trigger with AAI-SCN-REQ 0x4: Declare ABS unreachable: 0x5: Cancel HO 0x6 and 0x7: Reserved 0x6: Initiate mode change 0x7: Reserved | Actions 0x3 and 0x6 applies apply only to Functions 0x5 and 0x6. Action 0x4: If this ABS is the SABS (meaning the AMS is unable to maintain communication with the ABS), AMS sends AAI-HO-IND with HO Event Code 0b10 to the S-ABS and proceeds as specified in 16.2.6.3.4. If this ABS is a TABS, the AMS needs not take immediate action when this trigger condition is met for a single ABS. The AMS shall act only when this condition is met for all T-ABSs included in AAI-HOCMD during HO execution. The specific actions are described in 16.2.6.3.4. |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]