| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Rename Multicast Key and Parameters for IEEE 802.16n HR-Network |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Rename Multicast Key and Parameters for IEEE 802.16n HR-Network

Joseph Chee Ming Teo, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Zheng Shoukang

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Multicast Authentication Key (MAK)

# The current IEEE 802.16n AWD [1] specifies a key hierarchy for secured multicast operation, which is similar to the key hierarchy defined in IEEE 802.16m-2011 [2]. In IEEE 802.16m-2011, the Authentication Key (AK) derives subsequent keys that are used for MAC verification and traffic encryption. The 802.16n AWD uses the same key hierarchy with the root key named as the Multicast Authentication Key (MAK).

# However, in IEEE 802.16m-2011, the AK is derived from the Primary Master Key (PMK), which is in turn derived from the Master Session Key (MSK) obtained through the EAP transfer protocol at the higher layer. Furthermore, the AK is obtained from a key agreement procedure following the EAP transfer protocol or re-authentication.

# The case is different in 802.16n where the MAK is a pre-established shared key among an HR-BS and a group of HR-MSs in an HR multicast group. There is no key agreement procedure and the MAK is not a derived key. In fact, the MAK operates as a master key in the multicast operation.

# To avoid confusion and to improve semantics, we propose to rename the MAK to Multicast Master Key (MMK).

# 2. Parameter Names

The AAI Security defined in IEEE 802.16m-2011 [2] specifies for secured unicast operations on both UL and DL. Since multicast is only defined for DL, we may omit subscripts indicating DL, i.e., MCMAC_KEY_D, MCMAC_PN_D and MTEKDLE.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Adopt the following text in the 802.16n AWD Document (C802.16x-xx/xxxx)]

[Change section 17.3.10.2 as indicated:]

17.3.10.2 Security Procedure for Multicast Operation

PKMv3 as described in 16.2.5.2 provides HR-stations with strong protection from theft of service by encrypting connections between HR-MSs and HR-BSs.

PKMv3 also shall provide HR-stations with strong protection from theft of service by encrypting multicast connections between HR-MSs and HR-BSs, as defined in this subsection.

If a DL multicast connection is to be encrypted, each HR-MS participating in the connection shall have an additional security association (SA) (i.e., multicast SA), allowing that connection to be encrypted using keys that are independent of those used for other encrypted transmissions between HR-MSs and the HR-BS.

Similar to unicast key management, multicast traffic can be encrypted using multicast specific key management based on PMKv3 as described in Figure aaa. Multicast CMAC (MCMAC) key and Multicast TEK (MTEK) are derived from Multicast AK (MAK) Multicast Master Key(MMK). MAKMMK is a pre-established shared key among an HR-BS and a group of HR-MSs in an HR multicast group.

Figure 921 – MCMAC Key and MTEK Key derivation from MAKMMK

Shared security association (i.e., Multicast Security Association; MSA) is an SA for the multicast transport/control flow and it provides keying material. Security key related to parameter to support multicast and the context is secured till the key expires.

###### 17.3.10.2.1 Security context for multicast communication

The multicast security context is a set of parameters linked to a key in each hierarchy that defines the scope while the key usage is considered to be secure.

Examples of these parameters are key lifetime and counters ensuring the same encryption will not be used more than once. When the context of the key expires, a new key should be obtained to continue working. The purpose of this sub clause is to define the context that belongs to each key, how it is obtained and the scope of its usage.

17.3.10.2.1.1 MAKMMK context

The MAKMMK context includes all parameters associated with the MAKMMK. This context is created whenever a new MAKMMK is derived.

This context shall be deleted whenever the MAKMMK is no longer valid or used.

The MAKMMK context is described in Table 1251.

Table 1251 – The MAKMMK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MAKMMK | 160 | Shared by HR-MSs in a multicast group |
| MAKMMK Lifetime | 32 | MAKMMK Lifetime |
| MAKMMKID | 64 | Identifies the authorization master key. |
| MAKMMK_COUNT | 16 | A value used to derive the MCMAC key and MTEK |
| MulticastGrpID | 16 | Identifies the Multicast Group |
| MCMAC_KEY_D | 128 | The key which is used for signing DL MAC control messages. |
| MCMAC_PN_D | 24 | Used to avoid DL replay attack on the control connection before this expires, reauthorization is needed. The initial value of MCMAC_PN_D is zero and the value of MCMAC_PN_D is reset to zero whenever MAKMMK_COUNT is increased. |
| Next available counter_MTEK | 16 | The counter value to be used in next MTEK derivation, after derivation this is increased by 1. |

17.3.10.2.1.2 MSA context

The MSA context is the set of parameters managed by each MSA in order to ensure MTEK management and usage in secure way.

The MSA context holds MTEK context and additional information that belongs to the MSA itself.

17.3.10.2.1.2.1 MTEK context

The MTEK context includes all relevant parameters of a single MTEK and is described in Table 1252.

Table 1252 – The MTEK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MTEK | 128 | Key used for encryption or decryption of MAC PDUs from FIDs associated with the corresponding MSA |
| MEKS | 2 | Encryption key sequence number |
| COUNTER_MTEK | 16 | The counter value used to derive this MTEK |
| MTEK lifetime | 32 | MTEK lifetime |
| MTEK_PN_D | 22 | The PN used for encrypting DL packets. After each MAC PDU transmission, the value shall be increased by 1. (0x000000-0x1FFFFF) |
| PN Window Size | As negotiated in key agreement | The receiver shall track the PNs received inside PN window |

17.3.10.2.1.2.2 MSA context

The MSA context is described in Table 1253.

Table 1253 – The MSA context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MSAID | 8 | The identifier of this MSA, which describes the applied en/ decryption method and MTEK contexts. |
| MTEKDLE context | Sizeof(MTEK Context) | MTEK context used for downlink encryption and decryption. |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]