| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose Text for HR-MS Direct Communication with Supporting from Infrastructure Station |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Propose Text for HR-MS Direct Communication with Supporting from Infrastructure Station

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# 2. Summary

# 3. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2 Direct communication between HR-MSs

17.2.2.1 General Description

In HR-MS direct communication, data packets are exchanged between two HR-MSs directly or by passing through another HR-MS. Tthe two communicating HR-MSs are the source and the sink of data. The data packets are passed from upper layers to MAC at the source HR-MS and back to upper layers at the sink HR-MS. Data packets are exchanged between the two HR-MSs directly or by passing through another HR-MS.

HR-MS direct communication is applicable when 1) the two HR-MSs are in coverage of and are directly associated to an HR infrastructure station; 2) one HR-MS is in coverage of and directly associated to an HR infrastructure station, while the other HR-MS is out of coverage of any HR infrastructure stations; 3) the two HR-MSs are out of coverage of any HR infrastructure stations.

Resource for HR-MS direct communication can be allocated by the HR infrastructure station for cases (1) and (2). The resource can be allocated in both DL and UL subframes.

For case-3, direct communications between HR-MSs shall satisfy:

- When HR-MSs are out of coverage of any HR infrastructure stations, Tthe operation of HR-MSs shall not interfere with any existing infrastructure stations. When HR-MS cannot receive any BS preamble from any infrastructure station and HR-MS direct communication without infrastructure is permitted by device configuration, HR-MSs are allowed to communicate with each other in the same band without getting permission from infrastructure stations.

- A Coordinator is selected for the coordination of transmission among HR-MSs.  Until a coordinator is selected, an HR-MS is only allowed to transmit signals necessary to enable coordinator selection. To avoid collisions among HR-MSs in coordinator selection, the HR-MS follow a collision avoidance procedure. The procedure is defined in 17.2.2.5.

- A coordinator shall function as a simplified HR-BS except it may not support handover. How to select a coordinator among HR-MSs shall follow the operation described in TBD. The selection of coordinator is up to the decision of users.

- A coordinator supports the following topologies:

- HR-MS linked to the coordinator and the pair is the source and sink of data. This topology is implemented through the local source and sink capability of the HR-MS.

- Two HR-MS linked to the coordinator and the two HR-MS are the source and sink of data. This topology is implemented through the local forwarding capability of the HR-BS.

- A forwarding HR-MS forwards data of a forwarded HR-MS to the coordinator. This topology is implemented through the HR-BS capability to support HR-MS forwarding operation.

- Two HR-MS are linked (DC) and are the source and sink of data to each other under the control of the coordinator. This topology is implemented through the HR-BS ability to support DC between its subordinates.

- The coordinator and any HR-MS that are communicating through the coordinator shall continue cell search operation and shall cease DC operation as soon as the criteria for DC and prevention of interference above are not met.

HR-MS direct communication using distributed resource allocation among nearby HR-MSs, that is called talk-around direct communication, is described in 17.2.2.6.

17.3.2 Direct communication between HR-MSs

17.3.2.1 General Description

In HR-MS direct communication, data packets are exchanged between two HR-MSs directly or by passing through another HR-MS. Tthe two communicating HR-MSs are the source and the sink of data. The data packets are passed from upper layers to MAC at the source HR-MS and back to upper layers at the sink HR-MS. Data packets are exchanged between the two HR-MSs directly or by passing through another HR-MS.

HR-MS direct communication is applicable when 1) the two HR-MSs are in coverage of and are directly associated to an HR infrastructure station; 2) one HR-MS is in coverage of and directly associated to an HR infrastructure station, while the other HR-MS is out of coverage of any HR infrastructure stations; 3) the two HR-MSs are out of coverage of any HR infrastructure stations.

Resource for HR-MS direct communication can be allocated by the HR infrastructure station for cases (1) and (2). The resource can be allocated in both DL and UL subframes.

For case-3, direct communications between HR-MSs shall satisfy:

- When HR-MSs are out of coverage of any HR infrastructure stations, Tthe operation of HR-MSs shall not interfere with any existing infrastructure stations. When HR-MS cannot receive any BS preamble from any infrastructure station and HR-MS direct communication without infrastructure is permitted by device configuration, HR-MSs are allowed to communicate with each other in the same band without getting permission from infrastructure stations.

- A Coordinator is selected for the coordination of transmission among HR-MSs.  Until a coordinator is selected, an HR-MS is only allowed to transmit signals necessary to enable coordinator selection. To avoid collisions among HR-MSs in coordinator selection, the HR-MS follow a collision avoidance procedure. The procedure is defined in 17.2.2.5.

- A coordinator shall function as a simplified HR-BS except it may not support handover. How to select a coordinator among HR-MSs shall follow the operation described in TBD. The selection of coordinator is up to the decision of users.

- A coordinator supports the following topologies:

- HR-MS linked to the coordinator and the pair is the source and sink of data. This topology is implemented through the local source and sink capability of the HR-MS.

- Two HR-MS linked to the coordinator and the two HR-MS are the source and sink of data. This topology is implemented through the local forwarding capability of the HR-BS.

- A forwarding HR-MS forwards data of a forwarded HR-MS to the coordinator. This topology is implemented through the HR-BS capability to support HR-MS forwarding operation.

- Two HR-MS are linked (DC) and are the source and sink of data to each other under the control of the coordinator. This topology is implemented through the HR-BS ability to support DC between its subordinates.

- The coordinator and any HR-MS that are communicating through the coordinator shall continue cell search operation and shall cease DC operation as soon as the criteria for DC and prevention of interference above are not met.

HR-MS direct communication using distributed resource allocation among nearby HR-MSs, that is called talk-around direct communication, is described in 17.3.2.6.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.