| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Network Discovery Signals for HR-MS Direct Communication without Infrastructure Stations |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | In this contribution, we propose a framework for the HR-MSs communicate to each in case there is no infrastructure station such as HR-BS or HR-RS presented in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Network Discovery Signals for HR-MS Direct Communication without Infrastructure Stations

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# 2. Format of Network Discovery Signals

# 3. Summary

In this proposal, we have proposed format of signals used by HR-MS in discovering other HR-MSs nearby when there is no infrastructure stations.

# 4. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2.5 HR-MS Discovery for Direct Communication without Infrastructure

When HR-MS cannot receive any BS preamble from any infrastructure station or an HR-MS that is associated with an infrastructure station, and HR-MS direct communication without infrastructure is permitted by device configuration, then HR-MSs are allowed to transmit network discovery signals to the network.

An HR-MS stops the transmission of discovery message when it becomes coordinator or, it starts to associate/associates to an infrastructure station or coordinator.

17.2.2.5.1 Back-off Mechanism for the Transmitting of Discovery Message

1) A back-off timer shall be started before an HR-MS transmits a discovery message. HR-MS should get the value for the duration of back-off from a window [0, CW] based on uniform distribution, the size of window can be adjusted based on the traffic of networks. The value of CW shall be between CWmin and CWmax, and inclusive. The back-off value and size of contention window shall be counted in a time unit of OFDMA symbol duration.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles for a duration of four OFDMA symbols. If no preamble is detected for the selected channel, then the HR-MS should transmit the discovery message. If a preamble is detected, then the HR-MS should shall hold on the transmission and restart the timer. The HR-MS shall determine the preamble is from an infrastructure station or from an HR-MS. If it is from an isolated HR-MS that sends discovery messages also, the HR-MS shall double the value of CW if it is less than CWmax and restart the timer. If it is from an infrastructure station or HR-MS associated to an infrastructure station, the HR-MS shall stop the transmission of discovery message on the selected channel.

3) HR-MS should get reset the value of CW to CWmin whenever a successful transmission is made. for the duration of back-off from a window, for example, from a window of  [wmin, wmax], the size of window can be adjusted based on the traffic of networks. The value of Wmin and Wmax are TBD.  CWmin is 64 and CWmax is 1024.

17.2.2.5.2 Format of Discovery Message

The network discovery message shall take the following format: a frame preamble shall be transmitted first followed by control and discovery information. The control information includes FCH, DL-MAP. UL-MAP shall be omitted. Discovery information should follow the DL-MAP. A data packet may be transmitted as part of the discovery information.

For the FCH, it takes the same format as defined in 8.4.4.4.

For the DL-MAP, the DCD messages transmitted in the PHY Synchronization Field shall set the value of frame duration code to 255. The value indicates that the message is not from a BS, it is from an HR-MS for the discovery purpose.  DCD count shall be set to zero. The base station ID shall be set to the MAC address of the current HR-MS.

17.3.2.5.1 DC_DISCOV_Message

17.2.2.5.3 Direct Communication Discovery Messages Format

The discovery message shall take the following encoding format:

Table 1—DC discovery message encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_Message() { | — | — |
| MAC Address | 48 | MAC address of the device |
| Length | 16 | The length of the message |
| NBR Count | 8 | Number of neighboring HR-MSs |
| for(i=0;i<n;i++){ |  |  |
| DC_DISCOV_IE(); |  |  |
| } |  |  |
| } |  |  |

MAC Address

MAC address is the 48 bit address assigned to the HR-MS device.  It shall be used as unique identity of the HR-MS in network discovery.

NBR Count

The value indicates the number of neighboring HR-MSs that the current HR-MS discovered via the neighbor discovery process.

DC_DISCOV_IE

Various information such as name of the HR-MS, MAC address of the neighboring node, invitation for communication etc is contained in the IEs.

17.3.2.5.2 17.2.2.5.3.1 Encoding of DC_DISCOV_IEs

The IEs contained in discovery message has a common encoding format as follows:

Table 2—DC discovery IE encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_IE() { | — | — |
| Type | 8 | — |
| Length | 8 | The length of data contained in the value field |
| Value | variable |  |
| } |  |  |

A few type of IE has been defined in table [number TBD]

Table 3—DC discovery IE types

| Type | Name |
| --- | --- |
| 0x01 | DC_DISCOV_NODE_NAME |
| 0x02 | DC_DISCOV_NBR_ADDR |
| 0x03 | DC_DISCOV_INVITE |
| 0x04 | DC_DISCOV_INVITE_ACCEPT |
| 0x05 | DC_DISCOV_INVITE_REJECT |
| 0x06 – 0xfe | Reserved |
| 0xff | DC_DISCOV_DATA |

17.3.2.5.2.1 17.2.2.5.3.1.1 DC_DISCOV_NODE_NAME

The node name is an ASCII string. The maximum length is 16 bytes.

Table 4—DC HR-MS Name

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x01 | 1 – 16 | A name given by the user of HR-MS |

17.3.2.5.2.2 17.2.2.5.3.1.2 DC_DISCOV_NBR_ADDR

It contains MAC addresses of neighboring HR-MSs discovered by the current HR-MS. Each MAC address takes six bytes. Multiple MAC addresses can be transmitted in the same DC_DISCOV_NBR_ADDR IE.

Table 5—DC Neighbor Address IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x02 | variable | MAC Address of the HR-MSs |

17.3.2.5.2.3 17.2.2.5.3.1.3 DC_DISCOV_INVITE

The IE contains MAC address of the HR-MS that the current HR-MS want to setup connections. Multiple MAC addresses can be contained in the IE.

Table 6—DC Invitation IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x03 | variable | MAC address of the invited HR-MS |

17.3.2.5.2.4 17.2.2.5.3.1.4 DC_DISCOV_INVITE_ACCEPT

The current HR-MS decided to accept the invitation. It intends to join the HR-MS network once the HR-MS become an HR-BS.

Table 7—DC Accept IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x04 | 6 | MAC address of the inviting HR-MS |

The MAC address belongs to the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message

17.3.2.5.2.4 17.2.2.5.3.1.5 DC_DISCOV_INVITE_REJECT

The IE contains the MAC address of the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message and the current HR-MS reject the invitation. It intends to not join the HR-MS network when the HR-MS become an HR-BS.

Table 8—DC Reject IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x05 | 6 | MAC address of the inviting HR-MS |

17.3.2.5.2.6 17.2.2.5.3.1.6 DC_DISCOV_DATA

A short data packet is contained in the IE. The interpretation of the data is up to application.

Table 9—DC Data IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0xff | 1 – 255 | First 6 bytes is the MAC address of intended receving HR-MS and followed by data packets from upper layer |

17.3.2.5 HR-MS Discovery for Direct Communication without Infrastructure

When HR-MS cannot receive any BS preamble from any infrastructure station or an HR-MS that is associated with an infrastructure station, and HR-MS direct communication without infrastructure is permitted by device configuration, then HR-MSs are allowed to transmit network discovery signals to the network.

An HR-MS stops the transmission of discovery message when it becomes coordinator or, it starts to associate/associates to an infrastructure station or coordinator.

17.3.2.5.1 Back-off Mechanism for the Transmitting of Discovery Message

1) A back-off timer shall be started before an HR-MS transmits a discovery message. HR-MS should get the value for the duration of back-off from a window [0, CW] based on uniform distribution, the size of window can be adjusted based on the traffic of networks. The value of CW shall be between CWmin and CWmax, and inclusive. The back-off value and size of contention window shall be counted in a time unit of OFDMA symbol duration.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles for a duration of six OFDMA symbols. If no preamble is detected for the selected channel, then the HR-MS should transmit the discovery message. If a preamble is detected, then the HR-MS should shall hold on the transmission and restart the timer. The HR-MS shall determine the preamble is from an infrastructure station or from an HR-MS. If it is from an isolated HR-MS that sends discovery messages also, the HR-MS shall double the value of CW if it is less than CWmax and restart the timer. If it is from an infrastructure station or HR-MS associated to an infrastructure station, the HR-MS shall stop the transmission of discovery message on the selected channel.

3) HR-MS should get reset the value of CW to CWmin whenever a successful transmission is made. for the duration of back-off from a window, for example, from a window of  [wmin, wmax], the size of window can be adjusted based on the traffic of networks. The value of Wmin and Wmax are TBD.  CWmin is 64 and CWmax is 1024.

17.3.2.5.2 Frame Format for Discovery Message

The network discovery message shall take following format: frame preambles, PA-Preamble and SA-Preamble shall be transmitted first, and then followed by the discovery information as specified in 17.3.2.5.3.

Based on the preamble pattern, HR-MS knows the signals are from a BS or from HR-MSs. The discovery message shall be transmitted after the SA-Preamble and use radio resource specified by SA-Preamble.  The radio resource is TBD.

17.3.2.5.1 DC_DISCOV_Message

17.3.2.5.3  Direct Communication Discovery Messages Format

The discovery message AAI_DC_DISCOV_Message shall take the following encoding format:

Table 10—DC discovery message encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_DC_DISCOV_Message() { | — | — |
| MAC Address | 48 | MAC address of the device |
| Length | 16 | The length of the message |
| NBR Count | 8 | Number of neighboring HR-MSs |
| for(i=0;i<n;i++){ |  |  |
| AAI_DC_DISCOV_IE(); |  |  |
| } |  |  |
| } |  |  |

MAC Address

MAC address is the 48 bit address assigned to the HR-MS device.  It shall be used as unique identity of the HR-MS in network discovery.

NBR Count

The value indicates the number of neighboring HR-MSs that the current HR-MS discovered via the neighbor discovery process.

AAI_DC_DISCOV_IE

Various information Necessary information for HR-MS discovery such as name of the HR-MS, MAC address of the neighboring node, and invitation for communication etc. is contained in the IEs.

17.3.2.5.2 17.3.2.5.3.1 Encoding of AAI_DC_DISCOV_IEs

The IEs contained in discovery message has a common encoding format as follows:

Table 11—DC discovery IE encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_DC_DISCOV_IE() { | — | — |
| Type | 8 | — |
| Length | 8 | The length of data contained in the value field |
| Value | variable |  |
| } |  |  |

A few type of IE has types of IEs have been defined in table [number TBD] 1209.

Table 12—DC discovery IE types

| Type | Name |
| --- | --- |
| 0x01 | AAI_DC_DISCOV_NODE_NAME |
| 0x02 | AAI_DC_DISCOV_NBR_ADDR |
| 0x03 | AAI_DC_DISCOV_INVITE |
| 0x04 | AAI_DC_DISCOV_INVITE_ACCEPT |
| 0x05 | AAI_DC_DISCOV_INVITE_REJECT |
| 0x06 – 0xfe | Reserved |
| 0xff | AAI_DC_DISCOV_DATA |

17.3.2.5.2.1 17.3.2.5.3 .1.1 AAI_DC_DISCOV_NODE_NAME

The node name is an ASCII string. The maximum length is 16 bytes.

Table 13—DC HR-MS Name

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x01 | 1 – 16 | A name given by the user of HR-MS |

17.3.2.5.2.2 17.3.2.5.3 .1.2 AAI_DC_DISCOV_NBR_ADDR

It contains MAC addresses of neighboring HR-MSs discovered by the current HR-MS. Each MAC address takes six bytes. Multiple MAC addresses can be transmitted in the same DC_DISCOV_NBR_ADDR IE.

Table 14—DC Neighbor Address IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x02 | variable | MAC Address of the HR-MSs |

17.3.2.5.2.3 17.3.2.5.3 .1.3 AAI_DC_DISCOV_INVITE

The IE contains MAC address of the HR-MS that the current HR-MS want to setup connections. Multiple MAC addresses can be contained in the IE.

Table 15—DC Invitation IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x03 | variable | MAC address of the invited HR-MS |

17.3.2.5.2.4 17.3.2.5.3 .1.4 AAI_DC_DISCOV_INVITE_ACCEPT

The current HR-MS decided to accept the invitation. It intends to join the HR-MS network once the HR-MS become an HR-BS.

Table 16—DC Accept IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x04 | 6 | MAC address of the inviting HR-MS |

The MAC address belongs to the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message

17.3.2.5.2.4 17.3.2.5.3 .1.5 AAI_DC_DISCOV_INVITE_REJECT

The IE contains the MAC address of the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message and the current HR-MS reject the invitation. It intends to not join the HR-MS network when the HR-MS become an HR-BS.

Table 17—DC Reject IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x05 | 6 | MAC address of the inviting HR-MS |

17.3.2.5.2.6 17.3.2.5.3 .1.6 AAI_DC_DISCOV_DATA

A short data packet is allowed to be attached as an IE contained in the IE. The interpretation of the data packet is up to application.

Table 18—DC Data IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0xff | 1 – 255 | First 6 bytes is the MAC address of intended receving HR-MS and followed by data packets from upper layer |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 201.