| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Discovery for Forwarding to Network for 802.16 OFDMA Networks |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, , Shoukang Zheng, Jaya Shankar Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Network Discovery for Forwarding to Network for 802.16 OFDMA Networks

Haiguang Wang, Hoang Anh Tuan,

Shoukang Zheng, Jaya Shankar, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

This contribution provides the detailed text for Section 17.x.x.x, which describes how a forwarding HR-MS can help an out-of-coverage HR-MS to connect to network infrastructure.

# 2. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.7.1.3 Discovery for HR-MS Forwarding to Network

To enable coverage extension, a serving HR-BS/HR-RS shall schedule some of its directly-associated HR-MSs to transmit preambles so that HR-MSs who are out of coverage of HR-BS/HR-RS can detect and start network entry. The directly-associated HR-MS shall transmit frame preamble at the first OFDM symbol and followed by the network configuration information (NCI).

17.2.7.1.3.1 Frame Structure

The HR-BS/HR-RS shall schedule the selected HR-MSs transmitting preamble and NCI at the end of the UL subframe. A zone in uplink with duration of three OFDMA symbols shall be allocated for discovery. The preamble occupies one OFDMA symbol duration and the FCH and NCI. The format of NCI is defined in section in 17.2.7.1.3.2.

Figure xxx shows the frame structure when HR-MS is scheduled to transmit preamble and NCI. The format of NCI is defined in 17.2.7.1.3.2.

Figure xxx: frame structure for HR-MS transmitting preamble and NCI for FTN discovery

HR-MSs that are out of coverage of HR-BS/HR-RS scanning for DL preambles for possible network entry shall be able to differentiate between preambles transmitted by normal infrastructure stations (HR-BS/HR-RS) and those transmitted by a coverage-extending HR-MS.

After receiving the preamble and NCI, if HR-MS intends to start network entry process, it chooses an initial ranging code and shall transmit the ranging code on the specified ranging channel. Back-off is unnecessary for FTN initial ranging.

A subset of preambles shall be reserved for FTN discovery. The details are TBD.

17.2.7.1.3.2 Scheduling of Preamble and NCI

When HR-BS/HR-RS schedules some of HR-MSs to transmit the FTN discovery message, it shall reserve a zone at the end of uplink sub-frame for the transmission of preamble and NCI with UL_Zone_IE() as specified in Table 388. The zone shall have a minimum length of three symbols.

17.2.7.1.3.3 NCI Message Format

The NCI message is transmitted by the HR-MS so that HR-MS outside the coverage can access the network with the FTN scheme provided in this section.

Table xxx:

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| NCI_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| PHY Synchronization Field | 32 | Specified in Table 319, 8 bits for frame duration and 24 bits for frame number. |
| Base Station ID | 48 | What should be value of ID?? |
| Allocation Start Time | 8 | In number of OFDMA symbols, should including the RTG/TTG gap in the calculation. |
| OFDMA Symbol offset | 8 |  |
| Subchannel offset | 7 |  |
| No. OFDMA Symbols | 7 |  |
| No. Subchannels | 7 |  |
| Ranging Method | 2 | 0b00: Initial ranging over two symbols 0b01: Initial ranging over four symbols 0b10: BR/periodic ranging over one symbol 0b11: BR/periodic ranging over three symbols |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.