| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized Multicast connection in WirelessMAN-OFDMA/WirelessMAN-AAI |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Won-Ik Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0013r1,” in response to Call for Comments on 802.16n (GRIDMAN) AWD |  |
| Abstract | Multicast connection clarification |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Optimized Multicast in WirelessMAN-OFDMA/WirelessMAN-AAI

Eunkyung Kim, Sungcheol Chang, Won-Ik Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD [4] describes optimized MAC protocol for unicast and multicast transmission based on the 802.16n SRD [1]. This contribution provides clarification on the multicast operation in 802.16-2009[2] as well as 802.16m-2001[3].

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0015, “802.16n Amendment Working Draft,” August 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text into Section 6.3.13 in the 802.16n AWD.]

6.3.13 Establishment of multicast connections

[Change Section 6.3.13 as indicated:]

The BS may establish a DL multicast and broadcast service by creating a multicast connection with each SS to be associated with the service. Any available traffic CID value may be used for the service (i.e., there are no dedicated CIDs for multicast transport connections). To ensure proper multicast operation, the CID used for the service is the same for all SSs on the same channel that participate in the connection. The SSs need not be aware that the connection is a multicast connection. However, for multicast and broadcast services which utilize MBS specific features, the multicast connection shall be established using a multicast CID The data transmitted on the connection with the given CID shall be received and processed by the MAC of each involved SS. Thus, each multicast or broadcast SDU is transmitted only once per BS channel. Since a multicast connection is associated with a service flow, it is associated with the QoS and traffic parameters for that service flow.

ARQ is not applicable to multicast connections.

To ensure proper multicast based two-way communications such as Push to Talk (PTT) service among a group of HR-MSs, optimized multicast operation as described in 17.2.9 shall be provided.

If a DL multicast connection is to be encrypted, each SS participating in the connection shall have an additional security association (SA), allowing that connection to be encrypted using keys that are independent of those used for other encrypted transmissions between the SSs and the BS.

[Remedy2: Add the following text into Section 16.2.28 in the 802.16n AWD.]

16.2.28 Support for multicast service

[Change Section 16.2.28 as indicated:]

Multicast service provides an efficient method for concurrent transport of DL data common to a group of users, using a common Multicast Group ID and an FID in an ABS. Multicast service is associated with an ABS and is offered in the downlink only. Each multicast connection is associated with a service flow provisioned with the QoS and traffic parameters for that service flow. Service flows to carry multicast data are instantiated on individual AMSs participating in the service while in Connected State. During such instantiation, the AMS learns the parameters that identify the service and associated service flows.

The ABS shall use a combination of Multicast Group ID and FID to provide the multicast service. The Multicast Group ID is the ID of a group that receives multicast burst and is unique to an ABS. Multicast Group ID is assigned by AAI-DSA-REQ/RSP message and changed by AAI-DSC_REQ message.

To ensure proper multicast based two-way communications such as Push to Talk (PTT) service among a group of HR-MSs, optimized multicast operation as described in 17.3.9 shall be provided.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]