| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Key Usage and Update |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Won-Ik Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0013r1,” in response to Call for Comments on 802.16n (GRIDMAN) AWD |  |
| Abstract | Multicast key management including usage and update in 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Key Usage and Update

Eunkyung Kim, Sungcheol Chang, Won-Ik Kim, Seokki Kim, Sungkyung Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD [4] describes optimized MAC protocol for unicast and multicast transmission based on the 802.16n SRD [1]. According to the multicast key management, HR-MSs in a multicast group share MAK and MTEK and MCMAC key are derived from MAK. This contribution provides how to update MTEK during multicast operation.

# MTEK update

MTEK is a multicast security key to encrypt or decrypt of multicast MAC PDUs. To update MTEK, HR-BS shall transmit AAI-PKM-RSP including following:

- MEKS ~ Encryption key sequence number

- COUNTER_MEK ~ counter value used to derive the new MTEK

Figure 1 shows a procedure how to update MTEK. As shown Figure 1, HR-BS transmits AAI-PKM-RSP message before MTEK lifetime expires. Unless new MEKS is included in AAI-PKM-RSP message, MTEK is not updated but MTEK lifetime is reset. In order to update MTEK of HR-MSs receiving multicast, multicast related information such as Multicast Group ID and FID is included in the AAI-PKM-RSP message. To derive new MTEK, COUNTER_MEK is used based on the following derivation formula as describe in Figure 921 in IEEE 802.16n AWD [4]:

MTEKi=Dot16KEF(MCMAC-MTEK prekey, SAID|COUNTER_MTEK=i|”MTEK”, 128).

Figure 1 - MTEK procedure using AAI-PKM-RSP

# References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0015, “802.16n Amendment Working Draft,” August 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text into Section 16.2.3 in the 802.16n AWD.]

16.2.3.43 Privacy key MAC Control messages (AAI-PKM-REQ/AAI-PKM-RSP)

[Change Table 727 in section 16.2.3.43 as indicated:]

Table 727 – AAI-PKM-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 EAP-Transfer; PKM v3 message code =2 - PKMv3 Key_Agreement-MSG#1; PKM v3 message code =3 - PKMv3 Key_Agreement-MSG#3; PKM v3 message code =5 - PKMv3 TEK-Reply; PKM v3 message code =7 - PKMv3 TEK-Invalid; PKM v3 message code =8 - PKMv3 MTEK-Reply: PKMv3 message code = 9 910–16: Reserved |  |
| ….. | … | … | … |
| If (PKMv3 message code == 9) { |  |  |  |
| Multicast Group ID | 12 | Multicast Group ID to update MTEK |  |
| FID | 4 | FID to update MTEK |  |
| COUNTER_TEK | 16 | CONTER_MTEK used for deriving current MTEK |  |
| MEKS | 2 | Encryption key sequence number for current MTEK |  |
| } |  |  |  |

[Change Table 728 in section 16.2.3.43 as indicated:]

Table 728 – PKMv3 message types

| Code | PKM message type | MAC control message name |
| --- | --- | --- |
| 1 | PKMv3 Reauth-Request | AAI-PKM-REQ |
| 2 | PKMv3 EAP-Transfer | AAI-PKM-REQ/ AAI-PKM-RSP |
| 3 | PKMv3 Key_Agreement-MSG#1 | AAI-PKM-RSP |
| 4 | PKMv3 Key_Agreement-MSG#2 | AAI-PKM-REQ |
| 5 | PKMv3 Key_Agreement-MSG#3 | AAI-PKM-RSP |
| 6 | PKMv3 TEK-Request | AAI-PKM-REQ |
| 7 | PKMv3 TEK-Reply | AAI-PKM-RSP |
| 8 | PKMv3 TEK-Invalid | AAI-PKM-REQ/AAI-PKM-RSP |
| 9 | PKMv3 MTEK-Reply | AAI-PKM-RSP |
| 910–16 | Reserved | — |

[Remedy2: Add the following text into the end of Section 17.3.10.2 in the 802.16n AWD.]

17.3.10.2.x Key usage

17.3.10.2.x.1 MTEK usage

Each MSA maintains MTEK marked as DLE.

The MTEKDLE key is used for encrypting DL multicast data by the HR-BS. The decryption is done according to the MEKS so basically, in transition times.

Each MTEK has its own PN counter size 22bits.

The PN is used for DL multicast traffic and its range is 0x000000-0x1FFFFF.

17.3.10.2.x.2 MTEK update

The MTEK update is triggered by MTEKDLE running out the relevant PN space. In particular, HR-BS derives new MTEK when the DL PN space of MTEKDLE is exhausted.

MTEK update procedure is shown in Figure xxx. The HR-BS shall indicate the new MEKS, COUNTER_MTEK, and MTEK lifetime in the AAI-PKM-RSP message when the current MTEK lifetime expires. If the COUNTER_MTEK or EKS are updated, the HR-MS updates its MTEK accordingly. Unless the COUNTER_MTEK and MEKS are updated, it means the HR-BS did not derive new MTEK yet and the HR-MS shall maintain the current MTEKs but reset the value of MTEK lifetime.

Figure xxx – MTEK update procedure

[-------------------------------------------------End of Text Proposal----------------------------------------------------]