| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-coexistence cycle for 802.16n OFDMA Network |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Ming-Tuo Zhou1, Liru Lu1, Xin Zhang1, Hoang Vinh Dien1, Masayuki Oodo2, Hiroshi Harada1,2 National Institute of Information and Communications Technology 1: 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore 117674 2: 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | Voice: +65 6771 1007 E-mail: mingtuo@nict.com.sg |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | 802.16n amendment draft |  |
| Purpose | To propose AWD text about self-coexistence cycle of 802.16n OFDMA network |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Self-coexistence cycle for 802.16n OFDMA Network

Ming-Tuo Zhou, Liru Lu, Xin Zhang,

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

National Institute of Information and Communications Technology

# Introduction

Self-coexistence is required in 802.16n network due to mobile HR-BS/RS/MS and relatively long interference distance at VHF frequencies. It can be achieved by sharing the same channel in time. Particularly, each cell in self-coexistence operates on one or several frames of a self-coexistence cycle exclusively.

The number of frames in a self-coexistence cycle is limited mainly by two facts. First, the number of frames gives an upper bound of the number of cells that are allowed to share the same frequency channel. Second, the duration of a self-coexistence cycle affects choose of an easily recognized UTC time period for inter-HR-BS synchronization, which is required to occupy different frames by HR cells [1].  In a system design view, a larger frame number is preferred. However, in order to avoid accumulation of time shift at an HR-BS, a smaller number of frames would be better.

Table 1 Example of duration of frame, self-coexistence cycle, and synchronization period for IEEE 802.16n with HR-OFDMA air interface.

| Type of air interface | Frame duration (ms) | Example 1: 4 frames in one SCC |  | Example 2: 8 frames in one SCC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Self-coexistence cycle duration (ms) | Synchronization period (s) | Self-coexistence cycle duration (ms) | Synchronization period (s) |
| 802.16n HR-OFDMA air interface | 2 | 8 | 1 | 16 | 2 |
|  | 2.5 | 10 | 1 | 20 | 1 |
|  | 4 | 16 | 2 | 32 | 4 |
|  | 5 | 20 | 1 | 40 | 1 |
|  | 8 | 32 | 4 | 64 | 8 |
|  | 10 | 40 | 1 | 80 | 2 |
|  | 12.5 | 50 | 1 | 100 | 1 |
|  | 20 | 80 | 2 | 160 | 4 |

Table 1 lists durations of frame, self-coexistence cycle, and synchronization period, of two examples of frame number of a self-coexistence cycle. For the two self-coexistence cycles, the synchronization periods are 4 and 8 second, respectively. However, period of 8 second is more difficult to be recognized compared to a period of 4 second, because information of minute is required for period 8 second since its synchronization pattern is [0 8 16 24 32 40 48 56 4 12 20 28 36 44 52 0] second, while for period 4 second no information of minute is required.

In consideration of balance between synchronization period and maximum number of HR cells in self-coexistence, the frame number of frames in a self-coexistence cycle we chosen is 4 for HR network with WirelessMAN HR OFDMA air interface.

# Reference:

[1] IEEE C802.16n-11/0181r1, Inter-HR-BS Synchronization for 802.16n OFDMA Network

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[------------------------------------------Start of Text Proposal------------------------------------]

17.2.11.1 Self-coexistence cycle

A self-coexistence cycle (SCC) consists of TBD 4 frames as shown in Figure 1. A frame is identified by frame identification number (FIN), which is in range between 0 to TBD-1 3. Location of a frame identification number is TBD.

Figure 1—Self-coexistence cycle for WirelessMAN OFDMA air interface of HR OFDMA network.