| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Inter-HR-BS Synchronization for 802.16n OFDMA Network |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Ming-Tuo Zhou1, Liru Lu1, Xin Zhang1, Hoang Vinh Dien1, Masayuki Oodo2, Hiroshi Harada1,2 National Institute of Information and Communications Technology 1: 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore 117674 2: 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | Voice: +65 6771 1007 E-mail: mingtuo@nict.com.sg |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | 802.16n amendment draft |  |
| Purpose | To propose AWD text about inter-HR-BS synchronization for 802.16n OFDMA network |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Inter-HR-BS Synchronization for 802.16n OFDMA Network

Ming-Tuo Zhou, Liru Lu, Xin Zhang,

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

National Institute of Information and Communications Technology

# Abstract

# Introduction

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[------------------------------------------Start of Text Proposal------------------------------------]

17.2.11.6 Inter-HR-BS Synchronization

17.2.11.6.1 Synchronization of HR-BSs capable of receiving UTC

HR-BS capable of receiving UTC synchronize the absolute local start time of their superframe period, to the start of every 4 seconds referenced to UTC to a tolerance of less than or equal to ±2µs.

17.2.11.6.2 Synchronization between HR-BS capable of receiving UTC and HR-BS incapable of receiving UTC

HR-BS incapable of receiving UTC synchronize with HR-BS capable of receiving UTC.

When an HR station of an HR cell that its serving HR-BS is incapable of receiving UTC receives a self-coexistence beacon from an HR station of a neighboring cell that the severing HR-BS is capable of receiving UTC, it estimates the self-coexistence cycle starting time difference between its cell and the neighboring cell. The method to estimate the self-coexistence cycle starting time difference is as following: it compares the actual receiving time of the self-coexistence beacon and the expected receiving time of the self-coexistence beacon at its cell. The actual receiving time is an estimate of the time that the start of the self-coexistence beacon preamble arrived at the receiving stations’ antenna. The expected reception time is the start time of the corresponding beacon preamble at the cell of the receiving HR station.

In above, if the HR station receiving the self-coexistence beacon is HR-RS or HR-MS, the station reports the self-coexistence cycle starting time difference to the serving HR-BS. The serving HR-BS aligns the starting time of its self-coexistence cycle to the neighboring serving HR-BS based on the time difference received from the reception HR station.

17.2.11.6.3 Synchronization among HR-BSs incapable of receiving UTC

HR-BS incapable of receiving UTC can be classified into two categories: the first is HR-BS that is synchronized with an HR-BS that is capable of receiving UTC, the second is HR-BS is not synchronized with any other HR-BS that is capable of receiving UTC.

The second type of HR-BS shall synchronize with the first type of HR-BS, if an HR station of the former cell can receive self-coexistence beacon from HR station of the later cell. The method of synchronization is similar to the method in 17.2.11.6.2.

For synchronization among the second type HR-BSs, a faster HR-BS shall synchronize with a slower HR-BS. When an HR station with a serving HR-BS of the second type receives a self-coexistence beacon from an HR station of a neighboring cell with a same type of HR-BS, it determines the time difference between the beacon’s actual reception time and its expected reception time. The beacon’s actual reception time is an estimate of the time that the start of the self-coexistence beacon preamble arrived at the receiving stations’ antenna. The expected reception time is the start time of the corresponding beacon preamble at the cell of the receiving HR station. If the difference is positive, then the neighboring serving HR-BR is slower. In order to synchronize with a slower neighboring HR-BS, a serving HR-BS shall delay its frame by the time difference.