| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Inter-HR-BS Synchronization for 802.16n OFDMA Network |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Ming-Tuo Zhou1, Liru Lu1, Xin Zhang1, Hoang Vinh Dien1, Masayuki Oodo2, Hiroshi Harada1,2 National Institute of Information and Communications Technology 1: 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore 117674 2: 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | Voice: +65 6771 1007 E-mail: mingtuo@nict.com.sg |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | 802.16n amendment draft |  |
| Purpose | To propose AWD text about inter-HR-BS synchronization for 802.16n OFDMA network |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Inter-HR-BS Synchronization for 802.16n OFDMA Network

Ming-Tuo Zhou, Liru Lu, Xin Zhang,

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

National Institute of Information and Communications Technology

# Introduction

Inter-HR-BS synchronization is required in  self-coexistence of an HR network. To achieve self-coexistence of several HR networks operating on the same frequency channel by exclusively occupying different frames of a self-coexistence cycle [1], the HR networks must align the starting time of their self-coexistence cycles, and must be synchronized with each other for inter-HR network communications.

A simple way to synchronize several HR-BSs each other is by using UTC, which can be obtained by GPS or backbone network. However, not all HR-BSs are equipped with GPS or connected to backbone network, hence other mechanisms are needed such as by detecting preamble from neighbor HR network. The following introduces methods applicable.

1.  Synchronization of HR-BSs capable of receiving UTC

HR-BSs may be equipped with GPS, or have connection to backhaul network. In this case, an HR-BS is capable of receiving UTC from GPS signal or backhaul network. A simple way for synchronization of HR-BSs capable of receiving UTC is all of them align the absolute local start time of their self-coexistence cycle to the start of an easily recognized UTC time period, such as the starting of every 1 second or every 1 minute, etc. In order to do so, the length of the time period (1 second or 1 minute) is required to be an integral times of the duration of a self-coexistence cycle, for both HR-OFDMA air interface and HR-Advanced air interface.

In HR OFDMA network, a self-coexistence cycle consists of four frames. The duration of a self-coexistence cycle is listed in Table 1. The easily recognized synchronization period for different self-coexistence cycle duration is also listed. Clearly, the synchronization period of 4 seconds is more suitable for all self-coexistence cycle durations than other synchronization periods.

Table 1 Duration of frame, self-coexistence cycle, and synchronization period for IEEE 802.16n with HR-OFDMA air interface.

| Type of air interface | Frame duration (ms) | Self-coexistence cycle duration (ms) | Easily recognized synchronization period (s) |
| --- | --- | --- | --- |
| 802.16n HR-OFDMA air interface | 2 | 8 | 1 |
|  | 2.5 | 10 | 1 |
|  | 4 | 16 | 2 |
|  | 5 | 20 | 1 |
|  | 8 | 32 | 4 |
|  | 10 | 40 | 1 |
|  | 12.5 | 50 | 1 |
|  | 20 | 80 | 2 |

2. Synchronization between HR-BSs capable of receiving UTC and HR-BSs incapable of receiving UTC

In HR networks, some of HR-BSs may have connection to backhaul network or/and are equipped with GPS, but some of HR-BSs may have neither backhaul connection nor GPS receiver. In this case, the former HR-BSs are capable of receiving UTC but the later HR-BSs cannot receive UTC. Since HR-BSs capable of receiving UTC align the absolute local start time of their self-coexistence cycles to the start of every 4 second, a simple way for inter-HR-BS synchronization is that HR-BSs incapable receiving UTC synchronize with HR-BSs capable of receiving UTC. In order to do so, when an HR station of an HR cell that the serving HR-BS is incapable of receiving UTC detects self-coexistence beacons from a neighbor HR station that the serving HR-BS capable of receiving UTC, the former serving HR-BS adjusts start time of its self-coexistence cycle and synchronizes to the later.

3. Synchronization between HR-BSs incapable of receiving UTC

HR-BSs incapable of receiving UTC can be classified into two categories: the first is HR-BS that is synchronized with an HR-BS that is capable of receiving UTC (by using method in above, the second is HR-BS is not synchronized with any other HR-BS that is capable of receiving UTC.

Clearly, the second type of HR-BS needs to synchronize to the first type of HR-BS.

The method of synchronization among the second type of HR-BSs is as following. A faster HR-BS synchronizes to a slower HR-BS. When an HR station with a serving HR-BS of the second type receives a self-coexistence beacon from an HR station of a neighbor network with the same type of HR-BS, it determines the time difference between the beacon’s actual reception time and its expected reception time. The beacon’s actual reception time is an estimate of the time that the start of the self-coexistence beacon preamble arrived at the receiving stations’ antenna. The expected reception time is the start time of the corresponding beacon preamble at the cell of the receiving HR station. If the difference is positive, then the neighbor serving HR-BS is slower. In order to synchronize to a slower neighbor HR-BS, a serving HR-BS shall delay its frame by the time difference.

Reference

[1] IEEE C80216n-11/0178r1, Self-coexistence cycle for 802.16n OFDMA Network

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[------------------------------------------Start of Text Proposal------------------------------------]

17.2.11.6 Inter-HR-BS Synchronization at MAC-layer Level

Inter-HR-BS synchronization at MAC-layer level is needed for self-coexistence between HR networks. Synchronization of HR-BSs at MAC-layer level can be achieved by using common clock if exists or by transmission/reception of self-coexistence beacons if common clock does not exist.

17.2.11.6.1 Synchronization of HR-BSs capable of receiving UTC

An HR-BS capable of receiving UTC synchronizes the absolute local start time of  self-coexistence cycle to the start of every 4 seconds referenced to UTC to a tolerance of less than or equal to ±2µs.

17.2.11.6.2 Synchronization between an HR-BS capable of receiving UTC and an HR-BS incapable of receiving UTC

An HR-BS incapable of receiving UTC can synchronize with an HR-BS capable of receiving UTC. The following method can be used.

When an HR cell that its serving HR-BS is incapable of receiving UTC receives a self-coexistence beacon from a neighbor HR cell that the severing HR-BS is capable of receiving UTC, it estimates time difference between its cell and the neighbor cell and synchronizes to the later.

In above, if HR-RS or HR-MS receives self-coexistence beacon from a neighbor HR cell, it reports the estimated time difference to the serving HR-BS.

17.2.11.6.3 Synchronization between HR-BSs incapable of receiving UTC

HR-BS incapable of receiving UTC can be classified into two categories: the first is HR-BS that is synchronized with an HR-BS that is capable of receiving UTC, the second is HR-BS is not synchronized with any other HR-BS that is capable of receiving UTC.

If an HR cell with the second type of HR-BS can receive self-coexistence beacon from a neighbor HR cell with the first type of HR-BS, the second type of HR-BS can synchronize with the first type HR-BS, by using similar method described in 17.2.11.6.2.

The following method can be used for synchronization between two HR-BS of the second type:

When an HR cell with a serving HR-BS of the second type receives a self-coexistence beacon from a neighbor HR cell with the same type of serving HR-BS, it estimates the time difference between the two cells and synchronizes to it.