| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-coexistence Cycle for 802.16n Advanced Network |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Ming-Tuo Zhou1, Xin Zhang1, Liru Lu1, Hoang Vinh Dien1, Masayuki Oodo2, Hiroshi Harada1,2 National Institute of Information and Communications Technology 1: 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore 117674 2: 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | Voice: +65 6771 1007 E-mail: mingtuo@nict.com.sg |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | 802.16n amendment draft |  |
| Purpose | To propose AWD text about self-coexistence cycle for 802.16n Advanced network |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Self-coexistence Cycle for 802.16n Advanced Network

Ming-Tuo Zhou, Xin Zhang, Liru Lu,

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

National Institute of Information and Communications Technology

# Introduction

Self-coexistence is required in 802.16n network due to mobile HR-BS/RS/MS and relatively long interference distance at VHF frequencies. It can be achieved by sharing the same channel in time. Particularly, each cell in self-coexistence operates on one or several frames of a self-coexistence cycle exclusively.

Self-coexistence cycle is a periodical structure repeated along time for channel sharing in self-coexistence mechanism. A self-coexistence cycle may consist of a number of frames or superframes (depending on the standard that 802.16n is made amendment to). The structure of a self-coexistence needs to meet following requirements:

- an HR networks in self-coexistence is able to share a minimum number of frames for data packets transmission

- the HR networks in self-coexistence all have transmission opportunities for PA-/SA-preambles and superframe headers (SFHs)

- the duration of self-coexistence is easy for inter-HR-BS synchronization

- the self-coexistence cycle is backward compatible

Based on 802.16m frame/superframe structure and in consideration of the above requirements, a self-coexistence cycle structure for HR Advanced network is proposed in this document. It consists of twelve superframes. Structure of superframe is defined in 802.16m, i.e., each superframe consists of 4 frames and duration of each frame is 5 ms.

In normal mode of data service, an HR network transmits PA-/SA-preambles and SFH normally as that defined in 802.16m, i.e., in each of the twelve superframes of a self-coexistence cycle, it transmits an SA-preamble, an PA-preamble, and one more SA-preamble in the first symbol of the first, the second, and the third frame, respectively.

When self-coexistence mechanism is used in 802.16n network, it may lead to an HR-MS/RS loss synchronization to its serving HR-BS if no appropriate method is adopted. This is because that in the last subframe (self-coexistence zone) of the third frame of a superframe, an HR-MS/RS may receive an A-preamble from a neighbor HR network and then synchronize to the neighbor, while loss synchronization to its serving HR-BS.

In order to recover synchronization to the serving HR-BS when above case happens, it proposes that even in normal operation mode, in the first of symbol of the fourth frame of a superframe, an HR network transmits an SA-preamble.

An example is shown in Fig. 1. In this example, cell_1 operates in normal mode and it occupies all frames. It transmits an SA-preamble in the first symbol of the first frame of each superframe, a PA-preamble in the first symbol of the second frame of each superframe, an SA-preamble in the first symbol of the third frame of each superframe, an additional SA-preamble in the first symbol of the fourth frame of each superframe, and SFH in the first subframe of the first frame of each superframe of a self-coexistence cycle.

Fig. 1 Example of self-coexistence cycle structure for 802.16n cell with WirelessMAN HR-Advanced air interface operating in normal mode.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[------------------------------------------Start of Text Proposal------------------------------------]

17.3.11.1 Self-coexistence cycle

TBD

A self-coexistence cycle of consists of twelve superframes. Structure of superframe is described in WirelessMAN Advanced, i.e., each superframe consists of 4 frames and duration of each frame is 5 ms.

17.3.11.1.1 Self-coexistence cycle structure in normal mode

When an HR cell operates in normal mode, it occupies all frames of a superframe, and transmits SA-preamble, PA-preamble, and SFH (superframe header) in every superframe of a self-coexistence cycle as that described in WirelessMAN Advanced.

In addition to transmitting SA-preambles and PA-preamble as described in WirelessMAN Advanced, an HR cell transmits an SA-preamble in the first symbol of the fourth frame of each superframe.

An example is shown in Fig. xx01. Cell_1 operates in normal mode and it occupies all frames. It transmits an SA-preamble in the first symbol of the first frame of each superframe, a PA-preamble in the first symbol of the second frame of each superframe, an SA-preamble in the first symbol of the third frame of each superframe, an additional SA-preamble in the first symbol of the fourth frame of each superframe, and SFH in the first subframe of the first frame of each superframe of a self-coexistence cycle.

Figure xx01 Example of self-coexistence cycle structure for HR cell operating in normal mode.

17.3.11.1.2 Self-coexistence structure in self-coexistence mode

TBD