| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control message to configure HR-MS acting as HR-BS |  |
| Date Submitted | 2011-09-1219 |  |
| Source(s) | Wai Leong Yeow, Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Shoukang Zheng I2R | E-mail: wlyeow@i2r.a-star.edu.sg athoang@i2r.a-star.edu.sg hwang@i2r.a-star.edu.sg jshankar@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multimode MAC control messages for IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAC control messages to support HR-MS acting as BS

Wai Leong Yeow, Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Shoukang Zheng

Institute for Infocomm Research

# Introduction

IEEE 802.16n AWD describes multimode operation based on the 802.16n SRD but does not define the MAC control messages needed to support a HR-MS to operate in a BS mode.

This document provides MAC control messages to configure BS-related parameters for a HR-MS to act as a HR-BS for both WirelessMAN-AAI and WirelessMAN-OFDMA interfaces.

# Protocol

HR-BS transmits a MMBS-CFG message for WirelessMAN-OFDMA interface (or a AAI-MMBS-CFG message for WirelessMAN-AAI interface) to prepare a BS-mode capable HR-MS and configure BS-related operation parameters. The capabilities of the target HR-MS are already known to the HR-BS after capability exchange during network entry or re-entry.

HR-BS defines the BS ID and the operating physical channel parameters for the HR-MS to operate on. In addition, it defines when the mode change shall be initiated. Figures 1 and 2 show two possible scenarios:

Figure 1: A multi-mode BS configuration without a specified trigger.

Figure 2: A multi-mode BS configuration with a specified trigger.

The second scenario differs from the first with a specified trigger. This could be used for preparation of a standalone in the case of SPOF when the serving BS failed.

We propose the following text for the 802.16n AWD in the next section to support such operations.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Add the following rows in Table 38, section 6.3.2.3 of 802.16n AWD.]

| Type | Message Name | Message description | Connection |
| --- | --- | --- | --- |
| x | MMBS-CFG | Multi-mode BS configuration | Basic |

[Add the following text into the end of section 6.3.2.3 of 802.16n AWD.]

6.3.2.3.x	MMBS-CFG message

An MMBS-CFG message is transmitted by the serving HR-BS to HR-MS to configure and prepare the HR-MS for a mode change to BS. See Table aaa for details.

Table aaa—MMBS-CFG message field description

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BSID | 48 | - |
| Preamble Index/Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY specific preamble. For the OFDM PHY, the 5 LSB contain the active DL subchannel index and the 3 MSB shall be Reserved and set to ‘0b000’. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index of the neighbor BS. |
| PHY mode ID | 16 | - |
| Center Frequency | 32 | Indicates center frequency (in unit of Hz) |
| Action Time | 8 | It is the wait time in units of 1 ms before the mode change occurs. |
| TLV | variable |  |

A HR-BS shall generate MMBS-CFG messages in the format shown in Table aaa. The following parameters shall be included:

BSID

The Base Station ID is a 48-bit long field identifying the BS. See DL-MAP.

Preamble Index/Subchannel Index

For the OFDMA PHY this parameter defines the PHY-specific preamble. For the OFDM PHY the 5 LSB contain the DL subchannel index (as defined in Table 248) used in the advertised BS sector. The 3 MSB shall be reserved and set to 0b000.

PHY Mode ID (see 11.18.2)

A 16-bit value that specifies the PHY parameters, including channel bandwidth, FFT size, cyclic prefix, and frame duration.

Center Frequency

Indicates center frequency (in unit of Hz).

Action Time

The amount of time to wait before the mode change is initiated, after receiving the MMBS-CFG message or after the trigger specified in the message fires.

When configuring for a standalone network in case of SPOF, the following TLV parameters shall be included:

Trigger TLV

This specifies a condition when the HR-MS shall initiate a mode change to BS mode. If Action Time is non-zero, the mode change shall be initiated after the triggering condition occurred for the specified amount of time. See Table 577 for details of the triggering conditions. Only functions 0x5 and 0x6 and action 0x4 shall be applicable in this message.

[Modify the following text in Table 577 of 802.16n AWD.]

Table 577—Trigger; Type/function/action description

| Name | Length (bit) | Value |
| --- | --- | --- |
| Type | 2 (MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: Reserved |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor BS is greater than absolute value 0x2: Metric of neighbor BS is less than absolute value 0x3: Metric of neighbor BS is greater than serving BS metric by relative value 0x4: Metric of neighbor BS is less than serving BS metric by relative value 0x5: Metric of serving BS greater than absolute value 0x6: Metric of serving BS less than absolute value 0x7: Reserved NOTE 1—0x1–0x4 not applicable for RTD trigger metric NOTE 2—When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from –32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or 0x6, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as −103.75 dBm and 0xFF is interpreted as − 40 dBm |
| Action | 3 (LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with MOB_SCN-REP after the end of each scanning interval 0x2: Respond on trigger with MOB_MSHO-REQ 0x3: MS shall start neighbor BS scanning process by sending MOB_SCN-REQ, by initiating Autonomous neighbor cell scanning (see 8.4.14.1.3) or both. 0x4–0x7: Reserved 0x4: Initiate mode change 0x5–0x7: Reserved NOTE—0x3 is not applicable when neighbor BS metrics are defined (i.e., only Function values 0x5 or 0x6 are applicable). NOTE—0x4 is only applicable for function values 0x5 and 0x6. |

[Add the following rows in line 173, section 16.2.3 of 802.16n AWD.]

| No. | Functional areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| x | Multimode | AAI-MMBS-CFG | Multimode BS configuration | Encrypted/ICV | Unicast |

[Add the following text at the end of 16.2.3 of 802.16n AWD.]

16.2.3.x AAI-MMBS-CFG

An AAI-MMBS-CFG message is transmitted by the serving HR-BS to HR-MS to configure and prepare the HR-MS for a mode change to BS. See Table bbb.

Table bbb—AAI-MMBS-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| BSID | 48 | ABS ID |  |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with IEEE Std 802.16-2009 definition, with new MAC protocol version 10 defined for AAI. |  |
| CP time | 2 | CP time of the BS 0b00: 1/8 0b01: 1/16 0b10: ¼ |  |
| For(j=0; j<N-Carrier-Info; j++) { |  | N-Carrier-Info is the number of carrier information listed here for the ABSi |  |
| SA-PREAMBLE index | 10 |  |  |
| Physical carrier index | 6 | Refer to the physical carrier index in AAI-Global-CFG message |  |
| Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in ABS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| PGID | 16 | Indicates paging group identifier where the carrier belongs |  |
| } |  |  | // STR |
| Action Time | 8 | It is the wait time in units of 1 ms before the mode change is initiated after reception of the message or after the trigger fires. |  |
| Trigger | variable | Triggers defined in table 775. | Present when configured for standalone network in the case of SPOF. |

A HR-BS shall generate AAI-MMBS-CFG messages in the format shown in Table aaa. The following parameters shall be included:

— 48-bit BS-ID

— ABS SA-Preamble Index

— For every carrier:

— Physical carrier index (6 bits, refer to the “physical carrier index” defined in AAI-Global-CFG)

— MAC protocol versions (8 bits)

— Paging carrier indication (1 bit, refer to specify if a carrier is a paging carrier or not)

— Time to wait before the mode change is initiated

—Trigger condition when configured for standalone network in the case of SPOF.

The trigger condition indicates when a HR-MS shall initiate a mode change to BS with the specified parameters for operating a standalone network in the case of SPOF. If the action time is non-zero, the mode change shall be initiated after the triggering condition occurred for the specified amount of time. See Table 775.

[Modify the following text in Table 776 of 802.16n AWD.]

Table 776— Description of the trigger type/function/action

| Name | Size (Bits) | Value | Description |
| --- | --- | --- | --- |
| Type | 3 (MSB) | Trigger metric type: 0x0: CINR 0x1: RSSI 0x2: RTD 0x3: Number of consecutive PSFHs missed 0x4: RD 0x5–0x7: Reserved |  |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor ABS is greater than absolute value 0x2: Metric of neighbor ABS is less than absolute value 0x3: Metric of neighbor ABS is greater than S-ABS metric by relative value 0x4: Metric of neighbor ABS is less than S-ABS metric by relative value 0x5: Metric of S-ABS greater than absolute value 0x6: Metric of S-ABS less than absolute value 0x7: (For AMS in CA mode): Number of neighbor ABS’s carriers (whose CINR/RSSI is greater than absolute value) is higher than threshold value. The threshold value for the “Number of neighbor ABS’s carriers” can be AMS specific or ABS specific. (For AMS specific, the threshold value is defined as the number of AMS’s active carriers. For ABS specific, the threshold value is configured by the S-ABS and signaled through AAI-MC-ADV.) | Function 0x1-0x4 not applicable for RTD/RD trigger metric When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from –32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5, 0x6, or 0x07, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as –103.75 dBm and 0xFF is interpreted as –40 dBm Type 0x3 can only be used together with function 0x5 or function 0x6. Function 0x7 can only be used with Action 0x1 and applicable for AMS in CA mode. Non-CA mode AMS shall ignore this trigger. |
| Action | 3 (LSB) | Action performed upon trigger condition is satisfied: 0x0: Reserved 0x1: Respond on trigger with AAI-SCN-REP 0x2: Respond on trigger with AAI-HO-REQ 0x3: Respond on trigger with AAI-SCN-REQ 0x4: Declare ABS unreachable: 0x5: Cancel HO 0x6 and 0x7: Reserved 0x6: Initiate mode change 0x7: Reserved | Actions 0x3 and 0x6 applies apply only to Functions 0x5 and 0x6. Action 0x4: If this ABS is the SABS (meaning the AMS is unable to maintain communication with the ABS), AMS sends AAI-HO-IND with HO Event Code 0b10 to the S-ABS and proceeds as specified in 16.2.6.3.4. If this ABS is a TABS, the AMS needs not take immediate action when this trigger condition is met for a single ABS. The AMS shall act only when this condition is met for all T-ABSs included in AAI-HOCMD during HO execution. The specific actions are described in 16.2.6.3.4. |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]