| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | General Description of BS-controlled HR-MS Forwarding to Network |  |
| Date Submitted | 2011-09-19 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Text proposal to Sections 17.2.7.1 and 17.3.7.1.1 of the AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

General Description of BS-controlled HR-MS Forwarding to Network

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution provides descriptions on how BS-controlled HR-MS forwarding to network is supported in 802.16e and 802.16m baselines. The contributed text helps link multiple components and gives reader a more complete picture of the overall operation.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0015.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1 Modify Section 17.2.3.1 as indicated}

17.2.3 HR-MS Forwarding to Network

17.2.3.1 General Description

In HR-MS Forwarding to Network, an HR-MS forwards user data and control signaling between an HR-MS and an HR infrastructure station. The user data and control signaling do not go through higher layer at the forwarding HR-MS. The origination and termination of the user data and control signaling are at the forwarded HR-MS and the HR infrastructure station respectively and vice versa.

HR-MS Forwarding to Network is applicable when 1) the forwarded HR-MS and the forwarding HR-MS are in coverage of and directly associated to an infrastructure station; 2) the forwarding HR-MS is in coverage of and directly associated to an HR infrastructure station, while the forwarded HR-MS is out of coverage of any HR infrastructure stations.

Resource for HR-MS Forwarding to Network can be allocated by the HR infrastructure station with which the forwarding HR-MS is associated.

The general operation of BS-controlled HR-MS forwarding to network (FTN) can be described as follows:

- At the beginning of the process, the serving HR-BS/RS instructs one or a group of its associated HR-MS to broadcast discoverable signals so that an out-of-coverage HR-MS can start network entry and get associated to the HR-BS/RS.

- After the out-of-coverage HR-MS has completed network entry and become an attached forwarded HR-MS, the forwarding HR-MS helps forward control and data messages between the forwarded HR-MS and the serving HR-BS/RS.

{Remedy 2 Modify Section 17.3.3.1 as indicated}

17.3.3 HR-MS Forwarding to Network

17.3.3.1 General Description

In HR-MS Forwarding to Network, an HR-MS forwards user data and control signaling between an HR-MS and an HR infrastructure station. The user data and control signaling do not go through higher layer at the forwarding HR-MS. The origination and termination of the user data and control signaling are at the forwarded HR-MS and the HR infrastructure station respectively and vice versa.

HR-MS Forwarding to Network is applicable when 1) the forwarded HR-MS and the forwarding HR-MS are in coverage of and directly associated to an infrastructure station; 2) the forwarding HR-MS is in coverage of and directly associated to an HR infrastructure station, while the forwarded HR-MS is out of coverage of any HR infrastructure stations.

Resource for HR-MS Forwarding to Network can be allocated by the HR infrastructure station with which the forwarding HR-MS is associated.

The general operation of BS-controlled HR-MS forwarding to network (FTN) can be described as follows:

- At the beginning of the process, the serving HR-BS/RS instructs one or a group of its associated HR-MS to broadcast discoverable signals so that an out-of-coverage HR-MS can start network entry and get associated to the HR-BS/RS. This is described in 17.3.3.2.1.

- After the out-of-coverage HR-MS has completed network entry and become an attached forwarded HR-MS, the forwarding HR-MS helps forward control and data messages between the forwarded HR-MS and the serving HR-BS/RS. MAC-layer support for this process is described in 17.3.3.2 while PHY-layer support for this is described in 17.3.3.3.

Using talk-around direct communication described in 17.3.2.6, HR-MS forwarding to network is described in 17.3.3.5.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.