| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PHY-layer Forwarded Control Channels |  |
| Date Submitted | 2011-09-19 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Text proposal to Sections 17.2.7.1 and 17.3.7.1.1 of the AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

PHY-layer Control Structure for Forwarded HR-MS

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution provides further details for the PHY-layer control structure used to support BS-controlled HR-MS forwarding to network (FTN). We focus on the forwarded control channels, i.e., those transmitted by the forwarded HR-MS. The forwarding control channels, i.e., those transmitted by the forwarding HR-MS, are described in another contribution. The details are mainly for 802.16m baseline but the main principles should be applicable to 802.16-2009 baseline.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0015.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Insert the following new sections into 17.3.3}

17.3.3.3.2.5 HR-MS-to-HR-MS feedback channels

Direct-link feedback channel (DL-FBCH) carry channel quality feedback, MIMO feedback, and event-driven indicators between forwarding and forwarded HR-MS. DL-FBCH follows the same physical structure as that of the Primary Fast Feedback channel, i.e., PFBCH. However, the encoding mechanism for DL-FBCH is not the same as that for PFBCH. The OFDMA resource allocated to PFBCH shall be time-shared by DL-FBCH.

17.3.3.3.2.6 Ranging channel

Ranging channel is used by both forwarding and forwarded HR-MS to achieved transmission synchronization. The forwarding and forwarded HR-MS make use of the same synchronized ranging channel S-RCH defined in 16.3.8.1.4.2 and 16.3.8.1.4.3.

17.3.3.3.2.7 Forwarded BR channel

Forwarded BR channel is used for forwarded HR-MS to carry out contention-based random access bandwidth request, as described in 17.3.3.x.y. Forwarded BR channel follows the same physical structure as that of the UL BR channel described in 16.3.8.1.5, 16.3.8.2.5, and 16.3.8.3.2.

17.3.3.3.2.8 Timing implications to uplink transmission by forwarding HR-MS

As all uplink control channels described in 16.3.8, including feedback channels, BR channel, Ranging channel span 6 OFDMA symbols of the allocated subframe, so do the corresponding physical control channels between forwarding and forwarded HR-MS.

When forwarded HR-MS transmits control/data messages toward forwarding HR-MS, the transmissions span the last 5 OFDMA symbols of the corresponding subframe.

HR-BS and forwarding HR-MS shall adjust uplink communications to cater for the transmissions by the forwarded HR-MS in the following ways:

- Sub-frame conflict avoidance: When HR-BS allocates uplink resource for the forwarding HR-MS for its own uplink transmission, if the allocated subframe that is calculated based on the A-MAP relevance and timing (16.2.14.2.2) is the same as a subframe that is allocated to one of the attached forwarded HR-MSs, the subframe allocated to the forwarding HR-MS shall be delayed until the earliest subframe that is not allocated to any of the forwarding HR-MS’s forwarded HR-MSs. As both HR-BS and forwarding HR-MS are fully aware of the uplink allocations to the attached forwarded HR-MSs, this shifting of uplink subframe allocated to forwarding HR-MS is understood and applied by both HR-BS and forwarding HR-MS.

- Provision of transceiver switching time: The forwarding HR-MS may need to switch between transmitting and receiving states during uplink subframes. This is to allow forwarded HR-MS to transmit to the forwarding HR-MS in an uplink subframe. To do so:

  - If a forwarded HR-MS is going to transmit in a particular uplink subframe and the transmission does not span the first OFDMA symbol of that subframe, and if prior to that subframe, the forwarding HR-MS is in the transmitting state, then the forwarding HR-MS shall switch from the transmitting to the receiving state during the unoccupied first OFDMA symbol of the subframe allocated to the forwarded HR-MS to transmit.

  - If a forwarded HR-MS is going to transmit in a particular uplink subframe and the transmission spans the entire subframe, and if in the subframe that precedes the first subframe, the forwarding HR-MS also needs to transmit to the HR-BS, then the transmission by the forwarding HR-MS shall not occupy the last OFDMA symbol of the subframe that is used by the forwarding HR-MS to transmit to the HR-BS. The forwarding HR-MS shall use this unoccupied last OFDMA symbol for switching from the transmitting state to the receiving state.

  - If a forwarded HR-MS is going to transmit in a particular uplink subframe and if the forwarding HR-MS needs to transmit to the HR-BS in the next uplink subframe, then the transmission by the forwarding HR-MS in this next subframe shall not occupy the first OFDMA symbol of the subframe. The forwarding HR-MS shall use this unoccupied first OFDMA symbol for switching from the receiving state to the transmitting state;

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.