| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Procedures for BS-controlled HR-MS Forwarding to Network |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Text proposal to Sections 17.2.7.1 and 17.3.7.1.1 of the AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Procedures for BS-controlled HR-MS Forwarding to Network

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng,

Institute for Infocomm Research (I2R)

Introduction

This contribution describes MAC Data/Control plane procedure related to BS-controlled HR-MS forwarding to network.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0009r1.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

Remedy 1: Insert the following new section into 17.3.3

17.3.3.x.y MAC procedure for FTN

17.3.3.x.y.1 Construction and transmission of MAC PDUs by forwarding HR-MS

MAC PDUs shall be constructed and transmitted by forwarding HR-MS in the same way as described in 16.2.4, with the exception described in this clause.

17.3.3.x.y.2 Connection management involving HR-MS

Remedy 2: Insert the following new section and copy the text from 17.3.3.4

17.3.3.x.y.3 Bandwidth request involving forwarding HR-MS

An out-of-coverage forwarded HR-MS can request bandwidth by transmitting some known sequences (Bandwidth Request (BR) preambles) toward the forwarding HR-MS.

The process can be described as follows.

-	Serving HR-BS/RS schedules resources in an uplink subframe for forwarded HR-MSs to transmit BR messages to their corresponding forwarding HR-MS.

- The resource allocation information is conveyed to the forwarded HR-MS.

-	The forwarding HR-MS listens to bandwidth requests at times and resources indicated by the HR-BS. The forwarded HR-MS may transmit bandwidth requests using these resources.

-	The forwarding HR-MS, upon receiving BR messages from one of its forwarded HR-MS, forwards the requests to serving HR-BS/RS.

-	Any resource assignment from the HR-BS is forwarded to the forwarding HR-MS.

Remedy 3: Delete existing section 17.3.3.4

17.3.3.4 Bandwidth Requests sent from Forwarded HR-MS

For use case 2, an out-of-coverage forwarded HR-MS can request bandwidth by transmitting some known sequences (Bandwidth Request (BR) preambles) toward the forwarding HR-MS.

The process can be described as follows.

-	Serving HR-BS/RS schedules resources in an uplink subframe for forwarded HR-MSs to transmit BR messages to their corresponding forwarding HR-MS.

- The resource allocation information is conveyed to the forwarded HR-MS.

-	The forwarding HR-MS listens to bandwidth requests at times and resources indicated by the HR-BS. The forwarded HR-MS may transmit bandwidth requests using these resources.

-	The forwarding HR-MS, upon receiving BR messages from one of its forwarded HR-MS, forwards the requests to serving HR-BS/RS.

-	Any resource assignment from the HR-BS is forwarded to the forwarding HR-MS.

Remedy 4: Insert the following new sections

17.3.3.x.y.4 Network entry involving forwarding HR-MS

See 17.3.7.1.2

17.3.3.x.y.5 Synchronization

See 17.3.2.3

17.3.3.x.y.6 HO procedure involving forwarding HR-MS

Normal Operation

Use Case 1

Use Case 2

1

2

3

4

5

6

Figure xxx: Transitions between different operating modes.

Transitions 1 & 2 (from Normal Operation to FTN Operation):

Transitions from normal operation to Use case 1 or 2 can be triggered by the outcomes from the neighbor discovery process. It can also be due to changes in link, interference, and other operating conditions within the network.

Transitions 3 & 4 (from FTN Use Case 1 to Normal & FTN Use Case 2):

Transitions from Use case 1 of HR-MS FTN to either Normal operation or Use case 2 can be triggered by changes in the link conditions between HR-BS, HR-MS1, and HR-MS2. To detect these changes, idle periods can be scheduled by HR-BS for the forwarding and forwarded HR-MS to measure link qualities (e.g. strength of preamble signal from HR-BS and/or strength of sounding signals transmitted by HR-MSs).

Transitions 5 & 6 (from FTN Use Case 2 to Normal & FTN Use Case 1):

Transitions from Use case 1 of HR-MS FTN to either Normal operation or Use case 2 can be triggered by changes in the link conditions between HR-BS, and forwarding/forwarded HR-MSs. To detect these changes, idle periods can be scheduled by HR-BS for HR-MS1, HR-MS2 to measure link qualities (e.g. strength of preamble signal from HR-BS and/or strength of sounding signals transmitted by HR-MSs).

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.