| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging of HR-MS by HR-MS |  |
| Date Submitted | 2011-10-28 |  |
| Source(s) | Eldad Zeira Haiguang Wang, Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for comments to AWD in 802.16n-11/0025 |  |
| Abstract | The text proposal in this contribution provides details for paging by HR-MS |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Paging of HR-MS by HR-MS

# Eldad Zeira, Haiguang Wang, Anh Tuan Hoang

# Introduction

In Bangkok 802.16n has discussed and approved paging of out of coverage HR-MS by those that are in coverage. This contribution provides some more details necessary to specify this operation.

# Proposed Text for the 802.16n Amendment Working Document (AWD) [2]

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Modify 6.12.3.2.1.7 as follows:

The HR-BS may instruct an HR-MS or a group of HR-MS to broadcast a Paging Indicator

(PI) message or IE. The PI contains ID‘s of paged HR-MSs or groups of HR-MSs. Resources for the PI are signaled in the S-NCI. PI format is TBD.

An HR-MS that receives its ID or an assigned group ID in the PI shall access the paging HR-

MS as is done for HR-MS neighbor discovery 6.12.3.2.1.1 except that HR-MS shall use S-RCH resources indicated in the PI IE.

6.12.3.2.1.7.1 MAC operation

To command a forwarding HR-MS or a group of forwarding HR-MS to page, the serving HR-BS sends to a forwarding HR-MS or group of forwarding HR-MS the contents of the PI IE and the resources for its transmission.

To facilitate the paging set up, the HR-BS may establish a group of pagers. To add or remove an HR-MS to/from a group of pagers, the HR-BS sends an AAI-MSPG-GRP message. The message is described in 6.2.3.65.xx.

To control the paging operation of the group, the HR-BS may multicast an AAI-MSPG-PG message which sets the paging ID’s and access parameters paged by the group. The message is described in 6.2.3.65.xy.

Add clause 6.2.3.65.xx:

To add or remove an HR-MS to or from a pagers group the HR-BS shall unicast an AAI-MSPG-GRP described in table xx22:

Table xx22: the AAI-MSPG-GRP message

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| Purpose | 1 | 0b – remove from group, 1b – add to group |  |
| Pager group ID | TBD |  |  |

Add clause 6.2.3.65.xy:

Message AAI-MSPG-PG multicast to a group of paging HR-MS lists of paged HR-MS ID and the access resources to be used by them. The format is defined in table XX23.

Table xx23:

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| Pager group ID | TBD |  | May be omitted if can be uniquely mapped multicast stream identifier, TBD |
| N_PI | TBD | Indicates the number of distinct paging indicators |  |
| For (i=0; i<N_PI; i++) { |  |  | Support of WirelessMANOFDMA with FDM-based UL PUSC Zone |
| PI PID |  |  |  |
| Sub-frame offset of the RCH |  |  |  |
| Start RP code information of the RCH |  |  |  |
| Number of RP codes allocated |  |  |  |
| } |  |  |  |
| For (i=0; i<N_PI; i++) { |  |  | For AAI mode |
| PI PID | TBD | Paging ID |  |
| Start RP code information of the S-RCH | 4 | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |  |
| Number of RP codes allocated | TBD | Indicates number of codes allocated for the (group of) paged HR-MS |  |
| } |  |  |  |

6.12.3.2.1.7.1 PHY operation

The PI shall be transmitted in NPI contiguous DLRUs of an uplink subframe. When PI transmission is indicated by the HR-BS, the paging HR-MS transmits the PI in an UL sub-frame within either the 1’st or 3’rd frame of the superframe in which the I-NCI is located.

The index of the uplink subframe and the index of the starting LRU in which S-NCI is transmitted are signaled in the I-NCI fields. When transmitted, PI spans the last 5 OFDMA symbols of the selected subframe.

Add to table 1215 as follows:

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| PI IE is transmitted | 1 | 1b indicates PI IE is transmitted by HR-MS |
| If PI transmitted { |  |  |
| Subframe index for PI SI_PI | 3 | The 2 LSB bits indicate the index of the UL subframe in which PI is transmitted. The MSB bit indicates the frame in which PI is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame |
| LRU starting index for LRU_PI | 6 | Indicates the starting index of the LRUs allocated for PI |
| } |  |  |

The PI contains the ID of paged HR-MS or groups of HR-MS. How HR-MS or groups of HR-MS receive their paging ID is out of scope for this amendment. The contents and format of the PI IE is given in table xx11.

Table xx11: The PI IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| N_PI | TBD | Indicates the number of distinct paging indicators |
| If (Support of WirelessMANOFDMA with FDM-based UL PUSC Zone){ |  |  |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD | Paging ID |
| Sub-frame offset of the RCH | 2 | Indicates the sub-frame offset OSF of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 |
| Start RP code information of the RCH | 4 | Indicates the kns, which is the parameter for start of the RP code group (rns0). rns0(kns) = 16 × kns + 1, kns = 0,1, ...,15 |
| Number of RP codes allocated | TBD | Indicates number of codes allocated for the (group of) paged HR-MS |
| } |  |  |
| else { |  |  |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD | Paging ID |
| Start RP code information of the S-RCH | 4 | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |
| Number of RP codes allocated | TBD | Indicates number of codes allocated for the (group of) paged HR-MS |
| } |  |  |
| } |  |  |