| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for HR-MS Direct Communication |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HR-MS Direct Communication Link Setup

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# 2. Summary

# 3. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

#### 16.1.3.2 Reactive Operation

The HR-MSs which are capable of role changing to HR-BS may contend for operating at BS mode when the superordinate HR-BS fails. The HR-MSs may initiate a mode switch to HR-BS after expiration of a random backoff timer to avoid potential collision among adjacent HR-MSs trying to perform a mode switch to HR-BS at the same time.

After completion of mode switch, the HR-MS acting as HR-BS may request mode change to one of its subordinate HR-MSs in order to hand HR-BS role over. In this case, it follows the procedure for Proactive operation as described in16.1.3.1.

#### 16.1.3.2.1 Topology Discovery

Before an HR-MS change its role to HR-BS, it shall be allowed to do a topology discovery first by transmitting MM-MS-ADV as defined in 6.3.2.3.98. XX for certain period.

The MM-MS-ADV shall be transmitted after frame preamble, FCH and DL-MAP as a management message. The MM-MS-ADV may include geo-location information that may help the HR-MS decide which HR-MS may become HR-BS.

To avoid collision in topology discovery stage, an HR-MS functioning as HR-BS shall transmit MM-MS-ADV with following rules:

1) A back-off timer shall be started by HR-MS before transmitting a MM-MS-ADV message. HR-MS should get the value for the duration of back-off from a window [0, CW] based on uniform distribution, the size of window can be adjusted based on the traffic of networks. The value of CW shall be between CWmin and CWmax, and inclusive. The back-off value and size of contention window shall be counted in a time unit of OFDMA symbol duration.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles for a duration of four OFDMA symbols. If no preamble is detected for the selected channel, then the HR-MS should transmit the discovery message. If a preamble has been detected, then the HR-MS shall hold on the transmission. The HR-MS shall detect whether the preamble is from an infrastructure station or from an HR-MS. If it is from an isolated HR-MS that sends discovery messages also, the HR-MS shall double the value of CW if it is less than CWmax and restart the timer. If it is from an infrastructure station or HR-MS associated to an infrastructure station, the HR-MS shall stop the transmission of discovery message on the selected channel.

3) HR-MS should reset the value of CW to CWmin whenever a transmission is made. CWmin is 63 and CWmax is 1023.

Topology discovery messages shall not be transmitted if it can receive preambles from other HR-BSs.

## 16.2 Support for Direct Communication between HR-MSs

### 16.2.1 General Description

In HR-MS direct communication, data packets are exchanged between two HR-MSs directly or by passing through another HR-MS. The two communicating HR-MSs are the source and the sink of data. The data packets are passed from upper layers to MAC at the source HR-MS and back to upper layers at the sink HR-MS.

HR-MS direct communication is applicable when 1) the two HR-MSs are in coverage of and are directly associated to an HR infrastructure station; 2) one HR-MS is in coverage of and directly associated to an HR infrastructure station, while the other HR-MS is out of coverage of any HR infrastructure stations; 3) the two HR-MSs are out of coverage of any HR infrastructure stations.

HR-MS direct communication using centralized resource allocation allocated by HR-BS, that is called BS-controlled direct communication, is described in 16.2.2.

HR-MS direct communication using distributed resource allocation among nearby HR-MSs, that is called talk-around direct communication, is described in16.2.3.

For case-3, direct communications between HR-MSs shall satisfy:

- When HR-MSs are out of coverage of any HR infrastructure stations, the operation of HR-MSs shall not interfere with any existing infrastructure stations. When HR-MS cannot receive any BS preamble from any infrastructure station and HR-MS direct communication without infrastructure is permitted by device configuration, HR-MSs are allowed to communicate with each other in the same band without getting permission from infrastructure stations.

- A Coordinator is selected for the coordination of transmission among HR-MSs.  Until a coordinator is selected, an HR-MS is only allowed to transmit signals necessary to enable coordinator selection. To avoid collisions among HR-MSs in coordinator selection, the HR-MS follow a collision avoidance procedure. The procedure is defined in 16.2.4.

- A coordinator shall function as a simplified HR-BS except it may not support handover.

- The coordinator and any HR-MS that are communicating through the coordinator shall continue cell search operation and shall cease DC operation as soon as the criteria for DC and prevention of interference above are not met.

Resource for HR-MS direct communication may be allocated in a distributed manner among nearby HR-MSs independent of infrastructure node deployment for cases (1), (2), and (3).

### 16.2.2 BS-controlled direct communication HR-MS Direct Communication with Centralized Control

When HR-MSs are associated to an infrastructure station, Resource resource for HR-MS direct communication can be allocated by the HR infrastructure station for cases (1) and (2).

#### 16.2.2.1 Medium access control

##### 16.2.2.1.1 HR-MS Neighbor Discovery

HR-MS neighbor discovery is a key functionality to enable other 16n features such as path discovery and management, HR-MS direct communications (with or without presence of infrastructure), and HR-MS forwarding to network. HR-MS neighbor discovery procedures are specified for two scenarios: i) when HR-MSs associated with a common super-ordinate station (HR-BS/RS or a coordinating HR-MS) attempt to discovery each other and ii) when an out-of-coverage HR-MS attempts to discover an HR-MS in order to connect through it to network infrastructure.

###### 16.2.2.1.1.1 Neighbor Discovery among associated HR-MSs (Use Case 1)

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) signals so that other HR-MSs can try to receive and verify their neighbor relationship. Ranging preambles shall be used as PSA signals.

The process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS sends HR-DCV-CMD message to schedule one or multiple associated HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

- In the same HR-DCV-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for ranging signals.

- Each HR-MS that is scheduled to receive ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs may report their measurements to the serving HR-BS/HR-RS using HR-DCV-REP message. Whether a receiving HR-MS shall report its measurements or not may be based on a threshold.

The transmission of HR-DCV-CMD can be described as follows. The HR-BS unicasts HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the HR-DCV-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

##### 16.2.2.1.2 Connection establishment and management for associated HR-MSs

To support direct communication between a pair of HR-MSs, a direct communication link shall be setup between the HR-MSs. The HR-BS/HR-RS may help the source and destination HR-MSs setting up a direct communication link through the procedure specified in this clause.

HR-BS/HR-RS shall check DSA_REQ messages received from HR-MS and determine whether HR-MS direct communication can be adopted for a flow. The HR-BS/HR-RS may help the source and destination HR-MSs setting up a direct communication link through DSA signaling.

HR-BS knows the possibility of setting up a direct communication between two HR-MSs by checking the HR-MS neighbor tables. If the two nodes are neighbor, HR-MS may schedule the two HR-MSs to do channel measurement and determine whether a direct communication link should be setup.

To support direct communication between a pair of HR-MSs, a direct communication link shall be setup between the HR-MSs. When setting up a direct communication the link is first setup, two basic CIDs are assigned to each DC-link to facilitate the two way communication for control message. Of the two assigned, each HR-MS is assigned one of the CIDs for transmitting the control message such as ARQ.

The CID is referred in the link management messages such as link deletion and status report.

A security association may be setup between the two HR-MS linked by the direct communication.  The procedure to setup security association over a direct communication link is defined in section 16.10.1.1. The security association is shared by different flows over the direct communication link.

After a direct communication link is setup, data flows can be setup over the DC-link with the DSA transactions as specified in section 16.2.4. When a flow is assigned over a DC-link, the sender and receiver shall monitor on the CIDs within the MAP and transmit/receive over the allocated resources.

HR-BS may take a few steps to setup a direct communication link between two HR-MSs.

Firstly, the HR-BS shall schedule the two HR-MSs do a channel measurement with the method specified in section 16.2.2.1.3. The HR-MSs reports the channel measurement results to the HR-BS after the measurement.

If HR-BS decides to setup a direct communication link, it shall assign CIDs to the direct communication link and send CIDs to the two HR-MSs using DC-LINK-CREATION-REQ messages. The HR-MSs shall sends back DC-LINK-CREATION-ACK for confirmation.

After receiving DC-LINK-CREATION-ACK from both HR-MSs, the HR-BS may help the two HR-MSs establish a security association over the direct communication link if security is required. The setup of security association over direct communication link is specified in section 16.2.10.

Once a security association is setup, then the communication link is considered being established between the two HR-MSs. The HR-MSs shall find the existing flows between the two HR-MSs and move the existing flows by setting up new flows over the direct communication link with DSA method specified in section 16.2.4.

Figure 801shows the procedure to setup a direct communication link between HR-MSs.

When HR-MS want to delete the direct communication link, it shall send DC-LINK-DELETE-REQ to the two HR-MSs involved.

Figure 1— The overall procedure to setup direct communication

###### 16.2.2.1.2.1 Direct Communication Link Management

16.2.2.1.2.1.1 Direct Communication Link Creation

When HR-BS creates direct communication link between two HR-MSs. It shall allocate a CIDs for the direct communication link and send link creation message to both source and destination HR-MSs. Direct communication link creation can only be initiated by the HR-BS.

The HR-MSs shall send back a response once they receive the direct communication link creation request.

Once the HR-BS receives responses from both HR-MSs, it can continue on other steps of direct communication setup.

16.2.2.1.2.1.2  Direct Communication Link Deletion

When HR-BS wants to remove a direct communication link, it shall send deletion request to both HR-MS and wait for responses from the HR-MSs.

The HR-MS shall reply with reasons to HR-BS when it receives the link deletion request from HR-BS.

16.2.2.1.2.1.3 Direct Communication Link Report

HR-BS may require the HR-MS report the status of the direct communication link by sending a request to the relative HR-MS.

HR-MS shall send back report regarding the direct communication link when it receives a link report request from HR-BS.

Report Request TLVs

The HR-MS only shall provides measurement report for CINR and RSSI.

HR-MS shall send back report regarding the direct communication link when it receives a link report request from HR-BS.

Upon sending a Channel Measurement REP-RSP message, an SS MS shall reset all its measurement counters for each channel on which it reported.

###### 16.2.2.1.2.2 Direct communication service flow management

16.2.2.1.2.2.1 Service flow creation over direct communication link

After a direct communication link has been setup between the source and destination HR-MS, the source HR-MS can setup flows over the direct communication link.

A direct communication setup protocol is illustrated in Figure 2 and described in detail in 16.2.2.1.2.2.3.

When HR-BS receives DSA-REQ from HR-MS, it checks whether a direct communication can be setup between the source and destination. If direct communication can be setup, HR-BS sends back a DSX_RVD to source HR-MS with indication of direct communication link by setting the confirmation code to direct-com-setup as defined in table713. If direct communication is possible, the HR-BS holds on the transmission of DSA-RSP to source HR-MS until it finishes DSA negotiation with the destination HR-MS. The DSA_REQ from HR-BS to the destination HR-MS shall contain a TLV that indicates of direct communication link setting up. The TLV for direct communication is defined in 11.13.46. The DSA_RSP from HR-BS to the source HR-MS shall use a TLV to indicate that the flow will be served over a direction communication link between the source and destination HR-MS with the TLV defined for direct communication in 11.13.46.

Figure 2—The establishment of direct communication between HR-MS

16.2.2.1.2.2.2 Modification and Deletion of Dynamic Service Flow over direct communication link

In addition to the methods presented in 16.2.2.1.2.2.3 for creating service flows, protocols are defined for modifying and deleting service flows; see 16.2.2.1.2.2.4 and 16.2.2.1.2.2.5.

The modification of parameters of a service flow over direct communication link also involves both the source and destination HR-MS.

If the modification is initiated by one HR-MS, then if and only if the HR-BS and the other HR-MS agree with the modification, then the modification can be applied. If the modification is initiated by the HR-BS, then if and only if both HR-MSs agrees with the change then the modification can be applied.

16.2.2.1.2.2.3 Dynamic Service Addition

When HR-BS receives a DSA_REQ from an HR-MS and find that direct communication can be setup between the source and destination HR-MSs by checking the source and destination addresses after testing the integrity. If direct communication can be setup, the Confirmation Code in DSX_RVD message shall be set to direct-comm-setup as defined in table 713 so that the source HR-MS knows that a direct communication link is going to be setup between the source and destination HR-MSs and Timer 74 instead of timer T7. Else, the HR-BS processes the DSA_REQ as a normal request.

HR-BS creates a flow_id based on the QOS requirement in the DSA_REQ.

Before sending DSA_RSP back to the source HR-MS, the HR-BS shall finish the DSA transaction with destination HR-MS with indication of direct communication. The process is illustrated in the Table 1.

Table 1—DSA Process for the direct communication

| Source SS | Destination SS |  | BS |
| --- | --- | --- | --- |
| New service flow needed |  |  |  |
| Check if resources are available |  |  |  |
| Send DSA-REQ Set Timers T7 and T14 |  | ---DSA-REQ--> | Receive DSA-REQ |
|  |  |  | DSA-REQ integrity valid |
| Timer T14 Stops; If direct communication is feasible, Timer T7 stops; Set Timer T74. |  |  | Check whether source and destination SS can support direct communication. |
|  |  | <-- DSX-RVD-- | Check whether source and destination are neighbors. |
|  |  |  | Check whether SS is authorized for service |
|  |  |  | Check whether service flow QoS can be Supported |
|  |  |  | Create SFID |
|  | Receive DSA-REQ | <-- DSA-REQ-- | Send DSA-REQ Set Timer T7 |
|  | Confirm that SS can support service flow |  |  |
|  | Add DL SFID (if present) |  |  |
|  | Enable reception on any new DL service flow |  |  |
|  | Send DSA-RSP | ---DSA-RSP--> | Receive DSA-RSP Timer T7 Stops |
|  |  |  | Enable transmission (DL) or reception (UL) of data on new service flow |
|  | Receive DSA-ACK | <-- DSA-ACK-- | Send DSA-ACK |
|  | Enable transmission on new UL service Flow |  |  |
| Receive DSA-RSP Timer T74 Stops |  | <--DSA-RSP--- | Send DSA-RSP |
| If ActiveQoSParamSet is non-null, Enable transmission or reception of data on new service flow |  |  |  |
| Send DSA-ACK |  | ---DSA-ACK--> | Receive DSA-ACK |
|  |  |  | If DL ActiveQoSParamSet is non-null, Enable transmission of data on new DL service flow |

16.2.2.1.2.2.4 Dynamic Service Change

With direct communication, the data flow is uni-direction, from source to destination HR-MS. Only source HR-MS and HR-BS are allowed to change the service flow parameters.

Source HR-MS initiated DSC procedure

When receiving DSC-REQ from source HR-MS, the HR-BS shall send a DSX-RVD to the source HR-MS first and delay the DSC-RSP to the source HR-MS. A T74 Timer shall be used when the source HR-MS receives DSX_RVD from HR-BS. The HR-BS shall finish the DSC transaction with destination HR-MS first. Once the DSC transaction with destination HR-MS is finished, then the HR-MS sends a DSC-RSP to the source HR-MS and wait for the ACK. The process is illustrated in the Table 2.

Table 2—HR-MS initiated DSC procedure for direct communication

| Source HR-MS | Destination HR-MS |  | HR-BS |
| --- | --- | --- | --- |
| Service flow requires modifying |  |  |  |
| Send DSC-REQ Set Timers T74 and T14 |  | --- DSC-REQ ----> | Receive DSC-REQ |
| Timer T14 Stops |  | <-- DSX-RVD --- | DSC-REQ integrity valid |
|  |  |  | Validate Request |
|  | Receive DSC-REQ | <-- DSC-REQ --- | Set Timers T7 |
|  | Validate request |  |  |
|  | Modify service flow |  |  |
|  | Send DSC-RSP | --- DSC-RSP ----> | Receive DSC-RSP Timer T7 Stops |
|  |  |  | Increase Channel Bandwidth if Required |
|  | Receive DSC-ACK | <-- DSC-ACK --- | Send DSC-ACK |
| Receive DSC-RSP Timer T74 Stops |  | <-- DSC-RSP ---- | Send DSC-RSP |
| Reduce Payload Bandwidth if Required |  |  |  |
| Send DSC-ACK |  | --- DSC-ACK ----> | Decrease channel Bandwidth if Required |
| Increase Payload Bandwidth if Required |  |  |  |

HR-BS Initiated

HR-BS can do modify the flow parameters with the modification over DSC transaction. HR-BS performs the DSC transaction with source and destination HR-MSs separately in certain order.

In case bandwidth allocated to the direct communication link will be increased, the HR-BS should finish the transaction with destination HR-MS first and followed by the DSC transaction with source HR-MS.

In case bandwidth allocated to the direct communication link will be reduced, the HR-BS should finish the transaction with source HR-MS first and followed by the DSC transaction with destination HR-MS.

16.2.2.1.2.2.5 Dynamic Service Deletion

An HR-MS wishing to delete a service flow over HR-MS direct communication link generates a delete request to the HR-BS using a DSD-REQ message. The HR-BS finds the other HR-MS that also associates to the service flow first and then removes the service flow. After that, the HR-BS generates a response using a DSD-RSP message to the HR-MS who sends DSD_REQ. At the same time, the HR-BS also generates DSD-REQ to the other HR-MS who also associates to this service flow.

A BS wishing to delete a dynamic service flow over HR-MS direct communication link generates two delete requests to the associated HR-MSs respectively using DSD-REQs. The HR-MSs removes the service flows and generate responses using DSD-RSPs. This process is illustrated in Table 2.

##### 16.2.2.1.3 Synchronization between HR-MSs involving in HR-MS DC/FTN

This section describes the process of maintaining synchronization between two HR-MSs that communicate directly with each other under HR-MS DC and FTN. The process is employed after HR-MS DC/FTN has been setup, and therefore should be differentiated from the discovery process described in 16.3.2.1.1. Synchronization between HR-MSs is classified into two levels:

- The frame-level should allow HR-MSs to share a common understanding of frame and/or superframe timing and configuration.

- The symbol-level should allow reliable (i.e. received within the appropriate reception threshold) bi-directional transmissions between HR-MSs.

Synchronization mechanisms are specified for three different use cases as follows.

###### 16.2.2.1.3.1 Use case 1: both HR-MSs are within the coverage of HR-BS/RS

The following synchronization mechanisms are used for HR-MS DC/FTN scheduled in uplink area of a frame.

Frame-level Synchronization:

When both HR-MSs are able to receive preambles and DL control signals from a common serving HR-BS/HR-RS, they shall use these to achieve frame-level synchronization (with respect to HR-BS/HR-RS and between themselves).  When both HR-MSs involved in DC or FTN are within the coverage of HR-BS/HR-RS, frame-level synchronization means the HR-MSs acquire DL synchronization with the serving HR-BS/HR-RS and are able to achieve system configuration and control messages.

Symbol-level Synchronization:

When the HR-MS/HR-MS direct link is scheduled in a UL area of a frame, the transmitting HR-MS shall follow the same timing advance as has been adjusted and agreed with the serving HR-BS/HR-RS. This means the transmitting HR-MS shall time its direct transmissions as if these are normal UL transmissions toward the serving HR-BS/HR-RS.

It is the responsibility of the receiving HR-MS to adjust its receive timing to match the time of arrival (TOA) of the signal transmitted by the other HR-MS. This time adjustment shall be achieved by the serving HR-BS/HR-RS scheduling the HR-MSs to transmit ranging sequences to each other. Based on a received ranging sequence, an HR-MS can estimate and correct its time offset with the transmitting HR-MS. To facilitate this process, the serving HR-BS/HR-RS shall assign dedicated ranging sequences and ranging channels in UL area of a frame for HR-MS/HR-MS direct ranging.

To enhance bi-directional communication between HR-MSs, the serving HR-BS/HR-RS can allocate ranging resources to both involved HR-MSs in a single assignment. This allows the receiving HR-MS to transmit back a ranging sequence right after successfully processing the ranging sequence transmitted by the other HR-MS.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

###### 16.2.2.1.3.2 Use case 2: one HR-MS is inside and the other is outside the coverage of HR-BS/RS

The following synchronization mechanisms are used for HR-MS DC/FTN scheduled in uplink area of a frame.

Frame-level Synchronization:

When two HR-MSs need to achieve frame-level synchronization and only one of them is within the coverage of the serving HR-BS/HR-RS, the inside-of-coverage HR-MS shall first acquires DL synchronization with the serving HR-BS/HR-RS (based on preambles and control messages from the serving HR-BS/HR-RS). The inside-of-coverage HR-MS shall subsequently broadcast preambles and possibly network configuration information (NCI) for the outside-of-coverage HR-MS to co-synchronize.

The registered HR-MS shall transmit preambles either at the first OFDMA symbol or the last OFDMA symbol of the frame. The NCI shall be transmitted in an UL area. The location of the NCI, relative to the transmitted preambles, shall be determinable by the outside-of-coverage HR-MS.

Symbol-level Synchronization:

Using the preambles and NCI transmitted by the inside-of-coverage HR-MS, the outside-of-coverage HR-MS shall adjust its timing to receive messages transmitted from the inside-of-coverage HR-MS. To further improve synchronization in this direction, the inside-of-coverage HR-MS can transmit ranging signal toward the outside-of-coverage HR-MS so that this node can estimate and correct its time/frequency offsets. Symbol-level synchronization in the opposite direction, i.e., from the outside-of-coverage of HR-MS toward the inside-of-coverage HR-MS shall be achieved by the outside-of-coverage HR-MS transmitting ranging signal toward the inside-of-coverage HR-MS. Upon processing the received ranging signal, the inside-of-coverage HR-MS can either adjust its own receive timing or request the outside-of-coverage HR-MS to adjust the transmit timing.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

### 16.2.4 Coordinator-based direct communication

When HR-MS cannot receive any BS preamble from any infrastructure station or an HR-MS that is associated with an infrastructure station, and HR-MS direct communication without infrastructure is permitted by device configuration, then HR-MSs are allowed to transmit network discovery signals to the network.

An HR-MS stops the transmission of discovery message when it becomes coordinator, or when it starts to associate to an infrastructure station or coordinator.

#### 16.2.4.1 Back-off Mechanism for the Transmitting of Discovery Message

When HR-MS sends out network discovery messages, to avoid collision with other HR-MSs, it should follow a random-back off mechanism as follows:

1) A back-off timer shall be started before an HR-MS transmits a discovery message. HR-MS should get the value for the duration of back-off from a window [0, CW] based on uniform distribution, the size of window can be adjusted based on the traffic of networks. The value of CW shall be between CWmin and CWmax, and inclusive. The back-off value and size of contention window shall be counted in a time unit of OFDMA symbol duration.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles for a duration of four OFDMA symbols. If no preambles is detected for the selected channel, then the HR-MS should transmit the discovery message. If a preamble has been detected, then the HR-MS shall hold on the transmission. The HR-MS shall detect whether the preamble is from an infrastructure station or from an HR-MS. If it is from an isolated HR-MS that sends discovery messages also, the HR-MS shall double the value of CW if it is less than CWmax and restart the timer. If it is from an infrastructure station or HR-MS associated to an infrastructure station, the HR-MS shall stop the transmission of discovery message on the selected channel.

3) HR-MS should reset the value of CW to CWmin whenever a transmission is made. CWmin is 64 and CWmax is 1024.

#### 16.2.4.2 Format of Discovery Message

The network discovery message shall take the following format: a frame preamble shall be transmitted first followed by control and discovery information. The control information includes FCH, DL-MAP. UL-MAP shall be omitted. Discovery information should follow the DL-MAP. A data packet may be transmitted as part of the discovery information.

For the FCH, it takes the same format as defined in 8.4.4.4.

For the DL-MAP, the DCD messages transmitted in the PHY Synchronization Field shall set the value of frame duration code to 255. The value indicates that the message is not from a BS, it is from an HR-MS for the discovery purpose.  DCD count shall be set to zero. The base station ID shall be set to the MAC address of the current HR-MS.

#### 16.2.4.3 Synchronization between HR-MSs: in case of there is no HR-BS/RS

- The first level synchronization should be carried out in a Master-slave manner. It is understood that the master needs to take on some of the functionality of a BS and may also require new functionality.

- The second level of synchronization can be achieved by HR-MSs exchanging ranging signals.

An example of this scenario is when HR-MS1 and HR-MS2 are having direct communications in an infrastructure-less deployment (or due to single point of failure). For this, an HR-MS (which can be HR-MS1, HR-MS2, or another node) should first be elected as the network coordinator. It is assumed that either one or both HR-MS1 and HR-MS2 then are within the coverage of the elected coordinator. After being elected, the coordinator shall periodically broadcast preambles for frame-level synchronization. With this, the control is back to one of the two earlier scenarios.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.