| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct Communication Control Messages |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# 2. Summary

# 3. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

6.3.2.3.98.6 DSA_REQ message

When HR-BS establishes direct communication between the source and destination HR MSs, the DSA_REQ from HR-BS to the destination HR-MS shall contain a TLV that indicates of direct communication link setting up.

The TLV for direct communication is defined in 11.13.46.

6.3.2.3.98.7 DSA_RSP message

When HR-BS establishes service flow over direct communication link between the source and destination HR-MSs setting up direct communication, the DSA_RSP from HR-BS to the source HR-MS shall indicate by a TLV that a direct communication link should be used for the coming flow use a TLV to indicate that the flow will be served over a direction communication link between the source and destination HR-MS.

The TLV for direct communication is defined in 11.13.46.

6.3.2.3.98.8 DSA_ACK message

After receiving the DSA_ACK from the destination HR-MS, the HR-BS shall send DSA_RSP to the source HR-MS.

6.3.2.3.98.9 DSX_RVD message

When setting up a direct communication link between source and destination HR-MSs, HR-BS should set the confirmation code in DSX_RVD to direct-com-setup as defined in table713.

###### 6.3.2.3.98.10  DC_DISCOV (Direct Communication Discovery) message

The discovery message follows the DL-MAP and shall take the following encoding format:

Table 1—DC discovery message encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_Message() { | — | — |
| Length | 16 | The length of the message |
| MAC address | 48 | The MAC address of the device |
| NBR Count | 8 | Number of neighboring HR-MSs |
| for(i=0;i<n;i++){ |  |  |
| DC_DISCOV_IE(); |  |  |
| } |  |  |
| Name | variable | Name of the node |
| } |  |  |

MAC Address

MAC address is the 48 bit address assigned to the HR-MS device.  It shall be used as unique identity of the HR-MS in network discovery.

NBR Count

The value indicates the number of neighboring HR-MSs that the current HR-MS discovered via the neighbor discovery process.

DC_DISCOV_IE

Various information such as name of the HR-MS, MAC address of the neighboring node, invitation for communication etc is contained in the IEs.

Name

A string used for naming the node.

6.3.2.3.98.10.1 Encoding of DC_DISCOV_IEs

The IEs contained in discovery message has a common encoding format as follows:

Table 2—DC discovery IE encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_IE() { | — | — |
| Type | 8 | — |
| Length | 8 | The length of data contained in the value field |
| Value | variable |  |
| } |  |  |

A few type of IE has been defined in Table 3.

Table 3—DC discovery IE types

| Type | Name |
| --- | --- |
| 0x01 | DC_DISCOV_NODE_NAME |
| 0x02 | DC_DISCOV_NBR_ADDR |
| 0x03 | DC_DISCOV_INVITE |
| 0x04 | DC_DISCOV_INVITE_ACCEPT |
| 0x05 | DC_DISCOV_INVITE_REJECT |
| 0x06 – 0xfe | Reserved |
| 0xff | DC_DISCOV_DATA |

6.3.2.3.98.10.1.1 DC_DISCOV_NODE_NAME

The node name is an ASCII string. The maximum length is 16 bytes.

Table 4—DC HR-MS Name

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x01 | 1 – 16 | A name given by the user of HR-MS |

6.3.2.3.98.10.1.2 DC_DISCOV_NBR_ADDR

It contains MAC addresses of neighboring HR-MSs discovered by the current HR-MS. Each MAC address takes six bytes. Multiple MAC addresses can be transmitted in the same DC_DISCOV_NBR_ADDR IE.

Table 5—DC Neighbor Address IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x02 | variable | MAC Address of the HR-MSs |

6.3.2.3.98.10.1.3 DC_DISCOV_INVITE

The IE contains MAC address of the HR-MS that the current HR-MS want to setup connections. Multiple MAC addresses can be contained in the IE.

Table 6—DC Invitation IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x03 | variable | MAC address of the invited HR-MS |

6.3.2.3.98.10.1.4  DC_DISCOV_INVITE_ACCEPT

The current HR-MS decided to accept the invitation. It intends to join the HR-MS network once the HR-MS become a coordinator.

Table 7—DC Accept IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x04 | 6 | MAC address |

The MAC address belongs to the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message

6.3.2.3.98.10.1.5 DC_DISCOV_INVITE_REJECT

The current HR-MS rejects the invitation from the HR-MS. The IE contains the MAC address of the HR-MS who sends out a DC_DISCOV_INVITE message. It indicates that the current HR-MS declines to communicate with the other HR-MS.

Table 8—DC Reject IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x05 | 6 | MAC address of the inviting HR-MS |

6.3.2.3.98.10.1.6 DC_DISCOV_DATA

A short data packet is contained in the IE. The interpretation of the data is up to application.

Table 9—DC Data IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0xff | 1 – 255 | First 6 bytes is the MAC address of intended HR-MS and follows by the data from upper layer. |

###### 6.3.2.3.98.14 Direct Communication Link Creation RequestDC-LC-REQ

When HR-BS creates direct communication link between two HR-MSs. It shall allocate a CID for the direct communication link and send link creation message to both source and destination HR-MSs. Direct communication link creation can only be initiated by the HR-BS.

Table 10— Direct Communication Link Creation Request

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC-LINK-CREATELC-REQ () { |  |  |
| Management Message Type = [TBD] | 8 | — |
| CID assigned to for transmitting | 16 |  |
| CID assigned for receiving | 16 |  |
| } |  |  |

CID assigned for transmitting

The CID is used by the HR-MS for transmitting. The peer HR-MS of the DC-link shall receive on the resource specified by this CID.

CID assigned for receiving

The HR-MS shall receive on the resource specified by this CID since it is assigned to the peer HR-MS on the DC-Link for transmission.

###### 6.3.2.3.98.15 Direct Communication Link Creation Response DC-LC-RSP

The HR-MSs shall send back a response once they receive the direct communication link creation request.

Table 11— Direct Communication Link Creation Response

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC-LINK-CREATELC- RSP () { |  |  |
| Management Message Type = [TBD] | 8 | — |
| CID assigned to DC link | 16 | CID assigned for transmission |
| Confirmation Code | 1 | 0x00: accept 0x01: reject |
| Reserved | 7 | — |
| } |  |  |

###### 6.3.2.3.98.16  Direct Communication Link Deletion Request DC-LD-REQ

When HR-BS wants remove a direct communication link, it shall send deletion request to both HR-MS and wait for responses from the HR-MSs.

Table 12-- Direct Communication Deletion Request

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC-LINK-DEL LD-REQ () { |  |  |
| Management Message Type = [TBD] | 8 | — |
| CID of DC link | 16 | CID assigned for transmitting |
| } |  |  |

###### 6.3.2.3.98.17 Direct Communication Deletion Response DC-LD-RSP

The HR-MS shall reply with reasons to HR-BS when it receives the link deletion request from HR-BS.

Table 13—Direct Communication Deletion Response

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC-LINK-DELLD-ACK () { |  |  |
| Management Message Type = [TBD] | 8 | — |
| CID of DC link | 16 | CID assigned for transmitting |
| Confirmation Code | 1 | 0x00: accept 0x01: reject |
| Reserved | 7 | — |
| } |  |  |

###### 6.3.2.3.98.18 Direct Communication Link Report Request DC-LR-RSP

HR-BS may require the HR-MS report the status of the direct communication link by sending a request to the relative HR-MS.

Table 14—Direct Communication Link Report Request

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC-LINK-REPORT LR -REQ () { |  |  |
| Management Message Type = [TBD] | 8 | — |
| CID of DC link | 16 | CID assigned for transmitting |
| Report Request TLVs | variable | Specified in section 11.11 |
| } |  |  |

###### 6.3.2.3.98.19 Direct Communication Link Report DC-LR-RSP

HR-MS shall send back report regarding the direct communication link when it receives a link report request from HR-BS.

Table 15—Direct Communication Link Report

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC-LINK-REPORT-RSP LR-RSP () { |  |  |
| Management Message Type = [TBD] | 8 | — |
| CID of DC link | 16 | CID assigned for transmitting |
| Link state | 1 | 0x00: active 0x01: no link found |
| Reserved | 7 | — |
| Report Response TLVs | variable | Specified in section 11.11 |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.