| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for HR-MS Direct Communication |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HR-MS Direct Communication Link Setup

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# 2. Summary

# 3. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

#### DSx MAC control messages

##### AAI-DSA-REQ

[Change last paragraph in section 6.2.3.47.1 AAI-DSA-REQ as indicated:]

When an ABS commences multicast service, the following parameters shall be included in the AAI-DSA-REQ message.

- Multicast Group Zone ID: Indicates multicast group zone IDs for the connection that is associated with the service flow in AAI-DSA-REQ in HR-Network.

- Multicast Indication cycle: Indicates multicast indication cycle for the multicast in HR-Network

- Multicast Group ID: Indicates multicast group for the connection that is associated with the service flow in AAI-DSA-REQ.

[Change Table 740 as indicated:]

Table 740—AAI-DSA-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ………… | …… | ……… | …… |
| For(i=0; i<N-FIDs-Coupled- Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs The maximum value of N-FIDs-Coupled-Noncommon is 32. |  |
| FID | 4 |  | Shall be present if NFIDs-Coupled-Noncommon is not zero |
| Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List Service flow/convergence sublayer parameters in Table 788, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. | Shall be present if NFIDs-Coupled-Noncommon is not zero |
| } |  |  |  |
| } |  |  |  |
| Multicast Group Zone ID | 12 | Indicates a multicast group zone to add where the connection for associated service flow is valid. | Present if needed in HR-Network |
| Multicast Indication cycle | 8 | Start of multicast indication cycle. The first superframe is the multicast available interval and rest superframes are the multicast unavailable interval. 8 LSB of superframe number | Shall be present if Multicast Group Zone is included in this message and the Multicast indication cycle is different from that in AAI-SCD in HR-Network. If the value is the same as that in AAI-SCD, this may not be included in this message |
| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| } |  |  |  |
| If (sleep cycle setting is included) { |  |  |  |
| Operation | 2 | This indicates operation request type 0b00~0b01: Reserved 0b10: Change sleep cycle setting 0b11: Switch sleep cycle setting |  |
| ………… | …… | ……… | …… |
| ………… | …… | ……… | …… |
| DC | 21 | 0b0 – normal request 0b1 – DC request 10, 11 reserved | When direct communication is turned on |
| Reserved | 6 | -- | -- |
| if (DC == 01) { |  |  |  |
| STIDTWDC | 12 | STIDTWDC of assigned to HR-MS to identify the direct communication link | When direct communication is turned on |
| TWDC | 12 | TWDC assigned to peer HR-MS to identify the direct communication link | When direct communication is turned on |
| } |  |  |  |

##### 6.2.3.65.11 Direct Communication Link Creation AAI-DC-LC-REQ

When HR-BS creates direct communication link between two HR-MSs. It shall send link creation message to both source and destination HR-MSs. Direct communication link creation can only be initiated by the HR-BS.

Table 763dc1 – Direct Communication Link Creation Request

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-DC-LINK-CREATE-REQ AAI-DC-LC-REQ () { |  |  |
| TWDC address for transmitting | 12 | Address assigned to DC link |
| TWDC address for receiving | 12 | Address assigned for DC link |
| } |  |  |

TWDC address for transmitting

The TWDC address is used by the HR-MS for transmitting. The peer HR-MS of the DC-link shall receive on the resource scheduled with this TWDC address.

TWDC address for receiving

The HR-MS shall receive on the resource assigned to this TWDC address since it is assigned to the peer HR-MS on the DC-Link for transmission.

##### 6.2.3.65.12 Direct communication link creation response AAI-DC-LC-RSP

The HR-MSs shall send back a response once they receive the direct communication link creation request.

Table 763dc2 – Direct Communication Link Creation Response

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-DC-LINK-CREATE-ACK AAI-DC-LC-RSP () { |  |  |
| TWDC address for transmitting | 12 |  |
| Confirmation Code | 1 | 0x0b0: accept 0x0b1: reject |
| Reserved | 7 | — |
| } |  |  |

Once the HR-BS receives responses from both HR-MSs, it can continue on other steps of direct communication setup.

##### 6.2.3.65.13 Direct Communication Link Deletion Request AAI-DC-LD-REQ

When HR-BS wants remove a direct communication link, it shall send deletion request to both HR-MS and wait for responses from the HR-MSs.

Table 763dc3 – Direct Communication Link Deletion Request

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-DC-LINK-DEL AAI-DC-LD-REQ () { |  |  |
| TWDC address for transmitting | 12 |  |
| } |  |  |

##### 6.2.3.65.14 Direct Communication Link Deletion Response AAI-DC-LD-RSP

The HR-MS shall reply with reasons to HR-BS when it receives the link deletion request from HR-BS.

Table 763dc4 – Direct Communication Link Deletion Response

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-DC-LINK-DEL-ACK AAI-DC-LD-RSP () { |  |  |
| TWDC address for transmitting | 12 | STID TWDC assigned to DC link |
| Confirmation Code | 1 | 0x0b0: accept 0x0b1: reject |
| Reserved | 7 | — |
| } |  |  |

##### 6.2.3.65.15 Direct Communication Link Report Request AAI-DC-LR-REQ

HR-BS may require the HR-MS report the status of the direct communication link by sending a request to the relative HR-MS.

Table 763dc5 – Direct Communication Link Report Request

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-DC-LINK-REPORT-REQ AAI-DC-LR-REQ () { |  |  |
| TWDC address for transmitting | 12 |  |
| } |  |  |

##### 6.2.3.65.16 Direct Communication Link Report Response AAI-DC-LR-REQ

HR-MS shall send back report regarding the direct communication link when it receives a link report request from HR-BS.

Table 763dc6 – Direct Communication Link Report

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-DC-LINK-REPORT-REQ AAI-DC-LR-REQ () { |  |  |
| TWDC address for transmitting | 12 |  |
| Link state | 1 | 0x0b0: active 0x0b1: no link found |
| reserved | 7 | — |
| } |  |  |

##### 6.2.3.65.xx AAI-DC-DISCOV

The AAI-DC-DISCOV message is sent by HR-MS when there is no infrastructure station around.

Table 1—DC discovery message encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_DC_DISCOV_Message() { | — | — |
| MAC Address | 48 | MAC address of the device |
| Length | 8 | The length of the message |
| NBR Count | 8 | Number of neighboring HR-MSs |
| Node Name | Variable | Name for this node |
| } |  |  |

MAC Address

MAC address is the 48 bit address assigned to the HR-MS device.  It shall be used as unique identity of the HR-MS in network discovery.

NBR Count

The value indicates the number of neighboring HR-MSs that the current HR-MS discovered via the neighbor discovery process.

Node Name

A string used for naming the node.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.