| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Management messages |  |
| Date Submitted | 11/1/11 |  |
| Source(s) | Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu, Yen Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | lyt@nmi.iii.org.tw yihjan@yahoo.com hwtseng@mail.dlit.edu.tw mingtuo@nict.com.sg |
| Re: | GRIDMAN Task Group Meeting, November 2011 in Atlanta, Georgia, USA |  |
| Abstract | #### In this contribution we make contribution on MAC management messages(6.3.2.3) for 802.16n Amendment Working Document |  |
| Purpose | To discuss and adopt the proposed text in the P802.16n Draft AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Management messages

Youn-Tai Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu Yen

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

- National Institute of Information and Communications Technology (NICT)

- Introduction

This contribution provides comment on P802.16n Draft AWD.

- Proposed IEEE 802.16n AWD Text Modification

The text in BLACK color: the existing text in AWD

The text in BLUE color: the new text added to the Definitions

===================Start of Proposed Text====================

#### 6.3.2.3 MAC management messages

Table 54—MAC management messages

| Type | Message Name | Message Description | Connection |
| --- | --- | --- | --- |
| 110 | MM-ADV | Multimode Advertisement Message | Multimode Broadcast |
| 111 | MMRS-REQ | Multimode Relay Request Message | Multimode Broadcast |
| 112 | MMRS-RSP | Multimode Relay Response Message | Multimode Broadcast |
| 113 | MMRL-REQ | Multimode Release Request Message | Multimode Broadcast |
| 114 | MMRL-RSP | Multimode Release Response Message | Multimode Broadcast |
| 115 | DSA_REQ | Dynamic Service Add Request Message | Broadcast |
| 116 | DSA_RSP | Dynamic Service Add Response message | Broadcast |
| 117 | DSA_ACK | Dynamic Service Add Acknowledge Message | Broadcast |
| 118 | DSX_RVD | Dynamic Service Request Message | Broadcast |
| 119 | DC_DISCOV | Direct Communication Discovery Message | unicast |
| 120 | AP-NBR-REQ | Neighboring Alternate Path Request Message | Broadcast |
| 121 | AP-NBR-REP | Neighboring Alternate Path Response Message | Broadcast |
| 122 | DSD REQ | Dynamic Service Delete Request Message | unicast |
| 123 | DSD REP | Dynamic Service Delete Response Message | unicast |
| 124 | DSD ACK | Dynamic Service Delete Acknowledge Message | unicast |

====================End of Proposed Text====================

# References

- IEEE 802.16n-11/0024, “P802.16n Draft AWD”, Oct. 2011.