| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | TLV encoding |  |
| Date Submitted | 11/1/11 |  |
| Source(s) | Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu, Yen Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | lyt@nmi.iii.org.tw yihjan@yahoo.com hwtseng@mail.dlit.edu.tw mingtuo@nict.com.sg |
| Re: | GRIDMAN Task Group Meeting, November 2011 in Atlanta, Georgia, USA |  |
| Abstract | ## In this contribution we make contribution on Common encodings(11.1) for 802.16n Amendment Working Document |  |
| Purpose | To discuss and adopt the proposed text in the P802.16n Draft AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

TLV encoding

Youn-Tai Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu Yen

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

- National Institute of Information and Communications Technology (NICT)

- Introduction

This contribution provides comment on P802.16n Draft AWD

- Proposed IEEE 802.16n AWD Text Modification

The text in BLACK color: the existing text in AWD

The text in BLUE color: new text added or text modified

===================Start of Proposed Text====================

## 11.1 Common encodings

| Type | Name |
| --- | --- |
| 149 | HMAC Tuple |
| 148 | MAC Version Encoding |
| 147 | Current Transmit Power |
| 146 | Downlink Service Flow |
| 145 | Uplink Service Flow |
| 144 | Vendor ID Encoding |
| 143 | Vendor-Specific Information |
| 142 | SA-TEK-Update |
| 141 | CMAC tuple |
| 140 | Short-HMAC tuple |
| 139 | Enabled-Action-Triggered |
| 138 | SLPID_Update |
| 137 | Next Periodic Ranging |
| 136 | MAC Hash Skip Threshold |
| 135 | Paging Controller ID |
| 134 | Paging Information |
| 133 | NSP List |
| 132 | Verbose NSP Name List |
| 131 | MIHF frame |
| 130 | MIHF frame type |
| 129 | Query ID |
| 128 | MCID Pre-allocation and Transmission info |
| 127 | MCID Continuity and Transmission Info |
| 126 | HR multicast service flow update mapping info |

====================End of Proposed Text====================

# References

- IEEE 802.16n-11/0024, “P802.16n Draft AWD”, Oct. 2011.