| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Control for two HR-MS associated with an HR-BS |  |
| Date Submitted | 11/1/11 |  |
| Source(s) | Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu, Yen Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | lyt@nmi.iii.org.tw yihjan@yahoo.com hwtseng@mail.dlit.edu.tw mingtuo@nict.com.sg |
| Re: | GRIDMAN Task Group Meeting, November 2011 in Atlanta, Georgia, USA |  |
| Abstract | In this contribution we make contribution on Power control for two HR-MS associated with an HR-BS(16.2.2.2.2.1) for 802.16n Amendment Working Document |  |
| Purpose | To discuss and adopt the proposed text in the P802.16n Draft AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power Control for two HR-MS associated with an HR-BS

Youn-Tai Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu Yen

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

- National Institute of Information and Communications Technology (NICT)

- Introduction

This contribution provides comment on P802.16n Draft AWD

- Proposed IEEE 802.16n AWD Text Modification

Line 12, Page 52,”Definition of power control is TBD” should be modified as following

===================Start of Proposed Text====================

16.2.2.2.2.1 Power control for two HR-MS associated with an HR-BS

Three possible types of power control for the adjustment of the transmission powers of MSs when two HR-MS, HR-MS1 and HR-MS2, are associated an HR-BS can be categorized as in the following:

- Type 1: As shown in Table XXXXX1 where the power control algorithm is performed on the HR-BS after the HR-BS receives the power measurements information from HR-MS1 and HR-MS2.

- Type 2: As shown in Table XXXXX2 where the power control algorithm for the HR-MS1 is performed by the HR-MS1 and the power control algorithm for HR-MS2 is performed in HR-MS2 side. The HR-BS does not intervene with the power control process.

- Type 3: As shown in Table XXXXX3. It is the combination or the hybrid of Type 1 and Type 2 power control algorithms. Depending on the loading of the HR-BS and/or the ‘capability’ of the HR-MS1 and HR-MS2, the power control algorithm(s) may be performed on the HR-BS for some situations but are performed on the HR-MS1 and HR-MS2 for other cases. For example, the HR-BS performs the power control algorithm between HR-MS pairs within its coverage and the HR-MS(s) performs the power control algorithm between the paired HR-MS.

The power control may be performed on a regular (periodic) or on event involved basis. The power measurement may be defined on the traffic channel (PUSCH/PDSCH) or on the control channel (PUCCH/PDCCH).

Many channel estimation algorithms can be found in the literature and it is up to user’s choice.

Table XXXXX1 Power Control for two HR-MS associated with an HR-BS of type 1

Table XXXXX2 Power Control for two HR-MS associated with an HR-BS of type 2

Table XXXXX3 Power Control for two HR-MS associated with an HR-BS of type 3

Power Measurement Algorithm:

The estimated power measured can be expressed as:

P=minPmax,P0+10log10M+α⋅L

Where

Pmax: maximal allowable MS transmit Power, dBm

P0: it is a cell specific and is the minimum MS transmitting power per physical resource block in dBm that is used for controlling the received SNR target

L: path loss between HR-MS1 and HR-MS2 that can be estimated or calculated, in dB

M: number of physical Resource block in the direct communication

α: offset factor for path loss

Power Control Algorithm:

In direct communication the power is adjusted is to make the link SINR(signal to noise plus interference) ratio to meet the target requirement. The target SINR can be expressed as:

SINRTarget=10log10(max(10(SINRMin(dB)10),γIOT(SINRDL)−α))−β∗(10log10(TNS))

Where

SINRMin: the SINR requirement for the minimum data rate in the direct communication link

γIOT: the fairness and IOT(Interference over Thermal) control factor

SINRDL: a linear scale ratio of the downlink signal to interference power, measured by the forwarded HR-MS

α: a factor according to the number of receive antenna at the forwarding transmitter HR-MS

β: a parameter, it is either 0 or 1 depending on the influence of TNS on SINRTarget

TNS: the total number of streams in the direct communication. In SU-MIMO, it is the value of the number of streams in the link

Power Adjustment Algorithm

The transmitting power can be determined from the determined SINRTargetfrom the power control algorithm as:

Ptransmit=(NoisePower+InterferencePoawer)∗(10(SINRTraget10))

Where

SINRTarget:target signal to interference and noise power ration in dBW

Noise power: noise power in watts (=N0*BW) with noise power spectral densityN0 and system channel bandwidth BW

Interference Power: Interference power in watts.

====================End of Proposed Text====================

# References

- IEEE 802.16n-11/0024, “P802.16n Draft AWD”, Oct. 2011.