| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power control for one HR-MS associated with an HR-BS |  |
| Date Submitted | 11/10/11 |  |
| Source(s) | Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu, Yen Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | lyt@nmi.iii.org.tw yihjan@yahoo.com hwtseng@mail.dlit.edu.tw mingtuo@nict.com.sg |
| Re: | GRIDMAN Task Group Meeting, November 2011 in Atlanta, Georgia, USA |  |
| Abstract | In this contribution we make contribution on Power control for one HR-MS associated with an HR-BS(16.2.2.2.2.2) for 802.16n Amendment Working Document |  |
| Purpose | To discuss and adopt the proposed text in the P802.16n Draft AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power control for one HR-MS associated with an HR-BS

Youn-Tai Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu Yen

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

- National Institute of Information and Communications Technology (NICT)

- Introduction

This contribution provides comment on P802.16n Draft AWD

- Proposed IEEE 802.16n AWD Text Modification

Line20, Page 52,”Definition of power control is TBD” should be modified as following

===================Start of Proposed Text====================

16.2.2.2.2.2 Power control for one HR-MS associated with an HR-BS

Three possible types of power control for the adjustment of the transmission powers of MSs when only one HR-MS (forwarding HR-MS) is associated an HR-BS can be categorized as in the following:

- Type 1: As shown in Table YYYYY1 where the power control algorithm is performed on the HR-BS after the HR-BS receives the power measurements information from the forwarding HR-MS and the forwarded HR-MS (via forwarding HR-MS).

- Type 2: As shown in Table YYYYY2 where the power control algorithm for the forwarding HR-MS is performed by the forwarding HR-MS itself and the power control algorithm for the forwarded HR-MS is performed in the forwarded HR-MS side. The HR-BS does not intervene with the power control process.

- Type 3: As shown in Table YYYYY3. It is the combination of the hybrid of Type 1 and Type 2 power control algorithms. For example, the HR-BS performs the power control algorithm to state the possible power control range between HR-MS pairs within its coverage and the forwarding and forwarded HR-MSs performs the power control algorithm between the forwarding and forwarded HR-MS..

The power control may be performed on a regular (periodic) or on event involved basis. The power measurement may be defined on the data traffic channel or on the control channel.

Many channel estimation algorithms can be found in the literature and it is up to user’s choice.

The operation of power control requires that measurements be performed by the HR-forwarding MS and HR-forwarded MS. The process of power measurements and their associated signaling are TBD.

The estimated power measured can be expressed as:

P=minPmax,P0+10log10M+α⋅L

where

Pmax: maximal allowable MS transmit Power, dBm

P0: it is a cell specific and is the minimum MS transmitting power per physical resource block in dBm that is used for controlling the received SNR target

L: path loss between HR-MS1 and HR-MS2 that can be estimated or calculated, in dB

M: number of physical Resource block in the direct communication

α: offset factor for path loss

In direct communication the power is adjusted is to make the link SINR(signal to noise plus interference) ratio to meet the target requirement. The target SINR can be expressed as:

SINRTarget=10log10(max(10(SINRMin(dB)10),γIOT(SINRDL)−α))−β∗(10log10(TNS))

where

SINRMin: the SINR requirement for the minimum data rate in the direct communication link

γIOT: the fairness and IOT(Interference over Thermal) control factor

SINRDL: a linear scale ratio of the downlink signal to interference power, measured by the forwarded HR-MS

α: a factor according to the number of receive antenna at the forwarding transmitter HR-MS

β: a parameter, it is either 0 or 1 depending on the influence of TNS on SINRTarget

TNS: the total number of streams in the direct communication. In SU-MIMO, it is the value of the number of streams in the link

The estimated transmitting power can be determined from the determined SINRTargetfrom the power control algorithm as:

Ptransmit=(NoisePower+InterferencePoawer)∗(10(SINRTraget10))

where

SINRTarget:target signal to interference and noise power ration in dBW

Noise power: noise power in watts (=N0*BW) with noise power spectral densityN0 and system channel bandwidth BW

Interference Power: Interference power in watts.

The power control command parameters can be added in AAI-SCD message with fields including the power adjustment range, minimum and maximum allowable range, the power adjustment step size, and the fairness and interference control factor etc.

Table YYYYY1 Power control for one HR-MS associated with an HR-BS of type 1

Table YYYYY2 Power control for one HR-MS associated with an HR-BS of type 2

Table YYYYY3 Power control for one HR-MS associated with an HR-BS of type 3

====================End of Proposed Text====================

# References

- IEEE 802.16n-11/0024, “P802.16n Draft AWD”, Oct. 2011.