| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-coexistence of IEEE 802.16.1a Network |  |
| Date Submitted | 2011-10-3111-10 |  |
| Source(s) | Ming-Tuo Zhou1, Liru Lu1, Xin Zhang1, Hoang Vinh Dien1, Masayuki Oodo2, Hiroshi Harada1,2 National Institute of Information and Communications Technology 1: 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore 117674 2: 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | Voice: +65 6771 1007 E-mail: mingtuo@nict.com.sg |
| Re: | Call for Comments for 802.16.1a AWD |  |
| Abstract | 802.16n amendment draft |  |
| Purpose | To propose AWD text about self-coexistence of 802.16.1a network |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Self-coexistence of 802.16.1a Network

Ming-Tuo Zhou, Liru Lu, Xin Zhang,

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

National Institute of Information and Communications Technology

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[------------------------------------------Start of Text Proposal------------------------------------]

## [Remedy 1: Remove Section 6.12.11 of IEEE 802.16n-11/0025 and add following text in Section 6.2.21.]

6.2.21.4 Self-coexistence

Self-coexistence allows nearby cells to share the same frequency channel by occupying different DL and UL subframes of a self-coexistence cycle.

6.2.21.4.1 Self-coexistence cycle

A self-coexistence cycle consists of three AAI superframes that are described in 6.3.3.2.2.

The DL subframes of a self-coexistence cycle are indexed as: DLSF0, DLSF1, …, DLSF(ND×3×4-1), where ND is the number of DL subframes in a single frame, “4” is the number of frames in a superframe, and “3” is the number of superframes in a self-coexistence cycle. The UL subframes of a self-coexistence cycle are indexed as: ULSF0, ULSF1, …, ULSF(NU×3×4-1), where NU is the number of UL subframes in a single frame. Figure 521a illustrates structure of an example of self-coexistence cycle with ND of 5 and NU of 3.

Figure 521a ̶ ̶ Example of self-coexistence cycle, ND=5 and NU=3.

6.2.21.4.2 Self-coexistence of two cells

The DL subframes of a self-coexistence cycle are divided into two DL subframe groups: DLSG1={DLSF0, DLSF2, …, DLSF(ND×3×4-2)}，DLSG2={DLSF1, DLSF3, …, DLSF(ND×3×4-1)}. The UL subframes of a self-coexistence cycle are divided into two UL subframe groups: ULSG1={ULSF0, ULSF2, …, ULSF(NU×3×4-2)}，ULSG2={ULSF1, ULSF3, …, ULSF(NU×3×4-1)}.

The base station with Segment ID of 0 occupies DLSG1 and ULSG1. The base station with Segment ID of 1 occupies DLSG2 and ULSG2. The base station with Segment ID of 2 occupies DLSGj and ULSGj, where j=1+cos(i×π/2) and i is Segment ID of the base station of the other cell in self-coexistence.

The two cells in self-coexistence transmit A-preambles in each superframe as that described in 6.3.5.1, and transmit SFH in each superframe as that described in 6.3.5.2.1.

6.2.21.4.3 Self-coexistence of three cells

The DL subframes of a self-coexistence cycle are divided into three DL subframe groups: DLSG1={DLSF0, DLSF3, DLSF6, …, DLSF(ND×3×4-3)}，DLSG2={DLSF1, DLSF4, DLSF7, … DLSF(ND×3×4-2)}, and DLSG3={DLSF2, DLSF5, DLSF8, …, DLSF(ND×3×4-1)}. The UL subframes of a self-coexistence cycle are divided into three UL subframe groups: ULSG1={ULSF0, ULSF3, ULSF6, …, ULSF(NU×3×4-3)}，ULSG2={ULSF1, ULSF4, ULSF7, …, ULSF(NU×3×4-2)}, and ULSG3={ULSF2, ULSF5, ULSF8, …, ULSF(NU×3×4-1)}.

A base station occupies DLSGk and ULSGk, where k is given by: k=Segment ID + 1.

The three cells transmit A-preambles in each superframe as that described in 6.3.5.1, and transmit SFH in each superframe as that described in 6.3.5.2.1.

6.2.21.4.4 Inter-HR-BS synchronization

Inter-HR-BS synchronization is needed for self-coexistence. HR-RS or HR-MS may be instructed to report the time difference between a neighboring HR-BS and its serving HR-BS to the serving HR-BS.