| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed amendments to Privacy Key Management (PKM) messages in IEEE P802.16.1a |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute for Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Contribution for Security Procedures in IEEE 802.16n |  |
| Abstract | Detail description of Security procedures for Direct Communications to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt/amend the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed amendments to Privacy Key Management (PKM) messages in IEEE P802.16.1a

Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research

# Introduction

This contribution contains proposed text changes to the Privacy Key Management (PKM) messages in the P802.16.1a AWD to support Security procedure for direct communication data security.

It should be noted that:

- Proposed texts are placed in the Section they address.  If the text in the contribution addresses more than one section, then the text is split and placed under the appropriate Section numbers (or categories).

- All proposed texts from different contributions that address a particular section are together.

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- All proposed sections are colored blue with an underline and bold face.

- All proposed sections that are to be removed are denoted with a strikethrough and bold face as such This is cancelled text.

- All proposed text are initially colored blue with an underline.

[------------------------------Begin of Text Proposal----------------------------------------------------]

Amend the following sections

#### Forwarding between HR-infrastructure stations

In HR-Networks, the HR-MS may transmit/receive data to/from any one infrastructure station at any given time. And the HR-MS may forward previously received data to other infrastructure stations at other times. To provide higher reliability and have robustness against SPOF, HR-Network shall support Forwarding Between Infrastructure Stations.

#### Privacy key MAC Control messages (AAI-PKM-REQ/AAI-PKM-RSP)

PKMv3 employs two MAC message types: AAI-PKM-REQ (PKM request) and AAI-PKM-RSP (PKM response), as described in Table 726 and Table 727 respectively.

Table 726 —AAI-PKM-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 Reauth-Request; PKM v3 message code = 1 - PKMv3 EAP-Transfer; PKM v3 message code = 2 -PKMv3 Key_Agreement-MSG#2; PKM v3 message code = 4 - PKMv3 TEK-Request; PKM v3 message code = 6 - PKMv3 TEK-Invalid; PKM v3 message code =8 9–16: Reserved - Peer_KeyAgreement_MSG #2; PKM v3 message code = 10 12–16: Reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests |  |
| CMAC indicator | 1 | Indicates whether this message is protected by CMAC tuple 0: Not protected 1: Protected | Shall always be present |
| If( PKM v3 message code ==2) { |  |  |  |
| EAP payload | variable (1..1400 x 8) | Contains the EAP authentication data, not interpreted in the MAC |  |
| } |  |  |  |
| If( PKM v3 message code == 4) { |  |  |  |
| … |  |  |  |
| … |  |  |  |
| If( PKM v3 message code == 8 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| } |  |  |  |
| If( PKM v3 message code == 10) { |  |  |  |
| Key Agreement Type | 8 | Indicates whether this message is for which type of Direct communications key agreement 0: Pre-shared key 1: PKI 2: BS-to-BS security 3-255: Reserved |  |
| If(Key Agreement Type == 0) { |  |  |  |
| NONCE_HR-MS1 | 64 | A random number of 64 bits used for freshness |  |
| NONCE_HR-MS2 | 64 | A random number of 64 bits used for freshness |  |
| DAKID | 64 | identifies the direct communications authorization key |  |
| size of ICV | 1 | 0: size of ICV = 32 bits (default; Max Invalid value is 4096) 1: size of ICV = 64 bits (Max Invalid value is not used) |  |
| PN window Size | 16 | The receiver shall track PNs within this window to prevent replay attacks |  |
| } |  |  |  |
| If(Key Agreement Type == 1) { |  |  |  |
| Timestamp_HR-MS2 | 32 | Timestamp |  |
| NONCE_HR-MS2 | 64 | A random number of 64 bits used for freshness |  |
| HR-MS1Addr | 48 | MAC Address |  |
| HR-MS2Addr | 48 | MAC Address |  |
| NONCE_HR-MS1 | 64 | A random number of 64 bits used for freshness |  |
| Encrypted DMK | 1024 | Public key encryption using HR-MS1's Public key |  |
| SigHR-MS2 | 1024 | Signature of message generated by using its RSA private key |  |
| HR-MS2_Certificate | 1024 | RSA Digital certificate |  |
| } |  |  |  |
| If(Key Agreement Type == 2) { |  |  |  |
| Timestamp_HR-BS | 32 | Timestamp |  |
| NONCE_HR-BS | 64 | A random number of 64 bits used for freshness |  |
| HR-BSdAddr | 48 | MAC Address |  |
| HR-BSAddr | 48 | MAC Address |  |
| NONCE_HR-BSd | 64 | A random number of 64 bits used for freshness |  |
| Encrypted B2MK | 1024 | Public key encryption using HR-MS1's Public key |  |
| SigHR-BS | 1024 | Signature of message generated by using its RSA private key |  |
| HR-BS_Certificate | 1024 | RSA Digital certificate |  |
| }} |  |  |  |

Table 727 —AAI-PKM-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 EAP-Transfer; PKM v3 message code =2 - PKMv3 Key_Agreement-MSG#1; PKM v3 message code =3 - PKMv3 Key_Agreement-MSG#3; PKM v3 message code =5 - PKMv3 TEK-Reply; PKM v3 message code =7 - PKMv3 TEK-Invalid; PKM v3 message code =8 9–16: Reserved - Peer_KeyAgreement_MSG #1; PKM v3 message code = 9 - Peer_KeyAgreement_MSG #3; PKM v3 message code = 11 12–16: Reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests or an HR-MS response to another HR-MS request |  |
| CMAC indicator | 1 | Indicates whether this message is protected by CMAC tuple 0: Not protected 1: Protected | Shall always be present |
| If( PKM v3 message code ==2) { |  |  |  |
| EAP payload | variable (1..1400 x 8) | Contains the EAP authentication data, not interpreted in the MAC |  |
| } |  |  |  |
| If( PKM v3 message code == 3) { |  |  |  |
| … |  |  |  |
| … |  |  |  |
| } |  |  |  |
| If( PKM v3 message code == 8 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| } |  |  |  |
| If( PKM v3 message code == 9) { |  |  |  |
| Key Agreement Type | 8 | Indicates whether this message is for which type of Direct communications key agreement 0: Pre-shared key 1: PKI 2: BS-to-BS security 3-255: Reserved |  |
| If(Key Agreement Type == 0) { |  |  |  |
| NONCE_HR-MS1 | 64 | A random number of 64 bits used for freshness |  |
| DAKID | 64 | identifies the direct communications authorization key |  |
| Key_lifetime | 32 | DMK key lifetime |  |
| } |  |  |  |
| If(Key Agreement Type == 1) { |  |  |  |
| Timestamp_HR-MS1 | 32 | Timestamp |  |
| NONCE_HR-MS1 | 64 | A random number of 64 bits used for freshness |  |
| HR-MS2Addr | 48 | MAC Address |  |
| HR-MS1Addr | 48 | MAC Address |  |
| SigHR-MS1 | 1024 | Signature of message generated by using its RSA private key |  |
| HR-MS1_Certificate | 1024 | RSA Digital certificate |  |
| } |  |  |  |
| If(Key Agreement Type == 2) { |  |  |  |
| Timestamp_HR-BSd | 32 | Timestamp |  |
| NONCE_HR-BSd | 64 | A random number of 64 bits used for freshness |  |
| HR-BSAddr | 48 | MAC Address |  |
| HR-BSdAddr | 48 | MAC Address |  |
| SigHR-BSd | 1024 | Signature of message generated by using its RSA private key |  |
| HR-BSd_Certificate | 1024 | RSA Digital certificate |  |
| }} |  |  |  |
| If( PKM v3 message code == 11) { |  |  |  |
| Key Agreement Type indicator | 8 | Indicates whether this message is for which type of Direct communications key agreement 0: Pre-shared key 1: PKI 2: BS-to-BS security 3-255: Reserved |  |
| If(Key Agreement Type indicator == 0) { |  |  |  |
| NONCE_HR-MS1 | 64 | A random number of 64 bits used for freshness |  |
| NONCE_HR-MS2 | 64 | A random number of 64 bits used for freshness |  |
| size of ICV | 1 | 0: size of ICV = 32 bits (default; Max Invalid value is 4096) 1: size of ICV = 64 bits (Max Invalid value is not used) |  |
| PN window Size | 16 | The receiver shall track PNs within this window to prevent replay attacks |  |
| } |  |  |  |
| If(Key Agreement Type indicator == 1) { |  |  |  |
| NONCE_HR-MS2 | 64 | A random number of 64 bits used for freshness |  |
| HR-MS2Addr | 48 | MAC Address |  |
| HR-MS1Addr | 48 | MAC Address |  |
| } |  |  |  |
| If(Key Agreement Type indicator == 2) { |  |  |  |
| NONCE_HR-BS | 64 | A random number of 64 bits used for freshness |  |
| HR-BSAddr | 48 | MAC Address |  |
| HR-BSdAddr | 48 | MAC Address |  |
| }} |  |  |  |