| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed amendments to Privacy Key Management (PKM) messages in IEEE P802.16n |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute for Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Contribution for Security Procedures in IEEE 802.16n |  |
| Abstract | Detail description of Security procedures for Direct Communications to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt/amend the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed amendments to Privacy Key Management (PKM) messages in IEEE P802.16n

Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research

# Introduction

This contribution contains proposed text changes to the Privacy Key Management (PKM) messages in the 802.16rev3 baseline AWD to support Security procedure for direct communication data security.

It should be noted that:

- Proposed texts are placed in the Section they address.  If the text in the contribution addresses more than one section, then the text is split and placed under the appropriate Section numbers (or categories).

- All proposed texts from different contributions that address a particular section are together.

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- All proposed sections are colored blue with an underline and bold face.

- All proposed sections that are to be removed are denoted with a strikethrough and bold face as such This is cancelled text.

- All proposed text are initially colored blue with an underline.

[------------------------------Begin of Text Proposal----------------------------------------------------]

Amend the following sections

##### Privacy key management (PKM) messages (PKM-REQ/PKM-RSP)

Table 50 —PKM message codes

| Code | PKM message type | MAC management message name |
| --- | --- | --- |
| 0-2 | Reserved | - |
| 3 | SA Add | PKM-RSP |
| 4 | Auth Request | PKM-REQ |
| 5 | Auth Reply | PKM-RSP |
| 6 | Auth Reject | PKM-RSP |
| … | … | … |
| … | … | … |
| 33 | MIH Comeback Response | PKM-RSP |
| 34-255 | Reserved | - |
| 34 | DirectComms_KeyAgreement_MSG #1 | PKM-RSP |
| 35 | DirectComms_KeyAgreement_MSG #2 | PKM-REQ |
| 36 | DirectComms_KeyAgreement_MSG #3 | PKM-RSP |
| 37 | PKIDirectComms_KeyAgreement_MSG #1 | PKM-RSP |
| 38 | PKIDirectComms_KeyAgreement_MSG #2 | PKM-REQ |
| 39 | PKIDirectComms_KeyAgreement_MSG #3 | PKM-RSP |
| 40-255 | Reserved | - |