| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct Communication Security Procedure in IEEE P802.16.1a |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute for Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Contribution for Security Procedures in IEEE 802.16n |  |
| Abstract | Detail description of Security procedures for Direct Communications to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt/amend the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Direct Communication Security Procedure in IEEE P802.16.1a

Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research

# Introduction

This contribution contains proposed text changes to the P802.16.1a AWD in the section of Security procedure for direct communication data security.

It should be noted that:

- Proposed texts are placed in the Section they address.  If the text in the contribution addresses more than one section, then the text is split and placed under the appropriate Section numbers (or categories).

- All proposed texts from different contributions that address a particular section are together.

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- All proposed sections are colored blue with an underline and bold face.

- All proposed sections that are to be removed are denoted with a strikethrough and bold face as such This is cancelled text.

- All proposed text are initially colored blue with an underline.

[------------------------------Begin of Text Proposal----------------------------------------------------]

### Support for security

#### Security procedure for direct communication data security

##### Security procedure for BS-controlled secure direct communication

###### BS-coordinated key management procedure for secure direct communication

In order to support BS-coordinated secure direct communication, the security procedure described in this subsection shall be executed between HR-MS, HR-BS, Authenticator, and AAA Server. Upon successful completion of the security procedure, HR-MSs received the security key from the HR-BS and use this security key for secure direct communication between/among HR-MSs. This security key may be used as the pre-established shared key for secure direct communications in Section 6.12.10.1.1.2.1.

The HR-BS/Authenticator is used to denote that the HR-BS may pass the messages to the AAA-server via the Authenticator for verification and the AAA-server may compute the direct communication security key DMK and send it to the HR-BS via the Authenticator. The flow diagram is shown in Figure 1.

The BS-coordinated security procedure includes the following steps:

Step 1: Once it is determined that secure direct communications is allowed between HR-MS1 and HR-MS2, HR-BS/Authenticator generates the security key DMK, selects NHR-BS and encrypts EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) and computes CMACHR-BS = MACCMAC1(“DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr)|HR-MS1Addr|HR-MS2Addr) and sends Key-Transfer-MSG#1 message to HR-MS1, where Key-Transfer-MSG#1 = “DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr)|HR-MS1Addr|HR-MS2Addr|CMACHR-BS. HR-BS/Authenticator also encrypts EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr) and computes CMACHR-BS = MAC CMAC2(“DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr)|HR-MS2Addr|HR-MS1Addr) and sends Key-Transfer-MSG#2 message to HR-MS2, where Key-Transfer-MSG#2 = “DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr)|HR-MS2Addr|HR-MS1Addr|CMACHR-BS.

Step 2a: If HR-MS1 received Key-Transfer-MSG#1 message from HR-BS/Authenticator, HR-MS1 first checks THR-BS, NHR-BS for freshness and CMACHR-BS for message authentication. If the verifications fail, then HR-MS1 shall ignore Key-Transfer-MSG#1 message. If the verifications are correct, then HR-MS1 decrypts EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) and obtains the security key DMK and its lifetime key_lifetime.

Step 2b: Upon receiving the Key-Transfer-MSG#2 message, HR-MS2 first checks THR-BS, NHR-BS for freshness and CMACHR-BS for message authentication. If the verifications fail, HR-MS2 shall ignore the Key-Transfer-MSG#2 message. If the verifications are correct, then HR-MS2 decrypts EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr) and obtains the security key DMK and its lifetime key_lifetime.

###### BS-coordinated key management procedure for secure direct communication

In order to support BS-coordinated secure direct communication, the security procedure described in this subsection shall be executed between HR-MS, HR-BS, Authenticator, and AAA Server. Upon successful completion of the security procedure, HR-MSs received the security key from the HR-BS/Authenticator and use this security key for secure direct communication between/among HR-MSs. This security key may be used as the pre-established shared key for secure direct communications in Section 6.12.10.1.1.2.1.

The HR-BS/Authenticator is used to denote that the HR-BS passes the messages to the AAA-server via the Authenticator for verification and the direct communication security key DMK is generated by EAP methods and sent securely to the HR-MS via the HR-BS/Authenticator. The flow diagram is shown in Figure 1.

The BS-coordinated security procedure includes the following steps:

Step 1: Once it is determined that secure direct communications is allowed between HR-MS1 and HR-MS2, the security key DMK is generated via EAP methods and transferred securely to HR-MS1 and HR-MS2 using Key-Transfer-MSG#1 and Key-Transfer-MSG#2 messages respectively, where Key-Transfer-MSG#1 and Key-Transfer-MSG#2 are EAP-Transfer messages.

Step 2: HR-MS1 obtains the security key DMK and its lifetime from Key-Transfer-MSG#1 message while HR-MS2 obtains the security key DMK and its lifetime from Key-Transfer-MSG#2 message.

###### Figure 1—Flow Diagram of Authentication and Key Establishment of Network Aided Direct Communication scenario BS-coordinated key management procedure for secure direct communication.

            - Message type

Table 1 — BS-cordinated key management procedure for secure direct communication Message Type

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| 2 | Key-Transfer-MSG#1 | AAI-PKM-RSP |
| 2 | Key-Transfer-MSG#2 | AAI-PKM-RSP |

            - Message attributes

Table 2—Key-Transfer-MSG#1 message attribute

| Attribute | Contents |
| --- | --- |
| “DC_REPLY_OK_BS” | HR-BS response to HR-MS1 that HR-MS2 accepted direct communications |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Freshly generated random number of 64bits by HR-BS |
| EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) | Encryption of DMK, key lifetime by HR-BS using HR-MS1's KEK |
| HR-MS1Addr | Address of HR-MS1 |
| HR-MS2Addr | Address of HR-MS2 |
| CMACHR-BS | Message digest calculated using CMAC key by HR-BS |

| Attribute | Contents |
| --- | --- |
| EAP payload | Contains the DMK key generated using EAP methods, not interpreted in the MAC |

Table 3—Key-Transfer-MSG#2 message attribute

| Attribute | Contents |
| --- | --- |
| “DC_REPLY_OK_BS” | HR-BS response to HR-MS1 that HR-MS2 rejected direct communications |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Nonce generated by HR-BS |
| EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr) | Encryption of DMK, key lifetime by HR-BS using HR-MS2's KEK |
| HR-MS2Addr | Address of HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| CMACHR-BS | Message digest calculated using CMAC key by HR-BS |

| Attribute | Contents |
| --- | --- |
| EAP payload | Contains the DMK key generated using EAP methods, not interpreted in the MAC |

###### Autonomous mutual authentication of HR-MS and data security for direct communications

            - Secure direct communication using pre-established shared key

In order to support secure direct communication between two or among more HR-MSs, pre-established shared key is used.

The pre-established shared key is established prior to the start of this direct communications.

The pre-established shared key may be established using the procedure mentioned in Section  6.12.10.1.1.1.

The key agreement handshake procedure described below shall be used for HR-MSs to mutually authenticate themselves (without access to a security server) using the pre-established shared key and to derive data security keys for secure direct communications. Figure 2 shows the flow diagram while Figure 3 shows the flow chart for this scenario.

The key agreement handshake procedure using pre-established shared key includes the following steps:

Step 1: HR-MS1 selects nonce N HR-MS1 NONCE_HR-MS1 and uses the pre-established shared key DMK to compute DAK, DCMAC key and CMAC digest CMACHR-MS1 CMAC_HR-MS1 = MACDCMAC(N HR-MS1 NONCE_HR-MS1|DMK_Sequence_No|DAKID|Key_lifetime). Finally, HR-MS1 sends the DirectCommsPeer_KeyAgreement_MSG_#1 message to HR-MS2, where DirectCommsPeer_KeyAgreement_MSG_#1 = N HR-MS1 NONCE_HR-MS1|DMK_Sequence_No|DAKID|Key_lifetime|CMACHR-MS1 CMAC_HR-MS1.

Step 2: HR-MS2 first verifies the received nonce is fresh and uses the pre-established shared key DMK to compute DAK =Dot16KDF (DMK, HR-MS1Addr|HR-MS2Addr|”DAK”, 160), the DCMAC key and uses DCMAC key to checks CMACHR-MS1 CMAC_HR-MS1. If the verification fails, HR-MS2 shall ignore the DirectCommsPeer_KeyAgreement_MSG_#1 message. If the verification is correct, HR-MS2 selects N HR-MS2 NONCE_HR-MS2 and computes CMAC Digest CMACHR-MS2 CMAC_HR-MS2 = MACDCMAC(N HR-MS1 NONCE_HR-MS1| N HR-MS2 NONCE_HR-MS2|DAKID|DMK_Sequence_No|DC_Security_Parameters). Finally, HR-MS2 sends DirectCommsPeer_KeyAgreement_MSG_#2 message to HR-MS1, where DirectCommsPeer_KeyAgreement_MSG_#2 = N HR-MS1 NONCE_HR-MS1| N HR-MS2 NONCE_HR-MS2|DAKID|DMK_Sequence_No|DC_Security_Parameters| CMACHR-MS2 CMAC_HR-MS2.

Step 3: HR-MS1 receives the DirectCommsPeer_KeyAgreement_MSG_#2 message from HR-MS2 and checks the received nonces for freshness and also checks DAKID and CMACHR-MS2 CMAC_HR-MS2. If the verifications fail, HR-MS1 shall ignore the DirectCommsPeer_KeyAgreement_MSG_#2 message. If the verifications are correct, HR-MS1 computes CMAC digest CMACHR-MS1 CMAC_HR-MS1 ' = MACDCMAC(N HR-MS1 NONCE_HR-MS1| N HR-MS2 NONCE_HR-MS2|DMK_Sequence_No|DC_SAID|DC_Security_Parameters). Finally, HR-MS1 sends DirectCommsPeer_KeyAgreement_MSG_#3 message to HR-MS2, where DirectCommsPeer _KeyAgreement_MSG_#3 = N HR-MS1 NONCE_HR-MS1| N HR-MS2 NONCE_HR-MS2|DMK_Sequence_No|DC_SAID| DC_Security_Parameters| CMACHR-MS1 CMAC_HR-MS1 '. If HR-MS1 does not receive DirectCommsPeer_KeyAgreement_MSG_#2 message from HR-MS2 within DirectCommsPeer_KeyAgreement_MSG_#1 Timeout, it shall resend the DirectCommsPeer_KeyAgreement_MSG_#1 message up to DirectCommsPeer_KeyAgreement_MSG_#1 MaxResends times. If HR-MS1 reaches its maximum number of resends, it shall initiate another authentication or drop the request.

Step 4: Upon receiving the DirectCommsPeer_KeyAgreement_MSG_#3 message, HR-MS2 checks the received nonces for freshness and CMACHR-MS1 CMAC_HR-MS1 '. If the verifications are invalid, then HR-MS2 shall ignore the DirectCommsPeer_KeyAgreement_MSG_#3 message. If the verifications are correct, HR-MS2 applies the negotiated security parameters. Otherwise, if CMACHR-MS1 CMAC_HR-MS1 ' is invalid, then HR-MS2 shall ignore the DirectCommsPeer_KeyAgreement_MSG_#3 message. If HR-MS2 does not receive DirectCommsPeer_KeyAgreement_MSG_#3 message from HR-MS1 within DirectCommsPeer_KeyAgreement_MSG_#2 Timeout, it shall resend the DirectCommsPeer_KeyAgreement_MSG_#2 message up to DirectCommsPeer_KeyAgreement_MSG_#2 MaxResends times. If HR-MS2 reaches its maximum number of resends, it shall initiate another authentication or drop the request. HR-MS1 and HR-MS2 can now derive DTEK and commence secure direct communications.

            - Figure 2—Flow Diagram of Authentication and Key Establishment of Direct Communication without Infrastructure (Pre-shared key case) Secure direct communication using pre-established shared key.

HR-MS2 verifies the received CMAC tuple. If the verification is correct, HR-MS2 generates its nonce and uses the pre-shared DMK to compute DAK and DCMAC key. HR-MS2 then computes the CMAC tuple and sends DirectCommsPeer_KeyAgreement_MSG_#2 message to HR-MS1.

HR-MS1 verifies the received CMAC tuple and checks the DAKID. If the verifications are correct, then HR-MS1 computes its new CMAC tuple and sends DirectCommsPeer_KeyAgreement_MSG_#3 message to HR-MS2.

HR-MS1 selects its nonce and uses the pre-shared DMK to compute DAK and DCMAC key. HR-MS1 then computes the CMAC tuple and sends DirectCommsPeer_KeyAgreement_MSG_#1 message to HR-MS2.

HR-MS2 receives the above message and verifies the CMAC tuple. If the verification is correct, then HR- MS2 confirms that HR-MS1 has computed the correct keys and both HR-MS1 and HR-MS2 can now derive DTEK and commence secure direct communications.

Figure 3—Flow Chart of Pre-shared key-based Autonomous Direct Communication Authentication and Key Establishment Security Procedure Secure direct communication using pre-established shared key.

              - Message type

Table 4—DC_Request_MSG#1 message attribute Secure direct communication using pre-established shared key message types

| Code | Key Agreement Type | Message Type | MAC control message name |
| --- | --- | --- | --- |
| 9 | 0 | DirectCommsPeer_KeyAgreement_MSG #1 | AAI-PKM-RSP |
| 10 | 0 | DirectCommsPeer_KeyAgreement_MSG #2 | AAI-PKM-REQ |
| 11 | 0 | DirectCommsPeer_KeyAgreement_MSG #3 | AAI-PKM-RSP |

              - Message attributes

Table 5— DirectCommsPeer_KeyAgreement_MSG_#1 message attribute

| Attribute | Contents |
| --- | --- |
| N HR-MS1 NONCE_HR-MS1 | Freshly generated random number of 64bits by HR-MS1 |
| DMK_Sequence_No | new DMK sequence number |
| DAKID | identifies the direct communications authorization key |
| Key_lifetime | DMK key lifetime |
| CMACHR-MS1 CMAC_HR-MS1 | Message digest calculated using DCMAC key |

Table 6— DirectCommsPeer_KeyAgreement_MSG_#2 message attribute

| Attribute | Contents |
| --- | --- |
| N HR-MS1 NONCE_HR-MS1 | Nonce generated by HR-MS1 in DirectCommsPeer_KeyAgreement_MSG_#1 message |
| N HR-MS2 NONCE_HR-MS2 | Freshly generated random number of 64bits by HR-MS2 |
| DAKID | identifies the direct communications authorization key |
| DMK_Sequence_No | new DMK sequence number |
| DC_Security_Parameters | The requesting HR-MS's security capabilities |
| CMACHR-MS2 CMAC_HR-MS2 | Message digest calculated using DCMAC key |

Table 7 — DirectCommsPeer_KeyAgreement_MSG_#3 message attribute

| Attribute | Contents |
| --- | --- |
| N HR-MS1 NONCE_HR-MS1 | Nonce generated by HR-MS1 in DirectCommsPeer_KeyAgreement_MSG_#1 message |
| N HR-MS2 NONCE_HR-MS2 | Nonce generated by HR-MS1 in DirectCommsPeer_KeyAgreement_MSG_#2 message |
| DMK_Sequence_No | new DMK sequence number |
| DC_SAID | identifies the direct communications authorization key for protecting this message |
| DC_Security_Parameters | The supporting HR-MS's security capabilities |
| CMACHR-MS1 CMAC_HR-MS1 ' | Message digest calculated using DCMAC key |

            - Secure direct communication using public key infrastructure

When pre-established shared key is not used for direct communication, Public Key Infrastructure shall be used.

Each HR-MS has a public/private key pair and digital certificate (e.g. X.509) issued by a certification authority for mutual authentication and key exchange prior to the start of this direct communications.

The key agreement handshake procedure described below shall be used for HR-MSs to mutually authenticate themselves (without access to a security server) using Public Key Infrastructure and to derive data security keys for secure direct communications. The flow diagram for this scenario is depicted in Figure 4 and the Flow Chart for this scenario is shown in Figure 5.

The key agreement handshake procedure using Public Key Infrastructure includes the following steps:

Step 1: HR-MS1 first generates nonce N HR-MS1 NONCE_HR-MS1. Next, HR-MS1 computes the signature signatureHR-MS1 SigHR-MS1 = SIGN(T HR-MS1 Timestamp_HR-MS1 | N HR-MS1 NONCE_HR-MS1|HR-MS2Addr|HR-MS1Addr) and sends DirectCommsPeer _KeyAgreement_MSG_#1 message to HR-MS2, where DirectCommsPeer _KeyAgreement_MSG_#1 = T HR-MS1 Timestamp_HR-MS1 | N HR-MS1 NONCE_HR-MS1|HR-MS2Addr|HR-MS1Addr| signatureHR-MS1 SigHR-MS1| Cert(HR-MS1)  HR-MS1_Certificate.

Step 2: HR-MS2 first verifies the received timestamp and nonce for freshness and the certificate Cert(HR-MS1)  HR-MS1_Certificate and signature signatureHR-MS1 SigHR-MS1. If the verifications fail, then HR-MS2 ignores the DirectCommsPeer_KeyAgreement_MSG_#1 message. If the verifications are correct, then HR-MS2 generates nonce N HR-MS2 NONCE_HR-MS2 and security key DMK and computes DAK =Dot16KDF (DMK, HR-MS1Addr|HR-MS2Addr| “DAK”, 160) and the DCMAC key and CMACHR-MS2 CMAC_HR-MS2 = MACDCMAC(N HR-MS2 NONCE_HR-MS2| N HR-MS1 NONCE_HR-MS1|HR-MS2Addr|HR-MS1Addr). HR-MS2 then uses HR-MS1's public key to encrypt and obtain Encrypted DMK =  EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr). Finally, HR-MS2 computes signature signatureHR-MS2 SigHR-MS2 = SIGN(T HR-MS2 Timestamp_HR-MS2| N HR-MS2 NONCE_HR-MS2|HR-MS1Addr|HR-MS2Addr| N HR-MS1 NONCE_HR-MS1| EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) Encrypted DMK | CMACHR-MS2 CMAC_HR-MS2) and sends DirectCommsPeer_KeyAgreement_MSG_#2 message to HR-MS1, where DirectCommsPeer_KeyAgreement_MSG_#2  = T HR-MS2 Timestamp_HR-MS2| N HR-MS2 NONCE_HR-MS2|HR-MS1Addr|HR-MS2Addr| N HR-MS1 NONCE_HR-MS1| EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) Encrypted DMK | CMACHR-MS2 CMAC_HR-MS2 | signatureHR-MS2 SigHR-MS2|Cert({HR-MS2) HR-MS2_Certificate.

Step 3: HR-MS1 first verifies the received timestamp and nonces for freshness and the certificate Cert(HR-MS2)  HR-MS2_Certificate and signature signatureHR-MS2 SigHR-MS2. If the verification is invalid, then HR-MS1 ignores the DirectCommsPeer_KeyAgreement_MSG_#2 message. If the verifications are correct, then HR-MS1 decrypts EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) Encrypted DMK and obtains security key DMK and key_lifetime. Next, HR-MS1 computes DAK and DCMAC keys and verifies CMACHR-MS2 CMAC_HR-MS2. If the verification is invalid, then HR-MS1 ignores the DirectCommsPeer_KeyAgreement_MSG_#2 message. If the verification is correct, then HR-MS1 computes CMAC Digest CMACHR-MS1 CMAC_HR-MS1 = MACDCMAC(N HR-MS1 NONCE_HR-MS1| N HR-MS2 NONCE_HR-MS2|HR-MS1Addr|HR-MS2Addr) and sends DirectCommsPeer_KeyAgreement_MSG_#3 message to HR-MS2, where DirectCommsPeer_KeyAgreement_MSG_#3 = N HR-MS2 NONCE_HR-MS2|HR-MS2Addr|HR-MS1Addr| CMACHR-MS1 CMAC_HR-MS1. If HR-MS1 does not receive DirectCommsPeer_KeyAgreement_MSG_#2 message from HR-MS2 within DirectCommsPeer_KeyAgreement_MSG_#1 Timeout, it shall resend the DirectCommsPeer_KeyAgreement_MSG_#1 message up to DirectCommsPeer_KeyAgreement_MSG_#1 MaxResends times. If HR-MS1 reaches its maximum number of resends, it shall initiate another authentication or drop the request.

Step 4: HR-MS2 receives the DirectCommsPeer_KeyAgreement_MSG_#3 message and verifies received nonce and the CMAC tuple. If the verification fails, then HR-MS2 ignores DirectCommsPeer_KeyAgreement_MSG_#3 message. If the verification is correct, then HR-MS2 confirms that HR-MS1 has computed the correct keys and commence secure direct communications. If HR-MS2 does not receive DirectCommsPeer_KeyAgreement_MSG_#3 message from HR-MS1 within DirectCommsPeer_KeyAgreement_MSG_#2 Timeout, it shall resend the DirectCommsPeer_KeyAgreement_MSG_#2 message up to DirectCommsPeer_KeyAgreement_MSG_#2 MaxResends times. If HR-MS2 reaches its maximum number of resends, it shall initiate another authentication or drop the request. HR-MS1 and HR-MS2 can now derive DTEK and commence secure direct communications.

            - Figure 4—Flow Diagram of Authentication and Key Establishment of Direct Communication without Infrastructure Secure direct communication using public key infrastructure (HR-MS becomes HR-BS*case).

HR-MS1 sends a DirectCommsPeer_KeyAgreement_MSG_#1 message to HR-MS2 to request for a direct communication authentication and key establishment. The message contains a nonce, signature generated by HR-MS1, HR-MS1 and HR-MS2 addresses and HR-MS1’s digital certificate.

HR-MS2 verifies the received certificate and signature. If the verification is correct, HR-MS2 generates its nonce and DMK. Next, HR-MS2 computes DAK, DCMAC key and the CMAC tuple of the message. HR-MS2 then uses the public key of HR-MS1 to encrypt the DMK. Finally, HR-MS2 computes the signature and sends DirectCommsPeer_KeyAgreement_MSG_#2 message to HR-MS1. The message also contains HR-MS2’s digital certificate.

HR-MS1 verifies the received certificate and signature. If the verification is correct, then HR-MS1 decrypts the received encrypted message and obtains DMK. Next, HR-MS1 computes DAK and DCMAC key and verifies the received CMAC tuple. If the verification is correct, then HR-MS1 computes its CMAC tuple and sends DirectCommsPeer_KeyAgreement_MSG_#3 message to HR-MS2.

HR-MS2 receives the above message and verifies the CMAC tuple. If the verification is correct, then HR-MS2 confirms that HR-MS1 has computed the correct keys and both HR-MS1 and HR-MS2 derives DTEK and commence secure direct communications.

Figure 5—Flow Chart of PKI-based Autonomous Direct Communication Authentication and Key Establishment Security Procedure Secure direct communication using public key infrastructure.

              - Message type

Table 8— DC_Request_MSG#1 message attribute Secure direct communication using public key infrastructure message types

| Code | Key Agreement Type | Message Type | MAC control message name |
| --- | --- | --- | --- |
| 9 | 1 | DirectCommsPeer_KeyAgreement_MSG #1 | AAI-PKM-RSP |
| 10 | 1 | DirectCommsPeer_KeyAgreement_MSG #2 | AAI-PKM-REQ |
| 11 | 1 | DirectCommsPeer_KeyAgreement_MSG #3 | AAI-PKM-RSP |

              - Message attribute

Table 9— DirectCommsPeer_KeyAgreement_MSG_#1 message attribute

| Attribute | Contents |
| --- | --- |
| T HR-MS1 Timestamp_HR-MS1 | Timestamp generated by HR-MS1 |
| N HR-MS1 NONCE_HR-MS1 | Freshly generated random number of 64bits by HR-MS1 |
| HR-MS2Addr | Address of HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| signatureHR-MS1 SigHR-MS1 | Signature of message generated by HR-MS1 using its RSA private key |
| Cert(HR-MS1) HR-MS1_Certificate | Digital certificate of HR-MS1 |

Table 10— DirectCommsPeer_KeyAgreement_MSG_#2 message attribute

| Attribute | Contents |
| --- | --- |
| T HR-MS2 Timestamp_HR-MS2 | Timestamp generated by HR-MS2 |
| N HR-MS2 NONCE_HR-MS2 | Freshly generated random number of 64bits by HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| HR-MS2Addr | Address of HR-MS2 |
| N HR-MS1 NONCE_HR-MS1 | Nonce generated by HR-MS1 in DirectCommsPeer_KeyAgreement_MSG_#1 message |
| EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) Encrypted DMK | Public key encryption EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) using HR-MS1's Public key where DMK = DirectComms Direct Communications Master Key generated by HR-MS and key_lifetime = lifetime of DMK |
| CMACHR-MS2 CMAC_HR-MS2 | Message digest calculated using DCMAC key by HR-MS2 |
| signatureHR-MS2 SigHR-MS2 | Signature of message generated by HR-MS2 using its RSA private key |
| Cert(HR-MS2) HR-MS2_Certificate | Digital certificate of HR-MS2 |

Table 11— DirectCommsPeer_KeyAgreement_MSG_#3 message attribute

| Attribute | Contents |
| --- | --- |
| N HR-MS2 NONCE_HR-MS2 | Nonce generated by HR-MS2 in DirectComms\_KeyAgreement\_MSG\_\#2 Peer_KeyAgreement_MSG_#2 message |
| HR-MS2Addr | Address of HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| CMACHR-MS1 CMAC_HR-MS1 | Message digest calculated using DCMAC key by HR-MS1 |

###### Security context for BS-coordinate secure direct communication

The direct communications security context describes the set of parameters that links the direct communication security keys for BS-coordinate secure direct communications.

            - DMK context

The DMK context includes all parameters associate with the DMK. This context is created when the DMK is derived.

The DMK context is described in Table 12.

Table 12—The DMK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DMK | 160 | Multicast Direct Communication Master Key shared by HR-MS1 and HR-MS2 HR-BS and HR-MSs in a multicast group |
| DMK SN | 4 | DMK sequence number |
| DMK Lifetime | 32 | MMK DMK Lifetime |
| DAK_COUNT | 16 | Counter to ensure freshness of computed CMAC key and prevent replay attacks. |

            - DAK context

The DAK context includes all parameters associate with the DAK. This context is created whenever a new DAK is derived. This context shall be deleted when the DAK is not in used.

The DAK context is described in Table 13.

Table 13—The DAK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DAK | 160 | Direct Communications Authentication Key derived from DMK. |
| DAK Lifetime | 32 | DAK Lifetime |
| DAKID | 64 | Identifies the DAK key. |
| DCMAC_KEY | 128 | Key which is used for signing Direct Communications MAC control messages. |
| DCMAC_PN | 24 | Used to avoid multicast direct communication replay attack on the control connection. The initial value of DCMAC_PN is zero. |
| DAK_COUNT | 16 | Counter to ensure freshness of computed DCMAC key and prevent replay attacks. |

            - DSA context

The DSA context is the set of parameters managed by each DSA in order to ensure DTEK management and usage in a secure way for BS-coordinated secure direct communications.

The DSA holds the DTEK context and additional information that belongs to the DSA itself.

            - DTEK context

The DTEK context includes all parameters of the DTEK and is described in Table 14.

Table 14—The DTEK context

| Parameter | Size (bit) |  | Usage |
| --- | --- | --- | --- |
| DTEK | 128 |  | Key used for encryption or decryption of direct communications messages |
| DMK SN |  | 4 | DMK sequence number |
| COUNTER_DTEK | 16 |  | The counter used to derive this DTEK |
| DTEK lifetime | 32 |  | DTEK lifetime=DMK lifetime |
| DTEK_PN | 22 |  | The PN used for encrypting multicast direct communication packets. After each Multicast Direct Communication MAC PDU transmission, the value shall be increased by 1. (0x000000-0x1FFFFF) |

            - DSA context

The DSA context is described in Table 15.

Table 15—The DSA context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DSAID | 8 | The identifier of this DSA, which decribes describes the applied encryption/decryption method and DTEK contexts. |
| DTEK context | Sizeof(DTEK context) | DTEK context for encryption and decryption |

###### Key derivation for BS-coordinated secure direct communication

The key hierarchy defines what keys are present in the system for BS-coordinated secure direct communication and how the keys are generated.

            - DMK derivation

The DMK is the security key/pre-established shared key that is randomly generated by HR-BS or HR-MS or a network entity (e.g. an AAA Server etc). The DMK is a 160-bit key.

The DMK may be used as a source for keying materials required by upper layers.

The DMK is generated using EAP methods and transferred to the HR-MSs via EAP-Transfer messages.

The DMK is used to derive the Direct Communication Authentication Key (DAK).

            - DAK derivation

DAK is derived from DMK and belongs to a pair of HR-MSs. The DAK is used for BS-coordinated Direct Communications in the event of failure in the backbone.

The DAK derivation is as follows:

DAK =Dot16KDF (DMK, HR-MS1Addr|HR-MS2Addr|”DAK”, 160)

where: HR-MS1Addr and HR-MS2Addr are the addresses of HR-MS1 and HR-MS2 respectively.

The DCMAC-DTEK prekey is derived from DAK and is used to derive other keys:

- Direct Communication Cipher-based Message Authentication Code (DCMAC) key

- Direct Communication Traffic Encryption (DTEK) Key

The DCMAC-DTEK prekey derivation is done as follows:

DCMAC-DTEK prekey = Dot16KDF (DAK, DAK_COUNT|”DCMAC-DTEK prekey”, 160)

            - DCMAC key derivation

DCMAC key is derived from DAK and used for message authentication for the messages sent during BS-coordinated secure direct communications.

DCMAC key is derived as follows:

DCMAC key = Dot16KDF(DCMAC-DTEK prekey, “DCMAC_KEYS”, 128)

            - DTEK derivation

DTEK is the transport encryption key used to encrypt data in BS-coordinated secure direct communications.

DTEK is derived as follows:

DTEK = Dot16KDF(DCMAC-DTEK prekey, DSAID|COUNTER_DTEK|”DTEK_KEY”, 128)

Where

DSAID is the security association to which the DTEK belongs.

COUNTER_DTEK is a counter used to derive different DTEKs for the same DSAID, the value of the counter is changed everytime a new DTEK needs to be derived within the same DAK and DAK_COUNT pair is valid. Everytime a new DCMAC-DTEK prekey is derived, this counter is reset.

##### Security procedure for Talk-around secure direct communication

In order to support secure direct communication between two or among more HR-MSs, pre-established shared key is used.

PKMv3 provides HR-MSs with strong protection from theft of service by encrypting talk-around direct connections between two or among HR-MSs.

If a talk-around direct communication connection is to be encrypted, each HR-MS participating in the connection shall have an additional security association (SA) (i.e., talk-around direct communication SA), allowing that connection to be encrypted using keys that are independent of those used for other encrypted transmissions between HR-MSs.

Talk-around direct communication traffic can be encrypted using talk-around direct communication specific key management based on PKMv3.