| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures for Establishing Secured Communications between HR-BS in IEEE 802.16n |  |
| Date Submitted | 2011-10-31-11-10 |  |
| Source(s) | Yeow Wai Leong, Joseph Teo Chee Ming, Jaya Shankar, , Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Contribution for Security Procedures in IEEE 802.16n |  |
| Abstract | Detail description of Security procedures for Direct Communications to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Procedures for Establishing Secured Communications between HR-BS in IEEE 802.16n

Yeow Wai Leong, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research

# Introduction

This contribution contains the detailed proposed text from the procedures for establishing secured Communications between HR-BS in IEEE 802.16n. It is to be proposed to place into the AWD text for the IEEE 802.16n GridMAN standards (802.16.1 baseline AWD).

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Modify the following text into Section 6.12.10 Security in the 802.16.1a AWD accordingly]

6.12.10 Support for Security

### 6.12.10.3 Secure forwarding between HR-infrastructure stations using Public Key Infrastructure

This procedure provides strong protection and data integrity when data is forwarded between HR-infrastructure stations as described in Section 6.12.7.3 using Public Key Infrastructure. Each HR-BS has a public/private key pair and digital certificate (e.g. X.509) issued by a certification authority for mutual authentication and key exchange prior to the start of this direct communications.

The key agreement handshake procedure described below shall be used for HR-BSs to mutually authenticate themselves (without access to a security server) using Public Key Infrastructure and to derive data security keys for secure direct communications. A control connection between HR-BSs via a designated forwarding HR-MS shall be used for transmitting the key agreement messages. The flow diagram for this scenario is depicted in Figure XX and the Flow Chart for this scenario is shown in Figure XX+1.

### 6.12.10.3.1 Key Agreement Handshake Procedure

The key agreement handshake procedure using Public Key Infrastructure includes the following steps:

Step 1: Prior to secure connection establishment as described in Section 6.12.7.3.4, the degraded HR-BS (HR-BSd) shall generate nonce NONCE_HR-BSd. Next, HR-BSd computes the signature SigHR-BSd = SIGN(Timestamp_HR-BSd|NONCE_HR-BSd|HR-BS MAC Addr|HR-BSd MAC Addr) and sends BStoBS_KeyAgreement_MSG_#1 message to Designated HR-BS, where BStoBS_KeyAgreement_MSG_#1 = Timestamp_HR-BSd|NONCE_HR-BSd|HR-BS MAC Addr|HR-BSd MAC Addr| SigHR-BSd|HR-BSd_Certificate.

Step 2: HR-BS first verifies the received timestamp and nonce for freshness and the certificate HR-BSd_Certificate and signature SigHR-BSd. If the verifications fail, then HR-BS ignores the BStoBS_KeyAgreement_MSG_#1 message. If the verifications are correct, then HR-BS generates nonce NONCE_HR-BS and security key B2MK and computes B2AK =Dot16KDF (B2MK, HR-BSd MAC Addr|HR-BS MAC Addr| “B2AK”, 160) and the B2CMAC key and CMAC_HR-BS = MACB2CMAC(NONCE_HR-BS|NONCE_HR-BSd|HR-BS MAC Addr|HR-BSd MAC Addr). HR-BS then uses HR-BSd's public key to encrypt and obtain Encrypted B2MK = EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr). Finally, HR-BS computes signature SigHR-BS = SIGN(THR-BS|NONCE_HR-BS|HR-BSd MAC Addr|HR-BS MAC Addr|NONCE_HR-BSd| Encrypted B2MK |CMAC_HR-BS) and sends BStoBS_KeyAgreement_MSG_#2 message to HR-BSd, where BStoBS_KeyAgreement_MSG_#2  = THR-BS|NONCE_HR-BS|HR-BSd MAC Addr|HR-BS MAC Addr|NONCE_HR-BSd| Encrypted B2MK |CMAC_HR-BS|SigHR-BS|HR-BS_Certificate.

Step 3: HR-BSd first verifies the received timestamp and nonces for freshness and the certificate HR-BS_Certificate and signature signature HR-BS. If the verification is invalid, then HR-BSd ignores the BStoBS_KeyAgreement_MSG_#2 message. If the verifications are correct, then HR-BSd decrypts Encrypted B2MK and obtains security key B2MK and key_lifetime. Next, HR-BSd computes B2AK and B2CMAC keys and verifies CMAC_HR-BS. If the verification is invalid, then HR-BSd ignores the BStoBS_KeyAgreement_MSG_#2 message. If the verification is correct, then HR-BSd computes CMAC_HR-BSd = MACB2CMAC(NONCE_HR-BSd|NONCE_HR-BS|HR-BSd MAC Addr|HR-BS MAC Addr) and sends BStoBS_KeyAgreement_MSG_#3 message to HR-BS, where BStoBS_KeyAgreement_MSG_#3 = NONCE_HR-BS|HR-BS MAC Addr|HR-BSd MAC Addr|CMAC_HR-BSd. If HR-BSd does not receive BStoBS_KeyAgreement_MSG_#2 message from HR-BS within BStoBS_KeyAgreement_MSG_#1 Timeout, it shall resend the BStoBS_KeyAgreement_MSG_#1 message up to BStoBS_KeyAgreement_MSG_#1 MaxResends times. If HR-BSd reaches its maximum number of resends, it shall initiate another authentication or drop the request.

Step 4: HR-BS receives the BStoBS_KeyAgreement_MSG_#3 message and verifies received nonce and the CMAC tuple. If the verification fails, then HR-BS ignores BStoBS_KeyAgreement_MSG_#3 message. If the verification is correct, then HR-BS confirms thaTimestamp_HR-BSd has computed the correct keys and commence secure direct communications. If HR-BS does not receive BStoBS_KeyAgreement_MSG_#3 message from HR-BSd within BStoBS_KeyAgreement_MSG_#2 Timeout, it shall resend the BStoBS_KeyAgreement_MSG_#2 message up to BStoBS_KeyAgreement_MSG_#2 MaxResends times. If HR-BS reaches its maximum number of resends, it shall initiate another authentication or drop the request. HR-BSd and HR-BS can now derive B2TEK and commence secure connection establishment as described in Section 6.12.7.3.4.

Figure XX—Flow Diagram of Key Agreement Handshake Procedure for Secure forwarding between HR-infrastructure stations using Public Key Infrastructure.

HR-BSd sends a Peer_KeyAgreement_MSG_#1 message to HR-BS to request for a BS-to-BS authentication and key establishment. The message contains a nonce, signature generated by HR-BSd, HR-BSd and HR-BS addresses and HR-BSd’s digital certificate.

HR-BS verifies the received certificate and signature. If the verification is correct, HR-BS generates its nonce and B2MK. Next, HR-BS computes B2AK, B2CMAC key and the CMAC tuple of the message. HR-BS then uses the public key of HR-BSd to encrypt the B2MK. Finally, HR-BS computes the signature and sends Peer_KeyAgreement_MSG_#2 message to HR-BSd. The message also contains HR-BS’s digital certificate.

HR-BSd verifies the received certificate and signature. If the verification is correct, then HR-BSd decrypts the received encrypted message and obtains B2MK. Next, HR-BSd computes B2AK and B2CMAC key and verifies the received CMAC tuple. If the verification is correct, then HR-BSd computes its CMAC tuple and sends Peer_KeyAgreement_MSG_#3 message to HR-BS.

HR-BS receives the above message and verifies the CMAC tuple. If the verification is correct, then HR-BS confirms that HR-BSd has computed the correct keys and both HR-BSd and HR-BS derives B2TEK and commence secure direct communications.

Figure XX+1—Flow Chart of Key Agreement Handshake Procedure for Secure forwarding between HR-infrastructure stations using Public Key Infrastructure.

# YY.YY.YY.YY Message Type

Table YY— Secure forwarding between HR-infrastructure stations using Public Key Infrastructure message type

| Code | Key Agreement Type | Message Type | MAC control message name |
| --- | --- | --- | --- |
| 9 | 2 | Peer_KeyAgreement_MSG #1 | AAI-PKM-RSP |
| 10 | 2 | Peer_KeyAgreement_MSG #2 | AAI-PKM-REQ |
| 11 | 2 | Peer_KeyAgreement_MSG #3 | AAI-PKM-RSP |

# 6.12.10.3.2 Message Attribute

Table ZZ—Peer_KeyAgreement_MSG_#1 message attribute

| Attribute | Contents |
| --- | --- |
| Timestamp_HR-BSd | Timestamp generated by HR-BSd |
| NONCE_HR-BSd | Freshly generated random number of 64bits by HR-BSd |
| HR-BS MAC Addr | MAC Address of HR-BS |
| HR-BSd MAC Addr | MAC Address of HR-BSd |
| SigHR-BSd | Signature of message generated by HR-BSd using its RSA private key |
| HR-BSd_Certificate | Digital certificate of HR-BSd |

Table ZZ+1—Peer_KeyAgreement_MSG_#2 message attribute

| Attribute | Contents |
| --- | --- |
| Timestamp_HR-BS | Timestamp generated by HR-BS |
| NONCE_HR-BS | Freshly generated random number of 64bits by HR-BS |
| HR-BSd MAC Addr | MAC Address of HR-BSd |
| HR-BS MAC Addr | MAC Address of HR-BS |
| NONCE_HR-BSd | Nonce generated by HR-BSd in BStoBS_KeyAgreement_MSG_#1 message |
| Encrypted B2MK | Public key encryption EHR-BSd_PK(B2MK, key_lifetime, HR-BSd MAC Addr, HR-BS MAC Addr) using HR-BSd's Public key where B2MK = BStoBS Master Key generated by HR-MS and key_lifetime = lifetime of B2MK |
| CMAC_HR-BS | Message digest calculated using B2CMAC key by HR-BS |
| SigHR-BS | Signature of message generated by HR-BS using its RSA private key |
| HR-BS_Certificate. | Digital certificate of HR-BS |

Table ZZ+2—Peer_KeyAgreement_MSG_#3 message attribute

| Attribute | Contents |
| --- | --- |
| NONCE_HR-BS | Nonce generated by HR-BS in BStoBS_KeyAgreement_MSG_#2 message |
| HR-BS MAC Addr | MAC Address of HR-BS |
| HR-BSd MAC Addr | MAC Address of HR-BSd |
| CMAC_HR-BSd | Message digest calculated using B2CMAC key by HR-BSd |

# 17.2.10.1.1.3 Security Context for Secure forwarding between HR-infrastructure stations using Public Key Infrastructure

The BS to BS communications security context describes the set of parameters that links the BS to BS security keys for secure forwarding between HR-infrastructure stations.

### 17.2.10.1.1.3.1 B2MK context

The B2MK context includes all parameters associate with the B2MK. This context is created when the B2MK is derived.

The B2MK context is described in Table 1.

Table 1—The B2MK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| B2MK | 160 | BS-to-BS Master Key shared by HR-BS and HR-BSd |
| B2MK SN | 4 | B2MK sequence number |
| B2MK Lifetime | 32 | B2MK Lifetime |
| B2AK_COUNT | 16 | Counter to ensure freshness of computed CMAC key and prevent replay attacks. |

### 17.2.10.1.1.3.2 B2AK context

The B2AK context includes all parameters associate with the B2AK. This context is created whenever a new B2AK is derived. This context shall be deleted when the B2AK is not in used.

The B2AK context is described in Table 2.

Table 2—The B2AK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| B2AK | 160 | BS-to-BS Communications Authentication Key derived from B2MK. |
| B2AK Lifetime | 32 | B2AK Lifetime |
| B2AKID | 64 | Identifies the B2AK key. |
| B2CMAC_KEY | 128 | Key which is used for signing BS-to-BS Communications MAC control messages. |
| B2CMAC_PN | 24 | Used to avoid BS-to-BS communication replay attack on the control connection. The initial value of B2CMAC_PN is zero. |
| B2AK_COUNT | 16 | Counter to ensure freshness of computed B2CMAC key and prevent replay attacks. |

### 17.2.10.1.1.3.3 B2SA context

The B2SA context is the set of parameters managed by each B2SA in order to ensure B2TEK management and usage in a secure way for secure forwarding between HR-infrastructure stations.

The B2SA holds the B2TEK context and additional information that belongs to the B2SA itself.

### 17.2.10.1.1.3.4 B2TEK context

The B2TEK context includes all parameters of the B2TEK and is described in Table 3.

Table 3—The B2TEK context

| Parameter | Size (bit) |  | Usage |
| --- | --- | --- | --- |
| B2TEK | 128 |  | Key used for encryption or decryption of BS-to-BS communications messages |
| B2MK SN |  | 4 | B2MK sequence number |
| COUNTER_B2TEK | 16 |  | The counter used to derive this B2TEK |
| B2TEK lifetime | 32 |  | B2TEK lifetime=B2MK lifetime |
| B2TEK_PN | 22 |  | The PN used for encrypting BS-to-BS communication packets. After each BS-to-BS communication MAC PDU transmission, the value shall be increased by 1. (0x000000-0x1FFFFF) |

### 17.2.10.1.1.3.5 B2SA context

The B2SA context is described in Table 4.

Table 4—The B2SA context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| B2SAID | 8 | The identifier of this B2SA, which decribes the applied encryption/decryption method and B2TEK contexts. |
| B2TEK context | Sizeof(B2TEK context) | B2TEK context for encryption and decryption |

# 17.2.10.1.1.4 Key Derivation for Secure forwarding between HR-infrastructure stations using Public Key Infrastructure

The key hierarchy defines what keys are present in the system for secure forwarding between HR-infrastructure stations and how the keys are generated.

17.2.10.1.1.4.1 B2MK Derivation

The B2MK is the security key that is randomly generated by HR-BS. The B2MK is a 160-bit key.

The B2MK may be used as a source for keying materials required by upper layers.

The B2MK is used to derive the BS-to-BS Communication Authentication Key (B2AK).

17.2.10.1.1.4.2 B2AK Derivation

B2AK is derived from B2MK and belongs to a pair of HR-BSs (HR-BSd and HR-BS). The B2AK is used for secure forwarding between HR-infrastructure stations as described in Section 6.12.7.3.

The B2AK derivation is as follows:

B2AK =Dot16KDF (B2MK, HR-BSd MAC Addr|HR-BS MAC Addr|”B2AK”, 160)

where: HR-BSd MAC Addr and HR-BS MAC Addr are the addresses of HR-BSd and HR-BS respectively.

The B2CMAC-B2TEK prekey is derived from B2AK and is used to derive other keys:

- BS-to-BS Communication Cipher-based Message Authentication Code (B2CMAC) key

- BS-to-BS Communication Traffic Encryption (B2TEK) Key

The B2CMAC-B2TEK prekey derivation is done as follows:

B2CMAC-B2TEK prekey = Dot16KDF (B2AK, B2AK_COUNT|”B2CMAC-B2TEK prekey”, 160)

17.2.10.1.1.4.3 B2CMAC Key Derivation

B2CMAC key is derived from B2AK and used for message authentication for the messages sent during secure forwarding between HR-infrastructure stations.

B2CMAC key is derived as follows:

B2CMAC key = Dot16KDF(B2CMAC-B2TEK prekey, “B2CMAC_KEYS”, 128)

17.2.10.1.1.4.4 B2TEK Derivation

B2TEK is the transport encryption key used to encrypt data in secure forwarding between HR-infrastructure stations.

B2TEK is derived as follows:

B2TEK = Dot16KDF(B2CMAC-B2TEK prekey, B2SAID|COUNTER_B2TEK|”B2TEK_KEY”, 128)

Where

B2SAID is the security association to which the B2TEK belongs.

COUNTER_B2TEK is a counter used to derive different B2TEKs for the same B2SAID, the value of the counter is changed everytime a new B2TEK needs to be derived within the same B2AK and B2AK_COUNT pair is valid. Everytime a new B2CMAC-B2TEK prekey is derived, this counter is reset.

Table 726 —AAI-PKM-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 Reauth-Request; PKM v3 message code = 1 - PKMv3 EAP-Transfer; PKM v3 message code = 2 -PKMv3 Key_Agreement-MSG#2; PKM v3 message code = 4 - PKMv3 TEK-Request; PKM v3 message code = 6 - PKMv3 TEK-Invalid; PKM v3 message code =8 9–16: Reserved - Peer_KeyAgreement_MSG #2; PKM v3 message code = 10 12–16: Reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests |  |
| CMAC indicator | 1 | Indicates whether this message is protected by CMAC tuple 0: Not protected 1: Protected | Shall always be present |
| If( PKM v3 message code ==2) { |  |  |  |
| EAP payload | variable (1..1400 x 8) | Contains the EAP authentication data, not interpreted in the MAC |  |
| } |  |  |  |
| If( PKM v3 message code == 4) { |  |  |  |
| … |  |  |  |
| … |  |  |  |
| If( PKM v3 message code == 8 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| } |  |  |  |
| If( PKM v3 message code == 10) { |  |  |  |
| Key Agreement Type | 8 | Indicates whether this message is for which type of Direct communications key agreement 2: BS-to-BS security 3-255: Reserved |  |
| If(Key Agreement Type == 2) { |  |  |  |
| Timestamp_HR-BS | 32 | Timestamp |  |
| NONCE_HR-BS | 64 | A random number of 64 bits used for freshness |  |
| HR-BSdAddr | 48 | MAC Address |  |
| HR-BSAddr | 48 | MAC Address |  |
| NONCE_HR-BSd | 64 | A random number of 64 bits used for freshness |  |
| Encrypted B2MK | 1024 | Public key encryption using HR-MS1's Public key |  |
| SigHR-BS | 1024 | Signature of message generated by using its RSA private key |  |
| HR-BS_Certificate | 1024 | RSA Digital certificate |  |
| }} |  |  |  |

Table 727 —AAI-PKM-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 EAP-Transfer; PKM v3 message code =2 - PKMv3 Key_Agreement-MSG#1; PKM v3 message code =3 - PKMv3 Key_Agreement-MSG#3; PKM v3 message code =5 - PKMv3 TEK-Reply; PKM v3 message code =7 - PKMv3 TEK-Invalid; PKM v3 message code =8 9–16: Reserved - Peer_KeyAgreement_MSG #1; PKM v3 message code = 9 - Peer_KeyAgreement_MSG #3; PKM v3 message code = 11 12–16: Reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests or an HR-MS response to another HR-MS request |  |
| CMAC indicator | 1 | Indicates whether this message is protected by CMAC tuple 0: Not protected 1: Protected | Shall always be present |
| If( PKM v3 message code ==2) { |  |  |  |
| EAP payload | variable (1..1400 x 8) | Contains the EAP authentication data, not interpreted in the MAC |  |
| } |  |  |  |
| If( PKM v3 message code == 3) { |  |  |  |
| … |  |  |  |
| … |  |  |  |
| } |  |  |  |
| If( PKM v3 message code == 8 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| } |  |  |  |
| If( PKM v3 message code == 9) { |  |  |  |
| Key Agreement Type | 8 | Indicates whether this message is for which type of Direct communications key agreement 2: BS-to-BS security 3-255: Reserved |  |
| If(Key Agreement Type == 2) { |  |  |  |
| Timestamp_HR-BSd | 32 | Timestamp |  |
| NONCE_HR-BSd | 64 | A random number of 64 bits used for freshness |  |
| HR-BSAddr | 48 | MAC Address |  |
| HR-BSdAddr | 48 | MAC Address |  |
| SigHR-BSd | 1024 | Signature of message generated by using its RSA private key |  |
| HR-BSd_Certificate | 1024 | RSA Digital certificate |  |
| }} |  |  |  |
| If( PKM v3 message code == 11) { |  |  |  |
| Key Agreement Type indicator | 8 | Indicates whether this message is for which type of Direct communications key agreement 2: BS-to-BS security 3-255: Reserved |  |
| If(Key Agreement Type indicator == 2) { |  |  |  |
| NONCE_HR-BS | 64 | A random number of 64 bits used for freshness |  |
| HR-BSAddr | 48 | MAC Address |  |
| HR-BSdAddr | 48 | MAC Address |  |
| }} |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]