| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text amendment for Alternative path procedures for IEEE 802.16Rev3 |  |
| Date Submitted | 2011-10-31-11-10 |  |
| Source(s) | Wai Leong Yeow, Shoukang Zheng, Anh Tuan Hoang, Haiguang Wang, Jaya Shankar I2R Vinh Dien Hoang NICT | E-mail: wlyeow@i2r.a-star.edu.sg athoang@i2r.a-star.edu.sg hwang@i2r.a-star.edu.sg jshankar@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Alternative path management for IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text amendment for Alternative path procedures for IEEE 802.16Rev3

Wai Leong Yeow, Shoukang Zheng, Anh Tuan Hoang, Haiguang Wang, Jaya Shankar

Institute for Infocomm Research

Vinh Dien Hoang

NICT

# Introduction

After reorganization of the table of contents and the text pertaining to alternative path management in Section 16.7.1 in the last AWD revision, the text is fragmented and not clear. The objectives of this contribution are two-fold:

- Reorganize the text in the main and sub-sections of 16.7.1 and provide additional references to section numbers of the IEEE 802.16Rev3 document for improved clarity and readability.

- Improve alternative path preparation with infrastructure station scanning and more cases for MAC context sharing.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Amend the following text of 802.16Rev3 AWD.]

#### Alternative path management

Alternative path may be prepared and maintained in the following cases:

- before the SPOF occurs if SPOF is predicted or needed

- when the SPOF occurs with/without any preparing

- after the SPOF is recovered, to continue supporting high reliability

##### Alternative path preparing preparation

To prepare alternative path, the MAC context information of HR-MS may be shared between following HR-stations:

- HR-infrastructure stations (i.e., serving and neighbor HR-infrastructure stations)

- An HR-MS capable of forwarding to the network and the HR-MS performing forwarding to the network

To support fast recovery in the event of SPOF, an indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

To support fast network reentry to the neighbor HR-MSs, either HR-BS or HR-MS may prepare the alternative path using neighbor discovery described in 16.2.2.1.1.

To support fast recovery in the event of SPOF, an indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

To support fast network reentry to the neighbor HR-MSs,

To prepare the alternative path by an HR-BS, the HR-BS performs operation as follows:

- neighbor discovery as described in 16.2.2.1.1

- collecting HR-MS’ neighbor information as described in 16.2.2.1.1

- determining the alternative path for HR-MS

- informing HR-MS about its alternative path information

To prepare the alternative path by an HR-MS, the HR-MS and its serving HR-BS perform operation as follows:

- An HR-MS transmits AP-NBR-REQ to the HR-BS to initiate the neighbor discovery process

- After the HR-BS receives the AP-NBR-REQ, the HR-BS performs the neighbor discovery as described in 16.2.2.1.1

- The HR-BS collects the neighbor information of requesting HR-MS as described in 16.2.2.1.1

- The HR-BS transmits AP-NBR-REP message to HR-MS, where the AP-NBR-REP includes the neighbor information of requesting HR-MS.

- The HR-MS determines alternative path by itself based on its received neighbor information in the AP-NBR-REP message, but how to determine is out of scope of this specification.

An HR-BS or an HR-MS shall prepare the alternative path through scanning of neighbor infrastructure stations described in 6.3.21.1.2 and/or using neighbor discovery for neighbor HR-MSs described in 16.2.2.1.1.

An HR-BS prepares the alternative path for an HR-MS by performing the following operations:

- initiates scanning of infrastructure stations described in 6.3.21.1.2 and/or performs neighbor discovery as described in 16.2.2.1.1

- collects HR-MS’ scan report as described in 6.3.21.1.2 or neighbor information as described in 16.2.2.1.1

- determines the alternative path for HR-MS

- informs HR-MS and target HR-infrastructure station about the alternative path information

An HR-MS prepares its alternative path by performing the following operations:

- An HR-MS transmits the AP-NBR-REQ message to its serving HR-BS to request for neighbor discovery

- After the serving HR-BS receives the AP-NBR-REQ message, it performs neighbor discovery as described in 16.2.2.1.1

- The serving HR-BS collects the neighbor information of requesting HR-MS as described in 16.2.2.1.1

- The serving HR-BS transmits AP-NBR-REP message to HR-MS, where the AP-NBR-REP message includes the neighbor information of requesting HR-MS.

- The HR-MS determines alternative path by itself based on its received neighbor information in the AP-NBR-REP message, but how to determine is out of scope of this specification.

##### Alternative path information and switching to HR-infrastructure station

The alternative path information is transmitted to HR-MSs by the serving HR-infrastructure station using the MOB_BSHO-REQ message with mode = 0b111 and one target HR-infrastructure station. In addition, the message facilitates a fast network reentry procedure upon SPOF defined by a trigger condition, as described below.

When the trigger condition specified in the TLV of the last MOB_BSHO-REQ described in 6.3.2.3.47 is met, the alternative path is activated by an HR-MS. If the action time is non-zero, the HR-MS shall perform the fast network reentry as described in 6.3.21.2.7 after the action time expires. The action time allows the target HR-station of the alternative path to perform mode change into an HR-infrastructure station when necessary.

To support switching to alternative path with fast network reentry, the serving HR-BS transmits MOB_BSHO-REQ message with mode = 0b111 to the HR-MS shall include a recommended ranging code and slot from the ranging region of the target HR-infrastructure station of the alternative path. This reduces contention in ranging when multiple HR-MSs perform fast network recovery in the event of SPOF. However, how to select the ranging code and slot is out of this specification.

The MAC context information of HR-MS may be shared between the serving and target HR-infrastructure stations to facilitate optimized network reentry through the alternative path. An indication of whether MAC context information of the subordinate HR-MS is shared by infrastructure stations shall be indicated in the MOB_BSHO-REQ message.

The alternative path information may be updated with a new MOB_BSHO-REQ message.

The target HR-BS of the alternative path may share MS context information with the serving HR-BS and recommend a ranging code and slot from the ranging region to facilitate fast network reentry and reduce contention during ranging. However, how to request and recommend is out of this specification.

##### Alternative path information and switching to forwarding HR-MS

When HR-MS is disconnected with HR-BS in its current path, HR-MS start switching to its alternative path by scanning for DL preambles for possible network entry.

After receiving the preamble and NCI, HR-MS uses the suggested ranging code and transmit the ranging code on the specified ranging channel to complete the network entry process.

##### Alternative path maintaining maintenance

Alternative path may be selected during the role change or release the mode as described in 16.1.

An alternative path may be updated or changed with a new MOB_BSHO-REQ message. When a mode-change or mode-release occurs as described in 16.1, new alternative path may be selected.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]