| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Messages format for alternative path in Path Discovery and Management |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for contributions for the 802.16n Amendment Working Draft |  |
| Abstract | This provides a proposal of establishment and maintenance of alternative path in 802.16n networks (Original document in Session #72: C80216n-11_0026.doc) |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Message formats for determining alternative path in path management of 802.16n-GRIDMAN networks

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# In this proposal, we propose a change in AP-NBR-REQ and AP-NBR-REP message format.

# Scenario

An example of a HR-Network in operation, all HR-MS associate with HR-BS

Current message format did not take into account the case of 0 hop. It also did not conform with SRD about multi-hop cases.

# Text Proposal in AWD

The text in BLACK color: the existing text in the Amendment Draft Standard

The text in RED color: the removal of existing Amendment Draft Standard Text

The text in BLUE color: the new text added to the Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the following text into 802.16rev3 AWD]

[Delete the Condition column of Table 1010 and Table 1011]

Table 1010 ---- AP-NBR-REQ message field and description

| FieldSyntax | Size (bits) | Value/DescriptionNotes | Condition |
| --- | --- | --- | --- |
| AP-NBR-REQ_Message(){ |  |  |  |
| Discovery mode requested | 2 | 0b00: Normal neighbor discovery 0b01: Discovery to prepare for BS determine alternative path 0b10: Discovery to prepare for MS determine alternative path, HR-BS shall reply HR-MS in the message AP-NBR-REP. 0b11: reserve | In the case of 0b10, HR-BS shall reply HR-MS in the message AP-NBR-REP. |
| } |  |  |  |

Table 1 ---- AP-NBR-REP message field and description

| FieldSyntax | Size (bits) | Value/DescriptionNotes | Condition |
| --- | --- | --- | --- |
| AP-NBR-REP_Message(){ |  |  |  |
| For (i = 0; i < Number of neighbor HR-MSs; i ++){ |  |  |  |
| Index | 4 | Index of the neighbor HR-MSs |  |
| SINR | 4 | Indicates the received SINR of the ranging code from neighbor HR-MSs. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Hop | 2 | Number of hop to HR-BS 0b00 1 hop 0b01 2 hop 0b10 3 hop or more 0b11 4 hop Reserved |  |
| Reserved | 6 | Reserved |  |
| } |  |  |  |
| } |  |  |  |

[Insert the following text into Table 763pm2---- AP-NBR-REP message field and description  of 802.16.1 AWD]

Table 763pm2 ---- AP-NBR-REP message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i = 0; i < Number of neighbor HR-MSs; i ++){ |  |  |  |
| Index | 4 | Index of the neighbor HR-MSs |  |
| SINR | 4 | Indicates the received SINR of the ranging code from neighbor HR-MSs. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Hop | 2 | Number of hop to HR-BS 0b00 1 hop 0b01 2 hop 0b10 3 hop or more 0b11 4 hop Reserved |  |
| Reserved | 6 | Reserved |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.