| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Two-hop Operation to Relay Packets between Two TDC Links |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Eunkyung Kim, Miyoung Yun, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16.1a Draft AWD <80216n-11_0025> |  |
| Abstract | This provides AWD text proposals of two-hop operation to relay packets between two TDC links for talk-around direct communication in IEEE 802.16.1a |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Two-hop Operation to Relay Packets between Two TDC Links

Sungcheol Chang, Seokki Kim, Eunkyung Kim, Miyoung Yun, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

The following set of contributions provide ETRI’s proposals for talk-around direct communication:

- Modification of one-hop operations and MAC messages[1]

- New proposal of two-hop operation to relay packets between two links[2]

- Extension of talk-around direct communication between HR-MSs via HR-BS[3]

The 802.16.1a Draft AWD does not describe two-hop operation of talk-around direct communication, while it mention one hop operation of TDC. But 802.16n SRD requires 802.16n HR network to support two-hop operation of direct communication. Two HR-MSs shall communicate each other via relaying HR-MS using two direct communication links. This contribution provides two-hop operation of TDC in which a relaying HR-MS receives packets from an HR-MS in a slot of TDC frame and send the packets to a corresponding HR-MS in other slot of TDC frame. Figure 1 shows a configuration of two TDC links for relaying packets from a source HR-MS to a destination HR-MS via a relaying HR-MS.

Figure 1 Relaying packets between two TDC links

An relaying HR-MS broadcasts packets from a source HR-MS to HR-MSs within coverage and provides a fixed communication coverage for HR-MSs. The source HR-MS establishes an unicast transport connection to the relaying HR-MS and the relaying HR-MS broadcast packets from the source HR-MS. HR-MSs near the relaying HR-MS receive packets. Figure 2 shows a fixed service coverage is served by a relaying HR-MS. This communication scenario may be applicable to incidental area communication for disaster relief.

Figure 2 A relaying HR-MS broadcasts packets for relaying them to HR-MSs

PTT Token is a right that an HR-MS sends packets in turn. Token management of one-hop communication is described in the 802.16.1a Draft AWD. Two-hop token management mechanism is similar to one-hop token management, described in Section 6.12.2.3.1.8. Figure 3 shows that a relaying HR-MS manages a PTT token. When MS1 finishes sending packets and return its PTT tokens. MS1 sends the AAI-DC-TKN-HO message to the relaying MS. The relaying MS broadcasts PTT token status in the AAI-DC-TKN-ADV message. When getting a PTT token status, MS2 sends the AAI-DC-TKN-REQ message to request the PTT koen. The relaying MS sends the AAI-DC-TKN-RSP in response to the AAI-DC-TKN-REQ message. After that, the relaying MS sends the AAI-DC-TKN-HO message to MS2. MS2 is a traffic source and sends packets to the relaying MS.

Figure 3 Token management for two-hop operation of TDC links

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Add the following text in Section 6.12.2.3.1.11 in the 802.16.1a AWD]

6.12.2.3.1.11 Support for two-hop communication

For two-hop talk-around direct communication, an HR-MS relays packets from a source HR-MS to destination HR-MSs. The source HR-MS establishes a unicast TDC link to the relaying HR-MS. The relaying HR-MS establishes either a unicast TDC link or a multicast TDC link. Each TDC link of either unicast or multicast transmission works independently and additional functions required for two-hop operation are described in this section.

6.12.2.3.1.11.1 Advertising a relying HR-MS

An relaying HR-MS capable of relaying packets between two TDC links broadcast its information within a duration, Trelayadv using AAI-DC-RELAY-ADV message. If receiving DCTID of the relaying HR-MS, HR-MSs store it and used to request the relaying HR-MS to relay packets. An HR-MS does not start to send AAI-DC-RELAY-REQ message to the relaying HR-MS when Relay Status is ‘relaying packets’ and Relay Traffic Priority is higher than its traffic priority. An HR-MS may send AAI-DC-RELAY-REQ message to the relaying HR-MS when Relay Status is ‘available’.

An relaying HR-MS broadcasts a set of DCHs that are reserved for link establishment. The reserved DCHs are expected to establish a TDC link for relaying packets and are reserved by resource reservation algorithm with interference avoidance, which is described in 6.12.2.3.1.6.2. The other relaying HR-MSs avoid reserving DCHs in AAI-DC-RELAY-ADV message as many as possible.

6.12.2.3.1.11.2 Two links management

When an HR-MS need to communicate with corresponding HR-MSs via a relaying HR-MS, a source HR-MS shall request the relaying HR-MS to establish either one-way unicast TDC link or two-way unicast TDC link. The relaying HR-MS responds after establishing a TDC link to destination HR-MSs. During link establishments, a transport connection shall be established and multiple transport connections may be established.

The source HR-MS sends the AAI-DC-RELAY-REQ message to the relaying HR-MS and receives the AAI-DC-RELAY-RSP message from the relaying HR-MS in response to the AAI-DC-RELAY-REQ message. The AAI-DC-RELAY-REQ and AAI-DC-RELAY-RSP messages are piggybacked by AAI-DC-RTS and AAI-DC-CTS messages, respectively.

For release of a relaying link, the source HR-MS sends the AAI-DC-LREL-CMD message to the relaying HR-MS.

When the relaying HR-MS receives the AAI-DC-RELAY-REQ message from the source HR-MS, a TDC link is established between the relaying HR-Ms and one or more destination HR-Ms. The TDC link is either unicast or multicast one-way link, described in section 6.12.2.3.1.6.2.

After TDC link establishment from the relaying HR-MS to one or more destination HR-MSs, the relaying HR-MS shall send the AAI-DC-RELAY-RSP message in response to the AAI-DC-RELAY-REQ message. The relaying HR-MS may send multiple AAI-DC-RELAY-RSP messages with Confirmation Code == 0x02 to the source HR-MS until it sends the AAI-DC-RELAY-RSP message with Confirmation Code == 0x00 or 0x01.

6.12.2.3.1.11.2 Token management

For two TDC links of one-way unicast transmission, an HR-MS having a PTT token sends the AAI-DC-TKN-HO message and the relaying HR-MS relays it to a corresponding HR-MS. The corresponding HR-MS having the PTT Token sends packets.

For two TDC links of unicast and multicast transmissions, a relaying HR-MS manages a PTT token. The token is passed to HR-MSs and the relaying HR-MS using AAI-DC-TKN-HO message. The relaying HR-MS multicasts PTT token status using the AAI-DC-TKN-ADV message to a group of HR-MSs. When one of HR-MSs needs to get a PTT token, it sends the AAI-DC-TKN-REQ message to the relaying HR-MS. The relaying HR-MS sends the AAI-DC-TKN-RSP message in response to the AAI-DC-TKN-REQ message. If Confirmation Code==accept, the relaying HR-MS passes the PTT token to the requesting HR-MS using the AAI-DC-TKN-HO message. The HR-MS having the PTT token sends packets and the relaying HR-MS multicasts packets to the group of HR-MSs. Figure kkk shows that a relaying HR-MS manages a PTT token using the AAI-DC-TKN-HO message, the AAI-DC-TKN-REQ/RSP messages, and AAI-DC-TKN-ADV message.

Figure kkk Token management for two-hop operation of TDC links

## [Remedy2: Add the following text after Section 6.2.3.65.51 in the 802.16n AWD]

6.2.3.65.51 AAI-DC-RELAY-ADV

An relaying HR-MS transmits a AAI-DC-RELAY-ADV message to broadcast its information.

Table xxx – AAI-DC-RELAY-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Relay DCTID | 24 | DC Terminal Identifier. Indicate the HR-MS which relays packets between two TDC links |  |
| Relay Status | 1 | Indicate that the relaying HR-MS is available for two-hop operation. 0x0: available 0x1: relaying packets |  |
| Relay Traffic Priority | 3 | Indicate the traffic priority for relaying packets 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| For (i=0; i<N_Reserved_DCH; i++) { |  | N_Reserved_DCH is the number of dedicated channels for reservation |  |
| Direct Mode Zone Type for Reservation | 2 | Direct mode zone type of a reserved DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. |  |
| DCH Number for Reservation | 6 | Reserved DCH number on the zone of Direct Mode Zone Type |  |
| } |  |  |  |

6.2.3.65.51 AAI-DC-RELAY-REQ

An HR-MS transmits a AAI-DC-RELAY-REQ message to request a relaying HR-MS to establish a unicast TDC link

Table yyy – AAI-DC-RELAY-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Target DCTID or DCGID | 24 | Indicates a receiving HR-MS (Group) address. |  |

6.2.3.65.52 AAI-DC-RELAY-RSP

An HR-MS transmits a AAI- DC-RELAY-RSP message in response to a received AAI- DC-RELAY-REQ

message.

Table zzz – AAI-DC-RELAY-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | Link Change Count from the corresponding AAI-DCRELAY-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure. 0x0: accept 0x1: reject with a recommended DCH 0x2: wait next response message 0x3 to 0xF: reserved | Shall always be present |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. | Present if Confirmation Code == 0x1 |
| DCH Number | 6 | Recommended DCH number on zone of Direct Mode Zone Type | Present if Confirmation Code == 0x1 |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE C802.16n-11/0240, “Modification of one-hop operations and MAC messages.”

[2] IEEE C802.16n-11/0241, “New proposal of two-hop operation to relay packets between two links.”

[3] IEEE C802.16n-11/0242, “Extension of talk-around direct communication between HR-MSs via HR-BS.”