| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Base station function for HR-MS in IEEE 802.16Rev3 to be consistent with 802.16.1a |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Seokjoo Shin Chosun University Wai-Leong Yeow Institute for Infocomm Research | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr sjshin@chosun.ac.kr wlyeow@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16n-11/0020,” in response to Call for Comments on GRIDMAN AWD |  |
| Abstract | This contribution is a proposal related to multimode operation in IEEE 802.16Rev3 to be consistent with 802.16.1a. |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Base station function for HR-MS in IEEE 802.16Rev3 to be consistent with IEEE 802.16.1a

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Seokjoo Shin

Chosun University

Wai-Leong Yeow

Institute for Infocomm Research

- Introductions

At the session #75 meeting, Section 6.12.1.3 of IEEE 802.16.1a AWD has been updated by a contribution (IEEE C802.16n-11/0158r2). However, the contribution did not include a proposed text for IEEE 802.16Rev3. This contribution describes the proposed text related to multimode operation in IEEE 802.16Rev3 to be consistent with IEEE 802.16.1a.

- References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0024, “GRIDMAN Amendment Working Draft for 802.16rev3,” Oct 2011.

- IEEE 802.16n-11/0025, “GRIDMAN Amendment Working Draft for 802.16.1,” Oct 2011.

- Proposed Text for the 802.16n AWD for 802.16rev3

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adopt the following change in Section 16.1.3.1 in the GRIDMAN AWD for 802.16Rev3.]

16.1 Multi-mode operation

16.1.3 Base station function for HR-MS

16.1.3.1 Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS‘ status information such as the battery level. The superordinate HR-BS may transmit MM-ADV message with trigger condition for which the subordinate HR-MSs capable of role changing to HR-BS shall report its status information. When the trigger condition is met, the The subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP MAC control message. The triggering condition for reporting status information may be configured by the superordinate HR-BS.

After selecting the target HR-MS, the superordinate HR-BS requests the target HR-MS to change its mode to HR-BS by exchanging HRBS-REQ/RSP transmitting MM-BS-REQ message. The MM-BS-REQ message may include PHY operational parameters recommended by the superordinate HR-BS. If the target HR-MS accepts the request from the superordinate HR-BS to change the mode to HR-BS, the superordinate HR-BS may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters and the trigger conditions for operating as HR-BS. it shall transmit MM-BS-RSP message to the superordinate HR-BS when it is ready to start HR-BS role.

After receiving the MM-BS-RSP message, the superordinate HR-BS shall transmit MM-BS-CMD message to the target HR-MS to inform the action time or trigger conditions for starting as HR-BS mode. If the action time for mode change are included in the MM-BS-CMD message, the target HR-MS starts the HR-BS mode operation upon expiring the action time. At the same time, the superordinate HR-BS may stop the HR-BS role in order to avoid potential interference from the target HR-MS. If the trigger type is 0b1, trigger conditions for mode change shall be included in the MM-BS-CMD message and the mode change to HR-BS role starts after trigger event. One possible use is to prepare against an unforeseeable SPOF event, e.g., a HR-BS failure. The trigger shall be canceled if the target HR-MS performs a handover to other infrastructure stations.

If handover of subordinate MSs connected to the superordinate HR-BS is necessary, the superordinate HR-BS may transmit MS_Context-REQ message which contains MS context information before sending MM-BS-CMD message to the target HR-MS.

[Remedy2: Add the following rows in Table 54, section 6.3.2.3 in the GRIDMAN AWD for 802.16Rev3.]

| Type | Message Name | Message Description | Connection |
| --- | --- | --- | --- |
| xxx | MM-BS-REQ | Multimode Base station Request message |  |
| xxx+1 | MM-BS-RSP | Multimode Base station Response message |  |
| xxx+2 | MM-BS-CMD | Multimode Base station Command message |  |

[Remedy3: Add the following text into section 6.3.2.3.98 in the GRIDMAN AWD for 802.16Rev3.]

6.3.2.3.98 MAC management messages for HR-Network

6.3.2.3.98.x1 MM-BS-REQ (multimode base station request) message

To request to transfer HR-BS role to a multimode HR-MS, the MM-BS-REQ message shall be sent from superordinate HR-BS to a multimode HR-MS.

Table aaa – MM-BS-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MM-BS-REQ message format () { | — | — |
| Management Message Type = xx | 8 | — |
| Action code | 2 | Used to indicate the purpose of this message. 0b00: Request to transfer HR-BS role to the multimode HR-MS 0b01: Response for the unsolicited MM-BS-RSP message by the multimode HR-MS. 0b10: Reject for the unsolicited MM-BS-RSP message by the multimode HR-MS. 0b11: Reserved. |
| BS configuration indication | 2 | Indicates whether the superordinate HR-BS provides the recommended PHY parameters to multimode HR-MS or not. 0b00: There are no recommended PHY parameters. 0b01: There are no recommended PHY parameters. But, must be equal to the PHY operational parameters of the superordinate HR-BS. 0b10: There are recommended PHY parameters. 0b11: Reserved |
| Reserved | 4 | Shall be set to zero. |
| TLV encodings for MM-BS-REQ | variable | TLV-specific |
| } |  |  |

All parameters according to BS configuration indication are coded as TLV tuples, as defined in 11.xy.a.

The following parameter shall be included if BS configuration indication is set to 0b01 in MM-BS-REQ message:

DCD Configuration Change Count

Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change, except for the frame number for the OFDM PHY and the Available DL Radio Resources.

UCD Configuration Change Count

Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change, except for the Available UL Radio Resources.

The following parameter shall be included if BS configuration indication is set to 0b10 in the message:

Operator ID

Identifier of the network provider

PHY Profile ID

For systems using OFDM or OFDMA, the bit-by-bit definition of the PHY Profile ID is shown in Table 145.

FA index

Frequency assignment index. This field is present only if the FA index indicator in PHY profile ID is set. Otherwise, the HR-MS acting as BS has the same FA Index or the center frequency as the superordinate BS.

BS EIRP

Signed Integer from –128 to 127 in unit of dBm. This field is present only if the BS EIRP indicator is set in PHY Profile ID. Otherwise, the BS has the same EIRP as the superordinate BS.

BS_ID

Recommended HR-BS ID parameter for HR-MS acting as BS.

Preamble Index/Subchannel Index

For the OFDMA PHY this parameter defines the PHY specific preamble. For the OFDM PHY, the 5 LSB contain the active DL subchannel index and the 3 MSB shall be Reserved and set to ‘0b000’. For OFDMA PHY, bit 7 is used to indicate the reuse factor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index.

Scheduling Service Supported

Bitmap to indicate if BS supports a particular scheduling service. 1 indicates support, 0 indicates not support:

Bit 0: Unsolicited grant service (UGS)

Bit 1: Real-time polling service (rtPS)

Bit 2: Non-real-time polling service (nrtPS)

Bit 3: Best effort (BE) service

Bit 4: Extended real-time polling service (ertPS)

If the value of bit 0 through bit 4 is 0b00000, it indicates no information on service available.

Bits 5–7: Reserved; shall be set to zero.

6.3.2.3.98.x2 MM-BS-RSP (multimode base station response) message

After configuring HR-BS mode or expiring pre-provisioned BS configuration deadline, the MM-BS-RSP message shall be sent from multimode HR-MS to a superordinate HR-BS in response to an MM-BS-REQ message. The multimode HR-MS may send the MM-BS-RSP message in unsolicited manner with MMBS_RSP_type = 0b00 (i.e., Request by multimode HR-MS in Unsolicited manner).

Table bbb – MM-BS-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MM-BS-RSP message format () { | — | — |
| Management Message Type = xx+1 | 8 | — |
| MMBS_RSP_type | 2 | This indicates response type of MM-BS-RSP message. 0b00: Request by multimode HR-MS in Unsolicited manner 0b01: Approval of MM-BS-REQ 0b10: Rejection of MM-BS-REQ 0b11: Reserved |
| Reserved | 6 | Shall be set to zero. |
| TLV encodings for MM-BS-RSP | variable | TLV-specific |
| } |  |  |

All parameters according to action code are coded as TLV tuples, as defined in 11.xy.b.

The following parameter shall be included if MMBS_RSP_type is set to 0b10 in MM-BS-RSP message.

Operation code

0b00: Retransmit MM-BS-REQ message (HR-BS mode of multimode HR-MS is not yet completely configured.)

0b01: Retransmit MM-BS-REQ message after applying the configuration change count (Configuration change count of multimode HR-MS is out of date.)

0b10: Cancel BS configuration request

0b11: Reserved

6.3.2.3.98.x3 MM-BS-CMD (multimode base station command) message

To inform the time for starting HR-BS role, the MM-BS-CMD message shall be sent from superordinate HR-BS to the multimode HR-MS. The trigger condition indicates when a HR-MS shall initiate a mode change to BS. See Table 577.

Table ccc – MMBS-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MM-BS-CMD message format () { | — | — |
| Management Message Type = xx+2 | 8 | — |
| BS_Mode | 1 | 0b0: Start HR-BS mode after expiring action time 0b1: Start HR-BS mode upon reaching trigger conditions |
| Reserved | 7 | Shall be set to zero. |
| TLV encodings for MM-BS-CMD | variable | TLV-specific |
| } |  |  |

All parameters according to action code are coded as TLV tuples, as defined in 11.xy.c.

The following parameter shall be included if BS_Mode is set to 0b0 in MM-BS-CMD message.

Action time

This is the wait time in units of 100 ms before the HR-MS starts to perform HR-BS mode. The multimode HR-MS shall start HR-BS mode at the action time expires.

The following parameter shall be included if BS_Mode is set to 0b1 in MM-BS-CMD message.

Trigger condition

Triggers defined in table 577.

[Remedy4: Insert the following text in Section 11.4.1 of GRIDMAN AWD for 802.16Rev3.]

[Change Table 577 as indicated:]

Table 577—Trigger; Type/function/action description

| Name | Length (bit) | Value |
| --- | --- | --- |
| Type | 2 (MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: Reserved |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor BS is greater than absolute value 0x2: Metric of neighbor BS is less than absolute value 0x3: Metric of neighbor BS is greater than serving BS metric by relative value 0x4: Metric of neighbor BS is less than serving BS metric by relative value 0x5: Metric of serving BS greater than absolute value 0x6: Metric of serving BS less than absolute value 0x7: Reserved NOTE 1—0x1–0x4 not applicable for RTD trigger metric NOTE 2—When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from –32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or 0x6, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as −103.75 dBm and 0xFF is interpreted as − 40 dBm |
| Action | 3 (LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with MOB_SCN-REP after the end of each scanning interval 0x2: Respond on trigger with MOB_MSHO-REQ 0x3: MS shall start neighbor BS scanning process by sending MOB_SCN-REQ, by initiating Autonomous neighbor cell scanning (see 8.4.14.1.3) or both. 0x4–0x7: Reserved 0x4: Initiate mode change 0x5–0x7: Reserved NOTE—0x3 is not applicable when neighbor BS metrics are defined (i.e., only Function values 0x5 or 0x6 are applicable). NOTE—0x4 is only applicable for function values 0x5 and 0x6. |

[Remedy5: Insert the following text in at the end of Section 11 of GRIDMAN AWD for 802.16Rev3.]

11.xy Message encodings for HR-Networks

[Note to editor: expected after section 11.xy.3]

11.xy.a MM-BS-REQ message encodings

The encodings described in this subclause are specific to the MM-BS-REQ message (6.3.2.3.98.x1).

Table xxx – MM-BS-REQ message encodings

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| DCD Configuration Change Count | 1 | 1 | Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change, except for the frame number for the OFDM PHY and the Available DL Radio Resources. |
| UCD Configuration Change Count | 2 | 1 | Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change, except for the Available UL Radio Resources. |
| Operator ID | 3 | 3 | Identifier of the network provider |
| PHY Profile ID | 4 | 1 | For systems using OFDM or OFDMA, the bit-by-bit definition of the PHY Profile ID is shown in Table 145. |
| FA index | 5 | 1 | Frequency assignment index. This field is present only if the FA index indicator in PHY profile ID is set. Otherwise, the superordinate BS has the same FA Index or the center frequency is indicated using the TLV encoded information. |
| BS EIRP | 6 | 1 | Signed Integer from –128 to 127 in unit of dBm This field is present only if the BS EIRP indicator is set in PHY Profile ID. Otherwise, the BS has the same EIRP as the serving BS. |
| BS_ID | 7 | 6 | Same as the BSID parameter in the DL-MAP message of HR-MS acting as BS. |
| Preamble Index/Subchannel Index | 8 | 1 | For the OFDMA PHY this parameter defines the PHY specific preamble. For the OFDM PHY, the 5 LSB contain the active DL subchannel index and the 3 MSB shall be Reserved and set to ‘0b000’. For OFDMA PHY, bit 7 is used to indicate the reuse factor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index. |
| Scheduling Service Supported | 9 | 1 | Bitmap to indicate if BS supports a particular scheduling service. 1 indicates support, 0 indicates not support: Bit 0: Unsolicited grant service (UGS) Bit 1: Real-time polling service (rtPS) Bit 2: Non-real-time polling service (nrtPS) Bit 3: Best effort (BE) service Bit 4: Extended real-time polling service (ertPS) If the value of bit 0 through bit 4 is 0b00000, it indicates no information on service available. Bits 5–7: Reserved; shall be set to zero. |

11.xy.b MM-BS-RSP message encodings

The encodings described in this subclause are specific to the MM-BS-RSP message (6.3.2.3.98.x2).

Table yyy – MM-BS-RSP message encodings

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Operation code | 1 | 1 | 0x00: Retransmit MMBS-REQ message (HR-BS mode of multimode HR-MS is not yet completely configured.) 0x01: Retransmit MMBS-REQ message after applying the configuration change count (Configuration change count of multimode HR-MS is out of date.) 0x02: Cancel BS configuration request 0x03–0xFF: Reserved. |

11.xy.c MM-BS-CMD message encodings

The encodings described in this subclause are specific to the MM-BS-CMD message (6.3.2.3.98.x3).

Table zzz – MM-BS-CMD message encodings

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Action time | 1 | 1 | This is the wait time in units of 100 ms before the HR-MS starts to perform HR-BS mode. The multimode HR-MS shall start HR-BS mode at the action time expires. |
| Trigger condition | 2 | variable | Triggers defined in table 577. |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]