| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on relay link release of HR-MS in GRIDMAN AWD for 802.16.1a |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Seokjoo Shin Chosun University Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: sjshin@chosun.ac.kr woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0020,” in response to Call for Comments on GRIDMAN AWD |  |
| Abstract | This contribution is a proposal to clarify difficult text in multimode operation. |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on relay link release of HR-MS in GRIDMAN AWD for 802.16.1a

Seokjoo Shin

Chosun University

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

- Introductions

IEEE 802.16n AWD describes relay link release operation in Section 16.12.1.2.3. In this contribution, we propose a modified AAI-MM-RL-REQ message which is transmitted by HR-MS acting as relay to release relay mode. In the current message, the reason why the HR-MS acting as relay wants to release its relay mode is not defined in current documents. If a HR-MS acting as relay reports the reason of release to its superordinate HR-BS, the superordinate HR-BS may react more efficiently.

- References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0024, “GRIDMAN Amendment Working Draft for 802.16rev3,” Oct 2011.

- IEEE 802.16n-11/0025, “GRIDMAN Amendment Working Draft for 802.16.1,” Oct 2011.

- Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text into section 6.2.3.65.4 in the GRIDMAN AWD for 802.16.1a.]

6.2.3.65.4 AAI-MM-RL-REQ AAI-MMRL-REQ message

HR-MS transmits AAI-MM-RL-REQ AAI-MMRL-REQ message for the purpose as follows:

- to release its relay mode and to return its original role

- to response or reject the unsolicited AAI-MM-RL-RSP AAI-MMRL-RSP message by the HR-BS

- to indicate reason of relay mode release request

Table 763mm4 – AAI-MM-RL-REQ AAI-MMRL-REQ message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Release_Request_Code | 2 | Used to indicate the purpose of this message 0b00: multimode release 0b01: response for the unsolicited AAI-MM-RL-RSP AAIMMRL-RSP message by the HR-BS 0b10: reject for the unsolicited AAI-MM-RL-RSP AAIMMRL-RSP message by the HR-BS. This code is applicable only when UL data is pending to transmit. 0b11: reserved | Always present |
| Release_Reason_Code | 2 | Used to indicate the reason why HR-MS acting as relay wants to release its relay mode 0b00: battery is reaching to the predefined threshold level 0b01: interference level exceed the tolerable level 0b10: normal power off is initiated 0b11: reserved |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]