| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Synchronization control for HR-MS acting as relay in GRIDMAN AWD for 802.16.1a |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Seokjoo Shin Chosun University Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: sjshin@chosun.ac.kr woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0020,” in response to Call for Comments on GRIDMAN AWD |  |
| Abstract | This contribution is proposes the mechanisms for synchronization control of HR-MS acting as relay. |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Synchronization control for HR-MS acting as relay in GRIDMAN AWD for 802.16.1a

Seokjoo Shin

Chosun University

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Seokki Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

- Introductions

GRIDMAN AWD for IEEE 802.16.1a describes relay mode operation of HR-MS in Section 6.12.1.2. However, HR-MS acting as relay cannot receive A-Preambles from superordinate HR-BS after it establishes a relay link with the superordinate HR-BS (refer to 16m’s Section 16.6) [3]. Therefore, if the HR-MS acting as relay moves or is located indoor environment, it may be failed synchronization with superordinate HR-BS. In this contribution, we propose scanning procedures for synchronization between HR-MS acting as relay and its superordinate HR-BS. Since HR-MS acting as relay has mobility, preambles from the superordinate HR-BS shall be received by HR-MS acting as relay periodically in TTR mode. Two messages are defined in this purpose which are AAI-MMRS-SYN-REQ/RSP. The parameters used for A-preambles scanning time interval should be transmitted to its subordinate MSs since HR-MS acting as relay cannot communicate with its subordinate MSs during the scanning interval. AAI-MM-ADV shall be assigned to carry the scanning time interval information from HR-MS acting as relay to its subordinate MSs.

- References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0024, “GRIDMAN Amendment Working Draft for 802.16rev3,” Oct 2011.

- IEEE 802.16n-11/0025, “GRIDMAN Amendment Working Draft for 802.16.1,” Oct 2011.

- Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Insert a new subsection in Section 6.12.1.2 in the GRIDMAN AWD for 802.16.1.]

6.12.1 Multi-mode operation

6.12.1.2 Relay function for HR-MS

6.12.1.2.3 Relay link maintenance

6.12.1.2.3.1 Synchronization control for HR-MS acting as relay

Since HR-MS acting as relay has mobility, preambles from the superordinate HR-BS shall be received by HR-MS acting as relay periodically in TTR mode. A superordinate HR-BS may allocate time intervals to a relay mode HR-MS in order for an HR-MS acting as relay to receive preambles from the superordinate HR-BS in the purpose of synchronization between a superordinate HR-BS and an HR-MS acting as relay. Such a time interval during which the HR-MS acting as relay scans preambles from the superordinate HR-BS while not available to serve its MSs is referred to as a A-preamble scanning interval. Initial parameter values for synchronization time interval shall be given by the superordinate HR-BS through AAI-MMRS-SYN-RSP message when an HR-MS acting as relay performs network entry to the superordinate HR-BS. During the relay mode operation, a superordinate HR-BS may transmit unsolicited AAI-MMRS-SYN-RSP message to an HR-MS acting as relay or shall respond upon receiving AAI-MMRS-SYN-REQ message transmitted by HR-MS acting as relay to chase the timing offset. Allocated time interval information shall be transmitted to the subordinate MSs within the coverage of the HR-MS acting as relay through AAI-MM-ADV message. During the scanning interval HR-MS acting as relay may communicate with its serving HR-BS but not with its subordinate MSs.

6.12.1.2.34 Relay link release

An HR-MS acting as RS may end its relay service and remove the relay link from the HR BS.

……

[Remedy2: Add the following rows in Table 683, section 6.2.3 in the GRIDMAN AWD for 802.16.1.]

| No. | Functional Areas | Message Names | Message Description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| TBD | Multimode | AAI-MMRS-SYN-REQ | Multimode Relay Synchronization request |  | Unicast |
| TBD | Multimode | AAI-MMRS-SYN-RSP | Multimode Relay Synchronization response |  | Unicast |

[Remedy3: Add the following text into section 6.2.3 in the GRIDMAN AWD for 802.16.1.]

6.2.3.65 MAC control messages for HR-Networks

6.2.3.65.x1 AAI-MMRS-SYN-REQ message

To control the parameters for synchronization between HR-MS acting as relay and superordinate HR-BS, the HR-MS acting as relay may transmit AAI-MMRS-SYN-REQ message to the superordinate HR-BS. AAI-MMRS-SYN-REQ message asks the superordinate HR-BS to increase/decrease the synchronization time interval and duration.

Table xxx - AAI-MMRS-SYN-REQ message format

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration control | 1 | 0b0: increase the time duration 0b1: decrease the time duration |  |
| Time interval control | 1 | 0b0: increase the time interval 0b1: decrease the time interval |  |

6.2.3.65.x2 AAI-MMRS-SYN-RSP message

The S-ABS shall transmit AAI-MMRS-SYN-RSP to set parameters for maintaining synchronization between superordinate HR-BS and HR-MS acting as relay when a relay mode HR-MS performs network entry to superordinate HR-BS or receives AAI-MMRS-SYN-REQ message from an HR-MS acting as relay. Unsolicited AAI-MMRS-SYN-RSP message may be transmitted by superordinate HR-BS.

Table xxx - AAI-MMRS-SYN-RSP message format

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration | 8 | Duration (in units of AAI subframe) of the requested scanning period. |  |
| If(scan duration >0){ |  |  |  |
| Start superframe index | 6 | Represents the 6 least significant bits of the absolute superframe index in which the first scanning interval starts. |  |
| Start frame index | 2 | Represents start frame number in the Start super frame number. 0b00: 1st frame in the super frame 0b01: 2nd frame in the super frame 0b10: 3rd frame in the super frame 0b11: 4th frame in the super frame |  |
| Interleaving interval | 8 | The period of relay mode HR-MS’s active mode (in units of AAI subframes) that is interleaved between scan durations. |  |
| } //End if (Scan duration >0) |  |  |  |

[Remedy4: Add the following text into section 6.2.3in the GRIDMAN AWD for 802.16.1.]

6.2.3.65 MAC control messages for HR-Networks

6.2.3.65.1 AAI-MM-ADV message

Infrastructure stations and HR-MS acting as HR-BS or HR-RS may transmit AAI-MM-ADV message to support multimode operation in the case as follows:

- When the backhaul link is down or up

- During maintaining relay link due to unavailable backhaul link, PHY/MAC layer parameters need be reconfigured such as

  - Power down

  - Power reduction

  - FA change

- Multimode service establish/release/change to inform subordinate stations to perform handover

- Scanning operation for HR-MS acting as relay to maintain synchronization with the serving HR-BS

Table 763mm1- Parameters for AAI-MM-ADV message

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Type | 34 | Used to indicate the purpose of this message 0b0000000: Reconfiguration of HR-BS/RS including multimode BS/RS 0b0010001: Restart of HR-BS/RS including multimode BS/RS 0b0100010: Power down (including FA down) of HR-BS/RS including multimode BS/RS 0b0110011: Power reduction of HR-BS/RS including multimode BS/RS 0b1000100: Backhaul link down of HR-BS 0b1010101: Backhaul link up of HR-BS 0b1100110: FA change of HR-BS/RS including multimode BS/RS 0b1110111: Multimode service end of HR-MS 0b1000: Scanning operation for HR-MS acting as relay to maintain synchronization with the serving HR-BS 0b1001 – 0b1111: reserved |  |
| …. |  |  |  |
| } else if (Action Type == 0b1000) { |  |  |  |
| Scan duration | 8 | Duration (in units of AAI subframe) of the requested scanning period. |  |
| Interleaving interval | 8 | The period of relay mode HR-MS’s active mode (in units of AAI subframes) that is interleaved between scan durations. |  |
| Start superframe index | 6 | Represents the 6 least significant bits of the absolute superframe index in which the first scanning interval starts. |  |
| Start frame index | 2 | Represents start frame number in the Start super frame number. 0b00: 1st frame in the super frame 0b01: 2nd frame in the super frame 0b10: 3rd frame in the super frame 0b11: 4th frame in the super frame |  |
| } |  |  |  |
| … |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]