| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Security Key Derivation for IEEE 802.16.1a |  |
| Date Submitted | 2011-10-3111-07 |  |
| Source(s) | Joseph Teo Chee Ming, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Security Key Derivation for IEEE 802.16.1a

Joseph Chee Ming Teo, Yeow Wai Leong, Jaya Shankar, Hoang Anh Tuan, Zheng Shoukang

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the key shared within the group should be distributed securely and efficiently.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. Currently in the IEEE 802.16n AWD, the multicast keys are derived from a key hierarchy as summarized in Figure 934. We propose that derivation of the MCMAC-MTEK Prekey shall include the multicast group identifier for disambiguation since a HR-MS can be a member of a few multicast groups. We also propose additional text to supplement the key derivation procedure.

# 2. Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Adopt the following text in the 802.16.1a AWD Document (C802.16x-xx/xxxx)]

[Replace figure 934 as indicated:]

Figure 934 – MCMAC Key and MTEK Key derivation from MAK

[Amend Table 1233 as indicated:]

Table 1233 – The MAK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MAK | 160 | Shared by HR-MSs in a multicast group |
| MAK Lifetime | 32 | MAK Lifetime |
| MAKID | 64 | Identifies the authorization key. |
| MAK_COUNT | 16 | A value used to derive the MCMAC key and MTEK |
| MulticastGrpID | 16 | Identifies the Multicast Group |
| MCNonce | 128 | A random number used to derive the MCMAC-MTEK Prekey |
| MCMAC_KEY_D | 128 | The key which is used for signing DL MAC control messages. |
| MCMAC_PN_D | 24 | Used to avoid DL replay attack on the control connection before this expires, reauthorization is needed. The initial value of MCMAC_PN_D is zero and the value of MCMAC_PN_D is reset to zero whenever MAK_COUNT is increased. |
| Next available counter_MTEK | 16 | The counter value to be used in next MTEK derivation, after derivation this is increased by 1. |

[Insert the following before Section 6.12.10.2.1 as indicated:]

### 6.12.10.2.x Multicast Key Derivation

The multicast key hierarchy defines what keys are present in the system for secure multicast operations and how the keys are generated.

6.12.10.2.x.1 MAK

The 160bits Multicast Authentication Key (MAK) is the pre-established shared key that is shared among authorized devices of a multicast service. The generation and transport of the MAK is outside the scope of the IEEE 802.16 standard.

6.12.10.2.x.2 MCMAC-MTEK Prekey

The MAK, MulticastGrpID and MCNonce is used to derive the MCMAC-MTEK Prekey as follows:

MCMAC-MTEK Prekey = Dot16KDF(MAK, MulticastGrpID| MCNonce|| “MCMAC-MTEK prekey”, 160)

Where:

- MulticastGrpID is the identifier used for the multicast service.

- MCNonce is a randomly generated number by HR-BS or network entity to ensure different sets of MCMAC and MTEK keys are derived after rekeying.

The MCMAC-MTEK Prekey is used to derive the :

- Multicast Cipher-based Message Authentication Code (MCMAC) key

- Multicast Traffic Encryption (MTEK) Key

6.12.10.2.x.3 MCMAC Key Derivation

The 128bits MCMAC key is derived from MCMAC-MTEK Prekey and used for message authentication for the multicast messages sent during secure multicast operation.

MCMAC key is derived as follows:

MCMAC_KEY_D = Dot16KDF(MCMAC-MTEK Prekey, “MCMAC_KEYS”, 128)

6.12.10.2.x.3 MTEK Derivation

The 128bits MTEK is the multicast transport encryption key used to encrypt data for secure multicast operations.

MTEK is derived as follows:

MTEKi = Dot16KDF(MCMAC-MTEK Prekey, MSAID|COUNTER_MTEK=i|“MTEK”, 128)

[-------------------------------------------------End of Text Proposal----------------------------------------------------]