| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Key Request Procedure for IEEE 802.16.1a |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Wai-Leong Yeow, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: wlyeow@i2r.a-star.edu.sg cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Key Request Procedure for IEEE 802.16.1a

Wai-Leong Yeow, Joseph Chee Ming Teo,

Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the master key shared within the group should be distributed securely.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. Although a Multicast and Broadcast Rekeying Algorithm (MBRA) is already supported in IEEE 802.16-2009, however, it is based on PKMv2 and the rekeying is performed through multicast without any ARQ support. The current IEEE 802.16n AWD supports a key hierarchy for security in multicast but does not describe how the root key is shared between stations of the multicast group.

# In this contribution, we propose a procedure for an HR-MS to initiate a request to the HR-BS and obtain the Multicast Authentication key securely.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Change Table 71 as indicated:]

Table 71 – PKMv3 message types

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| 1 | PKMv3 Reauth-Request | AAI-PKM-REQ |
| 2 | PKMv3 EAP-Transfer | AAI-PKM-REQ/AAI-PKM-RSP |
| 3 | PKMv3 Key_Agreement-MSG#1 | AAI-PKM-RSP |
| 4 | PKMv3 Key_Agreement-MSG#2 | AAI-PKM-REQ |
| 5 | PKMv3 Key_Agreement-MSG#3 | AAI-PKM-RSP |
| 6 | PKMv3 TEK-Request | AAI-PKM-REQ |
| 7 | PKMv3 TEK-Reply | AAI-PKM-RSP |
| 8 | PKMv3 TEK-Invalid | AAI-PKM-REQ/AAI-PKM-RSP |
| x | PKMv3 MAK-Request | AAI-PKM-REQ |
| x+1 | PKMv3 MAK-Response | AAI-PKM-RSP |
| 9y—16 | Reserved | — |

[Add the following text into Section 6.2.3.43 Security in the 802.16n AWD]

### 6.2.3.43.x PKMv3 MAK-Request message

The HR-MS transmits the PKMv3 MAK-Request message as a first step for an HR-MS initiated MAK request. The MAK-Request message identifies the multicast group the MAK belongs to, the serving HR-BSID and STID of the requesting HR-MS.

The message shall include a nonce NHR-MS generated by the requesting HR-MS and a timestamp THR-MS for freshness. The CMAC Digest attribute (i.e., CMAC_PN_U and CMAC value) shall be transmitted for CMAC verification, which is computed from CMAC_KEY _U.

Code: x

The message attributes are shown in Table aaa.

Table aaa – PKMv3 MAK-Request message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | Multicast Group ID |
| THR-MS | Timestamp generated by HR-MS |
| NHR-MS | Nonce generated by HR-MS |
| HR-BSID | HR-BSID |
| STID | STID |
| CMACHR-MS | Message digest calculated by HR-MS |

### 6.2.3.43.x+1 PKMv3 MAK-Response message

The HR-BS transmits the PKMv3 MAK-Response message to an HR-MS for disseminating the multicast key of a multicast group. The message is transmitted in response as a second step to the HR-MS initiated MAK request event.

The message shall include a nonce NHR-MS generated by the requesting HR-MS, a nonce NHR-BS generated by the HR-BS and a timestamp THR-BS for freshness. The multicast group ID, STID, HR-BSID and the MAK shall be included in the message. The MAK is generated by the HR-BS

The messaged shall be encrypted with the current TEK for MAK confidentiality, and contain the CMAC Digest attribute (i.e., CMAC_PN_D and CMAC value) for CMAC verification, which is computed from CMAC_KEY _D.

Code: x+1

The message attributes are shown in Table bbb.

Table bbb – PKMv3 MAK-Response message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | Multicast Group ID |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Nonce generated by HR-BS |
| STID | STID |
| HR-BSID | HR-BSID |
| NHR-MS | Nonce generated by HR-MS |
| MAK | Multicast Authentication Key |
| MAK Lifetime | Remaining Multicast Authentication Key Lifetime |
| CMACHR-BS' | Message digest calculated using CMAC key by HR-BS |

[Add the following text after Section 6.12.10.2 in the 802.16n AWD]

### 6.12.10.z Multicast Authentication Key (MAK) Request

The Multicast Authentication Key (MAK) Key Request Procedure takes place whenever an HR-MS wishes to obtain an up-to-date Multicast Authentication Key (MAK) of a secured multicast group from the serving HR-BS. Figure X1 shows the flow diagram.

The MAK Request Procedure is described by the following steps:

Step 1: Mobile station HR-MS first generates timestamp THR-MS and nonce NHR-MS. Next, HR-MS transmits a MAK-Request message using with the nonce, timestamp, multicast group ID, the serving HR-BSID and its own STID to the HR-BS. The message is protected by the CMCACHR-MS digest tuple using the current CMAC_KEY_U.

Step 2: HR-BS first verifies the received timestamps and nonce for freshness. If the verifications are incorrect, the request is silently dropped. Otherwise, HR-BS generates nonce NHR-BS, timestamp THR-BS and Multicast Authentication key MAK (if it has not been generated yet). HR-BS then computes MCMAC and MTEK key (if MAK is freshly generated), and transmits an encrypted MAK-Response message to the HR-MS containing the MCMAC, the MAK and its remaining lifetime, nonce NHR-BS, timestamp THR-BS and the received nonce NHR-MS, its own BSID and STID of the HR-MS. This message shall be encrypted using the current TEK for confidentiality and protected by the CMACHR -MS' digest tuple using the current CMAC_KEY_D.

Step 3: HR-MS first verifies the received timestamp and nonces for freshness. The MAK-Response message shall be verified using the CMACHR -MS' digest tuple. If the verifications fail, the MAK-Response message is silently dropped. Otherwise, the message is decrypted with the current TEK. and MAK and its lifetime can be obtained. Next, HR-MS computes MCMAC and MTEK keys from the received MAK (that has not yet expire) for the multicast operation.

Figure X1 - Flow Diagram for MAK-Request

# 6.12.10.z.a Message Type

Table aaa – Message Type

| Code | Message Type | MAC control message name |
| --- | --- | --- |
|  | MAK-Request | AAI-PKM-REQ |
|  | MAK-Response | AAI-PKM-RSP |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]