| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Key Update Procedure for IEEE 802.16.1a |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Wai-Leong Yeow, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: wlyeow@i2r.a-star.edu.sg cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Key Update Procedure for IEEE 802.16.1a

Wai-Leong Yeow, Joseph Chee Ming Teo,

Jaya Shankar, Hoang Anh Tuan, Haiguang Wang, Zheng Shoukang

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the key shared within the group should be distributed securely and efficiently. The multicast communications should be able to take place with or without Base Station (HR-BS) in order to provide high reliability.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. Although multicast membership management is handled at the upper layers, the security of a multicast operation is managed within IEEE 802.16n. Currently in the IEEE 802.16n AWD, secured multicast operations are based on a pre-shared master key but there is no procedure to securely disseminate the Multicast Authentication key (MAK) of a multicast group.

# In this contribution, we propose a protocol for an HR-BS to securely and efficiently disseminate the most up-to-date MAK of a group of HR-MS in the same multicast group without solicitation over the broadcast connection. This could be due to a key update/refresh or a change in group membership. Note that management of multicast membership is handled at the upper layers, but our contribution describes the procedure for secure dissemination of MAK when the need from the multicast management at the upper layers arises.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Change Table 26 as indicated:]

Table 26—MAC Control messages (continued)

| No. | Functional areas | Message Names | Message Description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| 23 | Security | AAI-PKM-REQ | Privacy Key Management Request | Before AK is derived at network entry: NULL after AK is derived at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived at network entry and the other message is enclosed: CMAC | Unicast |
| 24 | Security | AAI-PKM-RSP | Privacy Key Management Response | Before AK is derived at network entry: NULL at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived after AK is derived at network entry and the other message is enclosed: CMAC | Unicast or Broad-cast |

[Change Table 71 as indicated:]

Table 71 – PKMv3 message types

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| 1 | PKMv3 Reauth-Request | AAI-PKM-REQ |
| 2 | PKMv3 EAP-Transfer | AAI-PKM-REQ/AAI-PKM-RSP |
| 3 | PKMv3 Key_Agreement-MSG#1 | AAI-PKM-RSP |
| 4 | PKMv3 Key_Agreement-MSG#2 | AAI-PKM-REQ |
| 5 | PKMv3 Key_Agreement-MSG#3 | AAI-PKM-RSP |
| 6 | PKMv3 TEK-Request | AAI-PKM-REQ |
| 7 | PKMv3 TEK-Reply | AAI-PKM-RSP |
| 8 | PKMv3 TEK-Invalid | AAI-PKM-REQ/AAI-PKM-RSP |
| x | PKMv3 MAK-Update | AAI-PKM-RSP |
| x+1 | PKMv3 MAK-Group_Update | AAI-PKM-RSP |
| 9y—16 | Reserved | — |

[Add the following text into Section 6.2.3.43 in the 802.16n AWD]

### 6.2.3.43.x PKMv3 MAK-Update message

The HR-BS transmits unsolicited PKMv3 MAK-Update message to disseminate the most up-to-date MAK of a multicast group to an HR-MS, a member of the multicast group.

The message shall include a nonce NHR-BS and timestamp THR-BS generated by the HR-BS for freshness. The message shall include HR-BSID and STID for further verification, the most up-to-date MAK, and the remaining lifetime.

The messaged shall be encrypted with the current TEK for MAK confidentiality, and contain the CMAC Digest attribute (i.e., CMAC_PN_D and CMAC value) for CMAC verification, which is computed from CMAC_KEY _D.

Code: x

The message attributes are shown in Table aaa.

Table aaa – PKMv3 MAK-Update message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | Multicast Group ID |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Nonce generated by HR-BS |
| HR-BSID | HR-BSID |
| STID | STID of the target HR-MS |
| MAK | Multicast Authentication Key |
| MAK Lifetime | Remaining Multicast Authentication Key Lifetime |
| CMACHR-MS | Message digest calculated by HR-BS |

### 6.2.3.43.x+1 PKMv3 MAK-Group_Update message

The HR-BS transmits unsolicited PKMv3 MAK-Group_Update message to disseminate the most up-to-date MAK of all HR-MS of the multicast group.

The message shall include a nonce NHR-BS and timestamp THR-BS generated by the HR-BS for freshness. The message shall include HR-BSID for further verification, the most up-to-date MAK, and the remaining lifetime.

The messaged shall be encrypted with the current group MTEK for MAK confidentiality, and contain the MCMAC Digest attribute (i.e., MCMAC_PN and MCMAC value) for MCMAC verification, which is computed from MCMAC_KEY.

Code: x+1

The message attributes are shown in Table aaa.

Table aaa – PKMv3 MAK-Group_Update message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | Multicast Group ID |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Nonce generated by HR-BS |
| HR-BSID | HR-BSID |
| MAK | Multicast Authentication Key |
| MAK Lifetime | Remaining Multicast Authentication Key Lifetime |
| MCMACHR-BS | Message digest calculated by HR-BS |

[Add the following text after Section 6.12.10.2 in the 802.16n AWD]

### 6.12.10.y Multicast Authentication Key (MAK) Update

The Multicast Authentication Key (MAK) Update Procedure takes place whenever a HR-BS decides to refresh the MAK and disseminate it via unicast to an HR-MS, a member of the multicast group. This may be due to an expiration of the current MAK, or a change in the group membership, e.g. a new HR-MS joins the multicast group. How the multicast group membership is managed is out of this specification. Figure X1 shows the flow diagram for the MAK Update procedure.

The MAK Update Procedure is as follows:

Step 1: Based on the new group master key MAK, HR-BS dervies the new MCMAC and MTEK keys through the key management procedure shown in Table 921. HR-BS generates nonce NHR-BS, and timestamp THR-BS for freshness, and transmits an encrypted MAK-Update message to the HR-MS containing the multicast group ID, the MAK and its remaining lifetime, nonce NHR-BS and timestamp THR-MS, its own BSID and STID of the HR-MS. This message shall be encrypted using the current TEK for confidentiality and protected by the CMACHR -MS digest tuple using the current CMAC_KEY_D.

Step 2: HR-MS first decrypts the message using the current TEK, and verifies the CMACHR-MS, and the received timestamp and nonce for freshness. If the verifications are incorrect, HR-MS silently drops the message. Otherwise, HR-MS updates its MAK and derives the new MCMAC and MTEK keys.

Figure X1 - Flow Diagram for MAK-Update

6.12.10.y.a Message Type

Table bbb – Message Type

| Code | Message Type | MAC control message name |
| --- | --- | --- |
|  | MAK-Update | AAI-PKM-RSP |

### 6.12.10.z Multicast Authentication Key (MAK) Group Update

The Multicast Authentication Key (MAK) Group Update Procedure takes place whenever a HR-BS decides to refresh the MAK and disseminate it via broadcast to all member of a multicast group. This may be due to an expiration of the current MAK, or a change in the group membership, e.g. a new HR-MS joins the multicast group. How the multicast group membership is managed is out of this specification. Figure X1 shows the flow diagram for the MAK Group Update procedure.

The MAK Group Update Procedure is as follows:

Step 1: Based on the new group master key MAK, HR-BS dervies the new MCMAC and MTEK keys through the key management procedure shown in Table 921. HR-BS generates nonce NHR-BS, and timestamp THR-BS for freshness, and transmits an encrypted MAK-Group_Update message to all HR-MS of that multicast group, containing the multicast group ID, the MAK and its remaining lifetime, nonce NHR-BS and timestamp THR-BS, its own BSID. This message shall be encrypted using the current MTEK for confidentiality and protected by the MCMACHR -BS digest tuple using the current MCMAC_KEY.

Step 2: Every HR-MS first decrypts the message using the current TEK, and verifies the MCMACHR-MS, and the received timestamp and nonce for freshness. If the verifications are incorrect, HR-MS silently drops the message. Otherwise, HR-MS updates its MAK and derives the new MCMAC’ and MTEK’ keys.

Figure X1 - Flow Diagram for MAK-Group_Update

6.12.10.z.a Message Type

Table bbb – Message Type

| Code | Message Type | MAC control message name |
| --- | --- | --- |
|  | MAK-Group_Update | AAI-PKM-RSP |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]