| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Join and Leave Security Procedure in IEEE 802.16.1a |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Wai-Leong Yeow, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: wlyeow@i2r.a-star.edu.sg cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Join and Leave Security Procedure in IEEE 802.16.1a

Wai-Leong Yeow, Joseph Chee Ming Teo,

Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the master key shared within the group should be distributed securely.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. The current IEEE 802.16n AWD does not support Multicast Authentication key rekeying. If members were to leave the multicast group, it is still possible to decrypt and access all previous multicast traffic after the leave event, i.e. no forward secrecy. Conversely, if new members were to join the multicast group, it is possible to access all previous multicast traffic prior to the join event, i.e. no backward secrecy.

# In this contribution, we propose that the Multicast Authentication key shall be rekeyed when members join or leave a multicast group in order to assure both backward and forward secrecy.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Add the following text after Section 6.12.10.2 in the 802.16n AWD]

### 6.12.10.2.x Multicast Join Procedure

In order to prevent newly joined stations from decrypting and accessing multicast traffic prior to the join event, multicast keys shall be changed whenever new members join a multicast group. Figure X1 shows the flow diagram of the Join Event.

The Join Procedure includes the following steps:

Step 1: The HR-MS requests to obtain a master key from the HR-BS for a multicast group as described in Section ???.

Step 2: The HR-BS verifies HR-MS is a member of the multicast group. Since multicast membership is managed at the upper layer, how to verify this is out of this specification. If it is a new member, HR-BS generates a new MAK and disseminates the new MAK through the MAK Update procedure as described in Section ??? to to all existing members of the multicast group efficiently and securely.

Step 3: HR-BS replies the new multicast member with the new MAK’ in the MAK-Response message.

Step 4: Both the existing multicast group members and new multicast group member HR-MS verifies the respective received message, obtains the new Multicast Authentication key and derives the MCMAC and MTEK keys.

Figure X1 - Flow Diagram for new multicast member

### 6.12.10.2.y Multicast Leave Procedure

In order to prevent stations from decrypting and accessing future multicast traffic after leaving the multicast group, multicast keys shall be changed whenever members leave a multicast group. Figure X2 shows the flow diagram of the Leave Event.

The Leave Event Procedure includes the following steps:

Step 1: After sending the AAI-DSD-RSP for leaving a multicast group, HR-BS computes a new MAK and unsolicitedly sends the MAK-Update message with the new MAK (as described in Section ???) to every remaining HR-MS in the multicast group.

Step 2: Each remaining multicast group member HR-MS verifies their respective received messages and obtains the new multicast keys to continue secure multicast communications.

Figure X2 - Flow Diagram for leaving procedure

[-------------------------------------------------End of Text Proposal----------------------------------------------------]