| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for PHY-layer Support for BS-controlled HR-MS Forwarding to Network (Rev3-based) |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 Eldad Zeira InterDigital | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0024 |  |
| Abstract | This contribution proposed modifications for text for PHY-layer support for BS-controlled HR-MS forwarding to network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for PHY-layer Support for BS-controlled HR-MS

Forwarding to Network (Rev3-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Eldad Zeira (InterDigital)

Introduction

This contribution proposed modifications for text for PHY-layer support for BS-controlled HR-MS forwarding to network.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0024.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Move Section 16.3.2.1.2.1 under Section 16.3.2.2 and make the following modifications}

16.3.2.2 Physical layer

16.3.2.2.1   Frame structure and resource allocation

See  16.2.2.2.1.

16.3.2.1.2.1 16.3.2.2.1 Frame Structure

The HR-BS/HR-RS shall schedule the selected HR-MSs transmitting preamble and NCI at the end of the UL subframe. A zone in uplink with duration of three OFDMA symbols shall be allocated for discovery. The preamble occupies one OFDMA symbol duration and the FCH and NCI. The format of NCI is defined in section in 6.3.2.3.98.13.

Figure 1 shows the frame structure when HR-MS is scheduled to transmit preamble and NCI. The format of NCI is defined in 6.3.2.3.98.13.

Figure 1----Frame structure for HR-MS transmitting preamble and NCI for FTN discovery.

HR-MSs that are out of coverage of HR-BS/HR-RS scanning for DL preambles for possible network entry shall be able to differentiate between preambles transmitted by normal infrastructure stations (HR-BS/HR-RS) and those transmitted by a coverage-extending HR-MS.

After receiving the preamble and NCI, if HR-MS intends to start network entry process, it chooses an initial ranging code and shall transmit the ranging code on the specified ranging channel. Back-off is unnecessary for FTN initial ranging.

A subset of preambles shall be reserved for FTN discovery. The details are TBD.

A forwarding HR-MS and an inside-of-coverage forwarded HR-MS fully follow the frame configurations set by the serving HR-BS/RS, as described in 8.4.4. A forwarded HR-MS that is out-of-coverage of the serving HR-BS/RS obtains essential frame configurations through the preamble and control channels from its corresponding forwarding HR-MS. These forwarding control channels include M-FCH, NCI, Forwarding MAP (F-MAP).

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.