| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text and Control Messages for Relay Function of HR-BS |  |
| Date Submitted | 2011-11-05 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0025 |  |
| Abstract | This contribution proposes modifications to the existing text and MAC control messages in 802.16.1-based Gridmad AWD to support relay function of HR-BS. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text and Control Messages for Relay Function of HR-BS

(16.1-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes modifications to the existing text and MAC control messages in 802.16.1-based Gridmad AWD to support relay function of HR-BS.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0025.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 6.12.1.1.1 and 6.12.1.1.2.2.1 as indicated}

6.12.1.1.1 Relay link establishment

The HR-BS having no connection to backhaul transmits AAI-MM-ADV message with action type = 0b100 described in 6.2.3.70 including expected time of backhaul link available. Based on the expected time, HR-MS handovers to neighbor infrastructure station or staying in the HR-BS until restarting service with available backhaul link.

….

For the case of affected HR-BS establishing a TTR relay link with a serving HR-BS, the following actions shall be carried out:

- In the AAI-MMRS-REQ message, the affected HR-BS reports its required switching gaps BSTTG and BSRTG to the serving HR-BS. Here, BSTTG is the minimum transmit-to-receive turnaround gap while BSRTG is the minimum receive-to-transmit turnaround gap required at the affected HR-BS. BSTTG and BSRTG are mapped into ST-TTG and ST-RTG fields in AAI-MMRS-REQ message, respectively. Based on the values of BSTTG and BSRTG, the two HR-BSs agree on the timing advance Ta of the frame boundary of the affected HR-BS, relative to that of the serving HR-BS. The affected HR-BS can propose a value for Ta in the AAI-MMRS-REQ message, and the serving HR-BS can reply with a confirmed Ta value in the AAI-MMRS-RSP message. With the values of BSTTG, BSRTG, and Ta, the two HR-BSs calculate shared values for the switching time R-TTI and R-RTI, based on the following equations:

R-TTI = 0 if RTD/2 + Ta > ≥ BSTTG and R-TTI = Ts if RTD/2 +Ta < BSTTG

and

R-RTI = 0 if Ta – RTD/2 > ≥ BSRTG and R-RTI = Ts if Ta – RTD/2 < BSRTG,

where RTD is the round trip delay between the affected HR-BS and the serving HR-BS and Ts is the OFDMA symbol duration.

- Also included in the AAI-MMRS-REQ message sent by affected HR-BS is the proposed dual-mode switching pattern (Tbs, Trs), as described in 6.12.1.1.2.2. This pattern shall be confirmed in the corresponding AAI-MMRS-RSP message sent by the serving HR-BS.

….

6.12.1.1.2.2.1 When R-TTI = 0

When R-TTI = 0, the affected HR-BS shall keep its original PHY-layer configuration, including IDCell, frame configuration, and FFR pattern. In addition, the affected HR-BS shall set the AAI_Relay_zone_AMS_allocation_indicator field in AAI-SCD and AAI-ARS-CONFIG-CMD messages to 0b0. The operation of the affected/dual-role HR-BS can be described as follows.

In BS Operation Mode:

- The affected/dual-role HR-BS shall only communicate with its subordinate MS/AMS/HR-MS/HR-RS stations and shall not be available to receive from or transmit to its serving HR-BS.

- The manner in which the affected/dual-role HR-BS control and communicate with its subordinate AMS/HR-MSs/HR-RSs shall be the same as that of a normal HR-BS. The serving HR-BS is not expected to know the specific configuration of the dual-role HR-BS during BS Operation. When the affected/dual-role HR-BS transmits to or receives from its subordinate MS/AMS/HR-MS/HR-RS during BS Operation, it does so independently to the serving HR-BS.

- The affected/dual-role HR-BS transmits AAI-ARS-CONFIG-CMD control messages regarding its role-switching behaviors toward its subordinate HR-RSs. Essentially, these role-switching messages to tell the subordinate HR-RSs when the HR-BS will switch to RS Operation and what are the specific behaviors of the HR-BS during RS Operation.

In RS Operation Mode:

- The affected/dual-role HR-BS shall communicate with the serving HR-BS and with the subordinate MS/AMS/HR-MS. It may or may not communicate with its subordinate HR-RS during this mode of operation. The frame structure of the affected HR-BS is divided into DL Access zone, DL Relay zone, UL Access zone, and UL Relay zone. Note that as R-TTI = 0, no time gap need to be inserted into the last OFDM symbol of the last subframe in the DL Access zone.

- As the affected HR-BS still transmits the same SA-Preamble, the subordinate MS/AMS/HR-MS are oblivious to the mode change of the affected HR-BS. The affected HR-BS continues to transmit to its subordinate MS/AMS/HR-MS in the DL Access zone, and receive from its subordinate MS/AMS/HR-MS in the UL Access zone.

- The affected/dual-role HR-BS receives from and transmits to its serving HR-BS during the DL Relay zone and UL Relay zone, respectively. The PHY-layer configuration for DL/UL Relay zones shall be sent by the serving HR-BS toward the affected HR-BS in the AAI-ARS-CONFIG-CMD message.

- The affected/dual-role HR-BS can communicate with its subordinate HR-RSs in the following ways:

  - The affected/dual-role HR-BS can instruct its subordinate HR-RSs to transmit UL data during the DL Relay zone, i.e., when the affected/dual-role HR-BS also receives from the serving HR-BS. While doing so, the affected/dual-role HR-BS shall instruct the transmitting HR-RSs to use the same PHY-layer configuration as used in the DL Relay zone of the serving HR-BS.

  - The affected/dual-role HR-BS can instruct its subordinate HR-RSs to receive DL messages during the UL Relay zone, i.e., when the affected/dual-role HR-BS also transmits to the serving HR-BS. While doing so, the HR-BS shall instruct the transmitting HR-RSs to use the same PHY-layer configuration as used in the UL Relay zone of the serving HR-BS. Furthermore, if an R-RTI = Ts is inserted in the first OFDMA symbol of the first subframe of the UL Relay zone, the dual-role HR-BS shall let its subordinate HR-RSs to be aware of this insertion.

6.12.1.1.2.2.2 When R-TTI = Ts

When R-TTI = Ts, the affected HR-BS shall change its IDCell, i.e., it shall pick one of the SA-Preamble sequences (and possibly new preamble carrier index) that are allocated for TTR ARS. The operation of the affected/dual-role HR-BS can be described as follows.

In BS Operation Mode:

- The affected/dual-role HR-BS shall only communicate with its subordinate AMS/HR-MS/HR-RS stations and shall not be available to receive from or transmit to its serving HR-BS.

- The affected/dual-role HR-BS shall behave like a normal HR-RS for its subordinate AMS/HR-MS. The subordinate AMS/HR-MS detect the SA-preamble transmitted by the affected/dual-role HR-BS and classify the HR-BS as a TTR HR-RS. In response, the affected HR-BS shall only transmit DL data toward its subordinate AMS/HR-MS in the DL Access zone and receive UL data from its subordinate AMS/HR-MS in the UL Access zone. Furthermore, as R-TTI = Ts, the affected HR-BS shall not transmit on the last OFDM symbol of the last subframe in the DL Access zone. The information regarding R-TTI = Ts  shall be transmitted in the SFH SP2 toward subordinate AMS/HR-MS.

- The affected HR-BS shall behave like a normal HR-BS for its subordinate HR-RS. That means the affected HR-BS shall transmit to its subordinate HR-RS in the DL Relay zone, and receive from its subordinate HR-RS in the UL Relay zone.

In RS Operation Mode:

- The affected HR-BS shall communicate with its serving HR-BS. It may or may not communicate with its subordinate HR-RS in the mode of operation, and the specifications are as described in 6.12.1.1.2.2.1.

- The affected/dual-role HR-BS shall behave like a normal HR-RS for its subordinate AMS/HR-MS. The subordinate AMS/HR-MS detect the SA-preamble transmitted by the affected/dual-role HR-BS and classify the HR-BS as a TTR HR-RS. In response, the affected HR-BS shall only transmit DL data toward its subordinate AMS/HR-MS in the DL Access zone and receive UL data from its subordinate AMS/HR-MS in the UL Access zone. Furthermore, as R-TTI = Ts, the affected HR-BS shall not transmit on the last OFDM symbol of the last subframe in the DL Access zone. The information regarding R-TTI = Ts  shall be transmitted in the SFH SP2 toward subordinate AMS/HR-MS.

{Remedy 2: Modify Section 6.2.3.57 as indicated}

6.2.3.57 AAI-ARS-CONFIG-CMD message format

An ABS shall use AAI-ARS-CONFIG-CMD message to configure the TTR mode ARS PHY layer operational parameters.

An HR-BS shall also use AAI-ARS-CONFIG-CMD message to configure PHY layer operational parameters the multimode HR-BS/HR-MS acting as relay. HR-RS PHY layer operational parameters.

[Change the table 757 in 6.2.3.57 as indicated:]

Table 757—AAI-ARS-CONFIG-CMD message field description

| Field | Size (bits) | Value/Description |  | Conditions |
| --- | --- | --- | --- | --- |
| If(subordinate RS (including HR-MS acting as RS) is TTR relay mode in HR-Network) { |  |  |  | // TTR mode |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | 0b0: The ABS does not allocate resources to the AMS in the AAI DL Relay zone; 0b1: The ABS may allocate resources to the AMS in the AAI DL Relay zone |  | Always present |
| MIMO Midamble indication in AAI DL Relay zone | 1 | 0b0: MIMO midamble is not transmitted in AAI DL Relay zone 0b1: MIMO midamble is transmitted in AAI DL Relay zone If AAI_Relay_zone_AMS_allocation_indicator == 0b0, this field is set to 0b1. |  | Always present |
| Superframe Number Action | 4 | LSBs of the superframe number when ARS start ARS operation and apply the PHY operational parameters. |  | Always present |
| R_IdleTime | 11 | Unit is 0.1 μs |  | Always present |
| If(ABS allocates resource for periodic ranging in AAI UL Relay zone) { |  |  |  |  |
| Allocation periodicity of the S-RCH | 2 | Indicates the periodicity of the S-RCH allocation. 0b00: Every frame 0b01: The second frame in every superframe 0b10: The second frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 0b11: The second frame in every 8th superframe, i.e., mod(superframe number, 8) = 0 |  | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation. The range of values is 0 ≤OSF ≤3. S-RCH is allocated in the (OSF +UAZ) subframe. |  | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| Start RP code information of the S-RCH | 4 | Indicates the ks which is the parameter controlling the start root index of the RP codes (rs0). rs0=6ks+1 The range of values is 0≤ks≤15. |  | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| NPE | 2 | Indicates the number of periodic code (NPE) according to the Table 917. |  | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| } |  |  |  |  |
| If(ABS allocates resource for BR channel in AAI UL Relay zone) { |  |  |  |  |
| UL BW REQ channel information | 2 | Indicates the number and the location of UL AAI subframe where the UL BW REQ channels are allocated. 0b00: i-th UL AAI subframe of UL relay zone in the first frame in every superframe 0b01: i-th UL AAI subframe of UL relay zone in the first and second frame in every superframe 0b10: i-th UL AAI subframe of UL relay zone in every frame 0b11: i-th and (i+1)-th UL AAI subframes of UL relay zone in every frame Where i-th is “first” if UL R-RTI = 0, and i-th is “second” if UL R-RTI=Ts. |  | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| UL BW REQ channel allocation | 4 | The DRU index for UL BW REQ channel within FPi defined by “Frequency partition location for UL control channels” in S-SFH SP1. |  | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) |  | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 |  | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| } |  |  |  |  |
| If(AAI_Relay_zone_AMS_allocation_indicator == 0b0){ |  |  |  |  |
| R_DCASSB0 | 5/4/3 | Indicates the number of subband-based CRUs in FP0 for AAI DL Relay zone. See 6.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_DCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 for AAI DL Relay zone. See 6.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_DCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subband, for FPi (i > 0) for AAI DL Relay zone. See 6.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_UCASSB0 | 5/4/3 | Indicates the number of total allocated CRUs, in a unit of a subband, for FPi (i > 0) for AAI DL Relay zone. See 6.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_UCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 for AAI UL Relay zone. See 6.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_UCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subbands, for FPi (i > 0) for AAI UL Relay zone. See 6.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| } |  |  |  |  |
| } |  |  |  | // TTR mode only |
| If (subordinate HR-MS is multimode MS acting as HR-RS in HR-Network) { |  |  |  |  |
| SA-PREAMBLE index | 10 |  |  | Always present |
| MS functionality maintenance indication | 1 | 0b0: MS functionality is maintained after role change 0b1: MS functionality is not maintained |  | Always present |
| Cell bar information | 1 | If Cell bar bit == 0b1, this cell shall not be allowed for network entry or reentry |  | Always present |
| If (subordinate HR-MS is acting as STR relay mode) { |  |  |  |  |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table 806, Table 807, and Table 808. |  | Always present |
| FFT size indication | 2 | 0b00: 2048 FFT 0b01: 1024 FFT 0b10: 512 FFT 0b11: reserved |  | Always present |
| DL carrier frequency for BS and RS (FBR_DL) | 10 | Indicates the DL carrier frequency in unit of 100KHz for MS acting as RS. Used to receive from HR-BS in the DL relay zone. |  | Present if needed |
| UL carrier frequency for BS and RS (FBR_UL) | 10 | Indicates the UL carrier frequency in unit of 100KHz for MS acting as RS. Used to transmit to HR-BS in the UL relay zone. |  | Present if needed |
| DL carrier frequency for RS and MS (FRM_DL) | 10 | Indicates the DL carrier frequency in unit of 100KHz for MS acting as RS in FDD. If the duplex mode is TDD, this carrier is used for DL/UL Used to transmit to subordinate HR-MS in the DL in FDD. Used to transmit/receive to/from subordinate HR-MS in TDD. |  | Shall be present if FRM_DL is different from that of HR-BS’ DL access zone |
| UL carrier frequency for RS and MS (FRM_UL) | 10 | Indicates the UL carrier frequency in unit of 100KHz for MS acting as RS in FDD. Used to transmit to subordinate HR-MS in the UL in FDD. |  | Shall be present if FRM_UL is different from that of HR-BS’ UL access zone |
| Superframe Number Action | 4 | LSBs of the superframe number when HR-RS start RS operation and apply the PHY operational parameters. |  | Always present |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| If (Transmitting or receiving station is affected HR-BS performing a TTR relay operation){ |  |  |  |  |
| Ta | 11 | Instructed value of timing advance Ta , in units of 0.1 μs | Present if receiving station is affected HR-BS performing TTR relay operation |  |
| Tbs | 5 | Instructed duration of the BS Operation mode, in units of frames | Always present |  |
| Trs | 5 | Instructed duration of the RS Operation mode, in units of frames | Always present |  |
| Start of RS Operation | 5 | Number of frames, not including the current one, that the HR-BS shall start the RS Operation mode; can be used to deduce the operation mode in the current frame. | Always present |  |
| R-RTI Indicator | 1 | Indicates the value of R-RTI when the HR-BS is assuming RS Operation mode 0b0: R-RTI = 0 0b1: R-RTI - Ts | Present if transmitting station is affected HR-BS performing TTR relay operation |  |
| } |  |  |  |  |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.