| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to the Text and Control Messages for BS-controlled Direct Communications |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0025 |  |
| Abstract | This contribution proposes minor modifications to the text and control messages to support BS-controlled HR-MS direct communications. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to the Text and Control Messages for BS-controlled

Direct Communications (16.1-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes minor modifications to the text and control messages to support BS-controlled HR-MS direct communications.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0025.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 6.12.2.2.1.1 as indicated}

6.12.2.2.1.1.1 Neighbor Discovery among associated HR-MSs (Use Case 1)

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) ranging signals so that other HR-MSs can try to receive and verify their neighbor relationship. Ranging preambles shall be used as PSA signals.

The process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS sends HR-DCV-CMD AAI-HR-DCV-CMD message to schedule one or multiple registered HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

- In the same HR-DCV-CMD Using AAI-HR-DCV-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for ranging signals.

- Each HR-MS that is scheduled to receive ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs may report their measurements to the serving HR-BS/HR-RS using HR-DCV-REP AAI-HR-DCV-REP message. Whether a receiving HR-MS shall report its measurements or not may be based on a threshold.

The transmission of HR-DCV-CMD AAI-HR-DCV-CMD can be described as follows. The HR-BS unicasts HR-DCV-CMD AAI-HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts HR-DCV-CMD AAI-HR-DCV-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the HR-DCV-CMD AAI-HR-DCV-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

{Remedy 2: Modify the last sentence of section 6.12.2.2.1.3.1 as indicated}

…

The serving HR-BS/RS schedules ranging between two HR-MSs through AAI-HR-DCV-CMD message.

{Remedy 3: Modify the last sentence of section 6.12.2.2.1.3.2 as indicated}

…

The serving HR-BS/RS schedules ranging between two HR-MSs through AAI-HR-DCV-CMD message.

{Remedy 4: Insert the following new rows into Table 683}

| No. | Functional Areas | Message names | Message description | Secuirty | Connection |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| TBD | BS-controlled neighbor discovery | AAI-HR-DCV-CMD | Neighbor discovery command | Encrypted/ICV | Unicast or multicast or broadcast |
| TBD | BS-controlled neighbor discovery | AAI-HR-DCV-REP | Neighbor discovery report | Encrypted/ICV | Unicast or multicast or broadcast |
| … | … | … | … | … | … |

{Remedy 5: Modify Sections 6.2.3.65.9 and 6.2.3.65.10 as indicated:}

6.2.3.65.9 AAI-HR-DCV-CMD message

HR-BS/RS sends AAI-HR-DCV-CMD message to instruct one or group of its associated HR-MS to carry out neighbor discovery. The field “Action” in AAI-HR-DCV-CMD message tells receiving HR-MS whether it (them) should transmit or receive the specified ranging signals.

Table 763dc1 – AAI-HR-DCV-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. |  |
| Action | 1 | 0b0: HR-MS to transmit the ranging signal as instructed. 0b1: HR-MS to receive the ranging signal as instructed. |  |
| If (Action == 0b0){ |  |  |  |
| Transmit power level | 5 | Unsigned integer from 1 to 64 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| } |  |  |  |
| Else{ |  |  |  |
| Report mode | 2 | Indicate if the report mode is exclusive (all receiving HR-MS should send AAI-HR-DCV-REP message) or triggered by threshold. 0b00: exclusive reporting 0b01: triggered-based reporting 0b10: for peer-to-peer synchronization and no reporting 0b00: reserved. |  |
| If (Report mode == 0b01){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.2.3.65.10 AAI-HR-DCV-REP message

As part of the neighbor discovery process among HR-MS associated to the same HR-BS/RS, HR-MS instructed to receive the ranging signals shall report the measurements of those signals it can properly decode in the AAI-HR-DCV-REP message.

Table 763dc2 – AAI-HR-DCV-REP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i = 0; i < Number of ranging codes to be reported; i++){ |  |  |  |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Received SINR | 4 | Indicates the received SINR of the ranging code. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Timing offset | 15 | Time offset, in units of Fs, of the received ranging signal, with respect to the frame timing of the HR-MS. MSB 1 bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1…16384] that corresponds to 0x0000 ~ 0x3FFF, respectively. | Optional |
| Frequency offset | 9 | Frequency offset, in units of 2% of the subcarrier spacing (ƒ), of the received ranging signal, with respect to the frequency of the HR-MS. MSB 1bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively | Optional |
| } |  |  |  |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.