| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to the Text and Control Messages for BS-controlled Direct Communications |  |
| Date Submitted | 2011-11-05 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0025 |  |
| Abstract | This contribution proposes minor modifications to the text and control messages to support BS-controlled HR-MS direct communications. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to the Text and Control Messages for BS-controlled

Direct Communications (16.1-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes minor modifications to the text and control messages to support BS-controlled HR-MS direct communications.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0025.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 6.12.2.2.1.1 as indicated}

6.12.2.2.1.1.1 Neighbor Discovery among associated HR-MSs (Use Case 1)

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) ranging signals so that other HR-MSs can try to receive and verify their neighbor relationship. Ranging preambles shall be used as PSA signals. The process can be described as follows:

The process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS sends HR-DCV-CMD AAI-HR-RNG-CMD message to schedule one or multiple registered HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

- In the same HR-DCV-CMD Using AAI-HR-RNG-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for ranging signals.

- Each HR-MS that is scheduled to receive ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs may report their measurements to the serving HR-BS/HR-RS using HR-DCV-REP AAI-HR-RNG-REP message. Whether a receiving HR-MS shall report its measurements or not may be based on a threshold.

The format of AAI-HR-RNG-CMD message is described in 6.3.2.3.98.x. The transmission of HR-DCV-CMD can be described as follows. The HR-BS unicasts HR-DCV-CMD AAI-HR-RNG-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts HR-DCV-CMD AAI-HR-RNG-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the HR-DCV-CMD AAI-HR-RNG-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

{Remedy 2: Modify the last sentence of section 6.12.2.2.1.3 as indicated}

6.12.2.2.1.3 Synchronization between HR-MSs involving in HR-MS DC/FTN direct communication

This section describes the process of maintaining synchronization between two HR-MSs involving in direct communication. that communicate directly with each other under HR-MS DC and FTN. The process is employed after HR-MS DC/FTN has been setup, and therefore should be differentiated from the discovery process described in 6.12.3.2.1.1. Synchronization between HR-MSs is classified into two levels:

- The fFrame-level synchronization should allows HR-MSs to share a common understanding of frame and/or superframe timing and configuration.

- The sSymbol-level synchronization should allows reliable (i.e. received within the appropriate reception threshold) bi-directional transmissions between HR-MSs to be received within an appropriate reception threshold.

Synchronization mechanisms are specified for three different use cases as follows.

6.12.2.2.1.3.1 Use case 1: both HR-MSs are within the coverage of HR-BS/RS

The following synchronization mechanisms are used for HR-MS DC/FTN scheduled in the uplink area of a frame.

Frame-level Synchronization:

When both HR-MSs are able to receive PA/SA-Preambles and DL control signals from a common serving HR-BS/HR-RS, they shall use these to achieve frame-level synchronization, (with respect to the common HR-BS/HR-RS and between themselves).  When both HR-MSs involved in DC or FTN are within the coverage of HR-BS/HR-RS, frame-level synchronization means the HR-MSs acquire DL synchronization with the serving HR-BS/HR-RS and are able to achieve system configuration and control messages.

Symbol-level Synchronization:

When the HR-MS/HR-MS direct link is scheduled in a UL area of a frame, the transmitting HR-MS shall follow the same timing advance as has been adjusted and agreed with the serving HR-BS/HR-RS. This means the transmitting HR-MS shall time its direct transmissions as if these are normal UL transmissions toward the serving HR-BS/HR-RS.

It is the responsibility of the receiving HR-MS to adjust its receive timing to match the time of arrival (TOA) of the signal transmitted by the other HR-MS. This time adjustment shall be achieved by the serving HR-BS/HR-RS scheduling the HR-MSs to transmit ranging sequences to each other. Based on a received ranging sequence, an HR-MS can estimate and correct its time offset with the transmitting HR-MS. To facilitate this process, tThe serving HR-BS/HR-RS shall transmit AAI-HR-RNG-CMD message to assign dedicated ranging resources sequences and ranging channels in UL area of a frame for HR-MS/HR-MS direct ranging.

To enhance bi-directional communication between HR-MSs, the serving HR-BS/HR-RS can allocate ranging resources to both involved HR-MSs in a single assignment. This allows the receiving HR-MS to transmit back a ranging sequence right after successfully processing the ranging sequence transmitted by the other HR-MS.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

{Remedy 3: Modify the last sentence of section 6.12.2.2.1.3.2 as indicated}

6.12.2.2.1.3.2  Use case 2: one HR-MS is inside and the other is outside the coverage of HR-BS/RS

The following synchronization mechanisms are used for HR-MS DC/FTN scheduled in the uplink area of a frame.

Frame-level Synchronization:

When two HR-MSs need to achieve frame-level synchronization and only one of them is within the coverage of the serving HR-BS/HR-RS, the inside-of-coverage HR-MS shall first acquires DL synchronization with the serving HR-BS/HR-RS (based on PA/SA-Preambles and control messages from the serving HR-BS/HR-RS). The inside-of-coverage HR-MS shall subsequently broadcast preambles and possibly network configuration information (NCI) for the outside-of-coverage HR-MS to co-synchronize . The transmission of preamble and NCI is as described in 6.12.3.2.2.1.

The inside-of-coverage HR-MS shall transmit PA/SA preambles at the first OFDMA symbols of 2nd and 3rd frames within each superframe. The NCI shall be transmitted in an UL area. The location of the NCI, relative to the transmitted preambles, shall be determinable by the outside-of-coverage HR-MS.

Symbol-level Synchronization:

Using the preambles and NCI transmitted by the inside-of-coverage HR-MS, the outside-of-coverage HR-MS shall adjust its timing to receive messages transmitted from the inside-of-coverage HR-MS. To further improve synchronization in this direction, the inside-of-coverage HR-MS can transmit ranging signal toward the outside-of-coverage HR-MS so that this node can estimate and correct its time/frequency offsets. Symbol-level synchronization in the opposite direction, i.e., from the outside-of-coverage of HR-MS toward the inside-of-coverage HR-MS shall be achieved by the outside-of-coverage HR-MS transmitting ranging signal toward the inside-of-coverage HR-MS. Upon processing the received ranging signal, the inside-of-coverage HR-MS can either adjust its own receive timing or request the outside-of-coverage HR-MS to adjust the transmit timing. The serving HR-BS/RS schedules ranging between two HR-MSs through AAI-HR-RNG-CMD message.

The serving HR-BS/RS schedules ranging between two HR-MSs through HR-DCV-CMD message.

{Remedy 4: Insert the following new rows into Table 683}

| No. | Functional Areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| TBD | BS-controlled neighbor discovery | AAI-HR-RNG-CMD | HR-MS/HR-MS ranging command | Encrypted/ICV | Unicast or multicast or broadcast |
| TBD | BS-controlled neighbor discovery | AAI-HR-RNG-REP | HR-MS/HR-MS ranging report | Encrypted/ICV | Unicast or multicast or broadcast |
| … | … | … | … | … | … |

{Remedy 5: Modify Sections 6.2.3.65.9 and 6.2.3.65.10 as indicated:}

6.2.3.65.9 AAI-HR-RNG-CMD HR-DCV-CMD message

HR-BS/RS sends AAI-HR-RNG-CMD message to instruct one or group of its associated HR-MS to carry out neighbor discovery. The field “Action” in AAI-HR-RNG-CMD message tells the receiving HR-MS whether it (them) should transmit or receive the specified ranging signals. The serving HR-BS/HR-RS can allocate ranging resources to both involved HR-MSs in a single assignment. This allows the receiving HR-MS to transmit back a ranging sequence right after successfully processing the ranging sequence transmitted by the other HR-MS.

Table 763dc1 – AAI-HR-RNG-CMD HR-DCV-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. |  |
| Action | 2 | 0b00: ranging for neighbor discovery and receiving node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out receiving 0b10: DC/FTN periodic ranging transmission 0b11: Reserved |  |
| Reversed action offset | 4 | 0b0001-0b1111: Indicates number of frames that this HR-MS should switch to receive the same ranging code in the same ranging slot 0b0000: no such reversed action. |  |
| if (Action == 0b00¦¦0b10) ¦¦ (Reversed action offset > 0x0){ |  |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| } |  |  |  |
| If (Action == 0b10){ |  |  |  |
| Periodicity | 2 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames |  |
| Tx/Rx Offset | 1 | Offset between transmitting and receiving ranging from the other HR-MS: 0b0: Offset = 1 frame 0b1: Offset = 3 frames |  |
| } |  |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |  |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| } |  |  |  |

6.2.3.65.10 AAI-HR-RNG-REP HR-DCV-REP message

HR-MS instructed to receive the ranging signals from other HR-MS shall report the measurements of those signals it can properly decode in the AAI-HR-RNG-REP message.

Table 763dc2 – AAI-HR-RNG-REP HR-DCV-REP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i = 0; i < Number of ranging codes to be reported; i++){ |  |  |  |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Received SINR | 4 | Indicates the received SINR of the ranging code. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Timing offset | 15 | Time offset, in units of Fs, of the received ranging signal, with respect to the frame timing of the HR-MS. MSB 1 bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1…16384] that corresponds to 0x0000 ~ 0x3FFF, respectively. | Optional |
| Frequency offset | 9 | Frequency offset, in units of 2% of the subcarrier spacing (ƒ), of the received ranging signal, with respect to the frequency of the HR-MS. MSB 1bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively | Optional |
| } |  |  |  |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.