| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text and Control Messages for MAC-layer Support for BS-controlled HR-MS Forwarding to Network (16.1-based) |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0025 |  |
| Abstract | This contribution proposed modifications for text and MAC control messages for BS-controlled HR-MS forwarding to network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text and Control Messages for MAC-layer Support for

BS-controlled FTN (16.1-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposed modifications for text and MAC control messages for BS-controlled HR-MS forwarding to network.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0025.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 6.12.3.1 and 6.12.3.2 as indicated}

6.12.3.1 General description

In HR-MS Forwarding to Network, an HR-MS forwards user data and control signaling between an HR-MS and an HR infrastructure station. The user data and control signaling do not go through higher layer at the forwarding HR-MS. The origination and termination of the user data and control signaling are at the forwarded HR-MS and the HR infrastructure station respectively and vice versa.

HR-MS Forwarding to Network is applicable when 1) the forwarded HR-MS and the forwarding HR-MS are in coverage of and directly associated to an infrastructure station; 2) the forwarding HR-MS is in coverage of and directly associated to an HR infrastructure station, while the forwarded HR-MS is out of coverage of any HR infrastructure stations.

Under BS-controlled direction communication described in 6.12.2.2,BS-controlled HR-MS forwarding to network is described in 6.12.3.2.

Using talk-around direct communication described in 6.12.2.3, HR-MS forwarding to network is described in 6.12.3.3.

6.12.3.2 BS-controlled HR-MF forwarding to network (FTN)

The general operation of BS-controlled HR-MS forwarding to network (FTN) can be described as follows:

- At the beginning of the process, the serving HR-BS/RS instructs one or a group of its associated HR-MS to broadcast discoverable signals so that an out-of-coverage HR-MS can start network entry and get associated to the HR-BS/RS. This is described in 6.12.3.2.1.1.

- After the out-of-coverage HR-MS has completed network entry and become an attached forwarded HR-MS, the forwarding HR-MS helps forward control and data messages between the forwarded HR-MS and the serving HR-BS/RS. MAC-layer support for this process is described in 6.12.3.2.1 while PHY-layer support for this is described in 6.12.3.2.2.

{Remedy 2: Modify Section 6.12.3.1 and 6.12.3.2 as indicated}

6.12.3.2.1.1.1 Coverage Extension with NCI Preceding Initial Ranging

The procedure starts when serving HR-BS/RS transmits an HR-CEX-CMD AAI-FN-CONFIG-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process. The format of HR-CEX-CMD AAI-FN-CONFIG-CMD message is defined in Table 780 and includes the following information:

- Idx and n: the index of the SA-Preamble and index of SA-Preamble carrier set, respectively, to be transmitted by the scheduled HR-MS (or group of HR-MS). These two parameters are indicated through the IDCell field. The scheduled HR-MS shall also transmit PA-Preamble, however this preamble is the same as that transmitted by serving HR-BS and therefore need not be specified.

- tadv: the timing advance that should be used by the scheduled HR-MS (or group of HR-MS) when transmitting PA/SA-Preambles. This offset is with respect to the DL frame timing as seen by the scheduled HR-MS.

- nstart : the superframe number in which the new coverage-extending process should be started.

- mprep: the number of superframes, counting from the superframe with number nstart, that the scheduled HR-MS (or group of HR-MS) shall transmit PA/SA-Preambles without transmitting NCI. The value of mprep shall not be greater than a system parameter Nprep_max.

- mnci: the number of superframes in which NCI is to be transmitted, i.e., the NCI shall be transmitted in superframes (nstart + mprep) to (nstart + mprep + mnci -1).

- mrng: the number of ranging opportunities, together with their particular configurations.

- The number of ranging opportunities per superframe, this value can be one (ranging opportunity is allocated in 2nd frame) or two (ranging opportunities are allocated in 2nd and 4th frames).

- The location of the channel used to feedback to the new HR-MS (if pre-allocated).

- The power setting to be used by scheduled HR-MS (or group of HR-MS) when transmitting preambles and NCI.

- Other information to be transmitted in NCI (see Error: Reference source not found and Error: Reference source not found).

Upon receiving the HR-CEX-CMD AAI-FN-CONFIG-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting PA/SA-Preambles in superframe nstart. The forwarding HR-MS transmits PA-Preamble in the first symbol of the 2nd frame and SA-Preamble in the first symbol of the 3rd frame of every superframe with superframe number from nstart. to (nstart + mprep. - 1).

Next, the forwarding HR-MS transmits NCI in superframes with number from (nstart + mprep) to (nstart + mprep + mnci – 1). During these mnci superframes, the forwarding HR-MS continue to transmit PA/SA-Preambles as specified above. The transmission of NCI can be described as follows:

- NCI partitioning: The NCI is divided into two subpackets, i.e., Initial Network Configuration Information (I-NCI) and Supplementing Network Configuration Information (S-NCI). I-NCI is transmitted first, with its location determinable from the SA-Preamble index, Idx, and subcarrier set index, n, of the SA-Preamble transmitted by the forwarding HR-MS. The content of I-NCI is described in Error: Reference source not found. The content of S-NCI is described in Error: Reference source not found. The location of S-NCI is specified in the I-NCI. The physical structure of I-NCI and S-NCI are described in 6.12.3.2.2.2.

- Transmission of I-NCI: The I-NCI shall be transmitted in the NI-NCI  Distirubted LRUs (DLRUs) in the first subframe of a superframe, with the particular resource index being determinable from the SA-Preamble transmitted by the forwarding HR-MS. Within the selected subframe, the I-NCI shall occupy the last 5 OFDM symbols, i.e., effectively forming a Type-3 subframe.

- Transmission of S-NCI: the resource allocated for S-NCI shall be defined in I-NCI (see Error: Reference source not found). In general, this allocation depends on the IDCell and the frame configuration of the serving HR-BS/RS.

When a new HR-MS detects PA/SA-Preambles and I/S-NCI transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the following process shall be carried out:

- The new HR-MS, after acquiring downlink synchronization and uplink transmission parameters shall pick one of the ranging codes specified in S-NCI using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel opportunity specified in S-NCI.

- Upon receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message. As specified in S-NCI, mrng ranging opportunities shall be allocated. If the new HR-MS has previously transmitted a ranging code in a ranging opportunity which is not the last opportunity, the HR-MS shall expect to receive an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message from the forwarding HR-MS before the next ranging opportunity. If the new HR-MS does not receive such an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message, it shall pick another ranging code using a uniform random process and transmit on the coming ranging opportunity. The new HR-MS may ramp-up its transmit power as specified in S-NCI. If the new HR-MS transmits a ranging code in the last ranging opportunity specified in S-NCI, it shall expect to receive an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message within Tx1 Timer.

- The HR-CEX-RNG-ACK AAI-FN-RNG-ACK message provides responses to all the successfully received and detected ranging preamble codes in the ranging opportunity preceding its transmission. There are four possible ranging status responses from the forwarding HR-MS to the new HR-MS in the HR-CEX-RNG-ACK AAI-FN-RNG-ACK message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a BW allocation in the uplink for the new HR-MS to send HR-CEX-RNG-REQ AAI-RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

  - Continue with 2nd Phase: This status notification is only applicable if the current coverage extension process follows the Two-Phase approach, as specified in HR-CEX-CMD AAI-FN-CONFIG-CMD message. The forwarding HR-MS informs the new HR-MS to prepare to receive post-access preambles and post-access S-NCI. This status shall only be sent once, i.e., it shall not be given when the new HR-MS has already been carrying out ranging using post-access preambles and post-access S-NCI from forwarding HR-MS.

- Based on the received response of ranging status, the new HR-MS performs the following:

  - Upon receiving a Continue status notification and parameter adjustments in HR-CEX-RNG-ACK AAI-FN-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by randomly selecting one ranging code and transmitting in the next available ranging opportunity, or as instructed in the HR-CEX-RNG-ACK AAI-FN-RNG-ACK message.

  - Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send HR- send HR-CEX-RNG-REQ AAI-RNG-REQ message. The BW shall be allocated to fit the size of send HR-CEX-RNG-REQ. The forwarding HR-MS forwards the corresponding request to the serving HR-BS, and subsequently acts on behalf of the HR-BS in sending an HR-CEX-RNG-RSP AAI-RNG-RSP message to the new HR-MS. If the new HR-MS does not receive BW allocation for HR-CEX-RNG-REQ AAI-RNG-REQ within Tx or the HR-CEX-RNG-REQ/RSP AAI-RNG-REQ/RSP is not completed in [TBD] frames, it stops the process and try to associate with another  HR-MS or a HR-BS/RS.

  - Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process. It shall not attempt to restart initial ranging with a forwarding HR-MS transmitting the same SA-Preamble in the next nclosed coverage extending process.

  - Upon receiving Continue with 2nd Phase status notification, the new HR-MS waits for post-access PA/SA-Preambles and post-access S-NCI to be transmitted by forwarding HR-MS. Upon receiving post-access PA/SA-Preambles and S-NCI, the new HR-MS start a new round of initial ranging as follows steps as described above. and carries out ranging process as described above. The initial transmission power for the ranging is determined based on the last ranging prior to detection of post-access SA-Preamble and S-NCI.

- HR-BS assigns and transfers a TSTID by AAI-RNG-RSP message when ranging status is Success. Initial ranging process is over after receiving the HR-CEX-RNG-RSP AAI-RNG-RSP message. The TSTID is used until STID is newly assigned and received at successful registration.

In the above procedure, the transmission of initial ranging code by the new HR-MS shall follow:

- For frame configuration without FDM-based UL PUSC zone support:

  - In the time domain, the S-RCH allocation is specified by the subframe offset OSF transmitted in S-NCI, i.e., ranging opportunity is located at mod(OSF + 1, NUL)th uplink AAI subframe of a particular frame. The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell of the superordinate HR-BS (transmitted in I-NCI) and the allocated number of subbands RSB according to the following equation, where IDcell is defined in 6.3.5.1.2 and RSB is LSB-CRU,FPi/4, where LSBCRU, FPi is the number of allocated subband CRUs as defined in 6.3.7.3 for FPi corresponding to reuse 1 partition or power-boosted reuse 3 partition only if there is no reuse 1 partition.

ISB,s = mod(IDcell + 1, RSB),

where ISB, s denotes the subband index (0, …, RSB– 1) for ranging resource allocation among RSB subbands.

- For frame configuration with FDM-based UL PUSC zone support:

  - The un-associated HR-MS shall transmit ranging preamble in a similar way to what specified in 6.3.8.2.4.3.

After a successful initial ranging, the forwarding HR-MS shall assist HR-BS and the new HR-MS to exchange control messages to complete basic capability negotiation, HR-MS authorization and key exchange, and registration.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI (i.e., in I/S-NCI messages), the forwarding HR-MS shall monitor the specified ranging opportunities for any initial-ranging attempt by new HR-MS. If no such ranging attempt is detected, the coverage extension process terminates.

- When the forwarding HR-MS detects one or more ranging codes transmitted on the allocated ranging opportunities, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An HR-CEX-RNG-ACK AAI-FN-RNG-ACK message shall be transmitted to the new HR-MS.

- If resource in the uplink area has been pre-allocated by HR-BS for the forwarding HR-MS to transmit HR-CEX-RNG-ACK AAI-FN-RNG-ACK message, the forwarding HR-MS shall transmit an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message to the new HR-MS, with corresponding status and adjustments (if necessary). The HR-BS shall also monitor the pre-allocated resource for the HR-CEX-RNG-ACK AAI-FN-RNG-ACK in order to carry out any further proactive resource allocation for the coverage extending process.

- If resource has not been pre-allocated for the forwarding HR-MS to transmit HR-CEX-RNG-ACK AAI-FN-RNG-ACK message, the forwarding HR-MS shall transmit an HR-CEX-RNG-REP AAI-FN-RNG-REP report to the HR-BS. The HR-CEX-RNG-REP AAI-FN-RNG-REP message contains signal strength and possibly necessary adjustments for the new HR-MS. The HR-BS may receive HR-CEX-RNG-REP AAI-FN-RNG-REP messages from multiple scheduled forwarding HR-MS. The HR-BS is supposed to follow up with an HR-CEX-FLU AAI-FN-RNG-FLU message before the next allocated ranging opportunity of the coverage extending process if there is any. Otherwise, if this is the last ranging opportunity as specified in S-NCI, the HR-CEX-FLU AAI-FN-RNG-FLU message is supposed to be transmitted within Tx2 Timer, where Tx2 < Tx1. The HR-CEX-FLU AAI-FN-RNG-FLU message specifies the forwarding HR-MS that shall transmit an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message to the new HR-MS, (if such a message needs to be transmitted). The resource to transmit such an HR-CEX-RNG-ACK AAI-FN-RNG-ACK message has been pre-specified in the original HR-CEX-CMD AAI-FN-CONFIG-CMD message, and has been made known to the new HR-MS through the S-NCI.

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit HR-CEX-RNG-REQ AAI-RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the A-MAP and inform the new HR-MS accordingly (using a CDMA Allocation IE). The resource allocation for HR-CEX-RNG-REQ message shall be preceded by HR-UL-RCV-IE which allocates uplink resource for the forwarding HR-MS to relay ranging, registration, capability negotiation, security exchanges from the new HR-MS to serving HR-BS.

- The HR-BS may request the forwarding HR-MS to report any ranging attempt, whether they have crossed the response threshold or not, for management purposes.

{Remedy 3: Modify Section 6.12.3.2.1.6 as indicated }

6.12.3.2.1.6 HO procedures involving forwarding HR-MS Changes in FTN operation modes

6.12.3.2.1.6.1 General description

Normal Operation

Use Case 1

Use Case 2

1

2

3

4

5

6

Figure 1 – Transitions between different (FTN) operating modes.

Figure 927 highlights changes in HR-MS operation, from normal mode in which HR-MS communicate directly with serving HR-BS/RS to the two possible use cases of HR-MS FTN.

Figure 927 illustrates possible changes in HR-MS operation modes. Normal mode refers to the scenario when HR-MS communicates directly with its serving HR-BS/RS, FTN use case 1 is when both forwarding and forwarded HR-MS are within the coverage of HR-BS/RS, and use case 2 is when the forwarded HR-MS is outside-of-coverage of HR-BS/RS.

For the transition from normal operation to FTN use case 1, serving HR-BS/RS is the entity that makes the decision and informs the two involved HR-MS of the FTN connection setup.

For the transition from normal operation to FTN use case 2, serving HR-BS/RS first sends AAI-FN-CONFIG-CMD message to instruct the forwarding HR-MS to start relevant transmissions, e.g., of SA/PA-Preambles, NCI …, unless the HR-MS has already been doing so. Next the HR-BS/RS informs the other HR-MS, the one that is about to go out of HR-BS/RS coverage, of the potential forwarding HR-MS, together with relevant operation parameters such as IDCell, ranging opportunities. This may be follow by ranging between the two HR-MS before the FTN connection setup.

For the transitions from FTN use case 1 to normal operation, the serving HR-BS/RS makes the decision and initiates FTN connection release. For the transition from FTN use case 2 to normal operation, the forwarded HR-MS can monitor the channel to see if it can receive PA/SA-Preambles transmitted by the serving HR-BS/RS. If such PA/SA-Preambles are detected, and the forwarded HR-MS can subsequently achieve DL synchronization, the normal network entry process with the HR-BS/RS shall be applied. The transition from FTN use case 1 to FTN use case 2 is similar to the transition from normal operation to FTN use case 2.

In both FTN use cases 1 and 2, the forwarded HR-MS carries out handover to a neighboring HR-BS/RS in the same manner as in normal operation.

6.12.3.2.1.6.2 Transitions from normal operation to FTN operation:

Transitions from normal operation to Use case 1 or 2 can be triggered by the outcomes from the neighbor discovery process. It can also be due to changes in link, interference, and other operating conditions within the network.

6.12.3.2.1.6.3 Transitions from FTN use case 1 to normal & FTN use case 2:

Transitions from Use case 1 of HR-MS FTN to either Normal operation or Use case 2 can be triggered by changes in the link conditions between HR-BS, Forwarding HR-MS, and Forwarded HR-MS. To detect these changes, idle periods can be scheduled by HR-BS for the forwarding and forwarded HR-MS to measure link qualities.

6.12.3.2.1.6.4 Transitions from FTN use case 2 to normal & FTN use case 1:

Transitions from Use case 1 of HR-MS FTN to either Normal operation or Use case 2 can be triggered by changes in the link conditions between HR-BS, and forwarding/forwarded HR-MSs. To detect these changes, idle periods can be scheduled by HR-BS for Forwarding HR-MS, Forwarded HR-MS to measure link qualities.

{Remedy 3: Insert the following new rows into Table 683}

| No. | Functional Areas | Message names | Message description | Secuirty | Connection |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| TBD | BS-controlled HR-MS forwarding to network | AAI-FN-CONFIG-CMD | Forwarding to network configuration |  | Unicast or multicast |
| TBD | BS-controlled HR-MS forwarding to network | AAI-FN-RNG-ACK | Acknowledging successful receiving of ranging preamble by forwarding HR-MS |  | Broadcast (toward new HR-MS that is carrying out network entry through the forwarding HR-MS) |
| TBD | BS-controlled HR-MS forwarding to network | AAI-FN-RNG-REP | Forwarding HR-MS report to serving HR-BS/RS the reception of ranging signal from new HR-MS | Encrypted/ICV | Unicast |
| TBD | BS-controlled HR-MS forwarding to network | AAI-FN-RNG-FLU | HR-BS/RS selects one or group of HR-MS to follow-up with the ranging process of new HR-MS | Encrypted/ICV | Unicast or multicast |
| … | … | … | … | … | … |

{Remedy 4: Modify Sections 6.2.3.65.36 and 6.2.3.65.37 as indicated:}

6.2.3.65.36 HR-CEX-CMD AAI-FN-CONFIG-CMD Message

An HR-BS/RS transmits AAI-FN-CONFIG-CMD message to instruct an HR-MS or group of HR-MS to start the coverage extension process. This involves the scheduled HR-MS to transmit PA/SA-Preambles and network configuration information so that an HR-MS outside of HR-BS/RS coverage may start network entry (through the forwarding HR-MS).

Table 763fn1 – HR-CEX-CMD AAI-FN-CONFIG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Superframe Number Action (nstart) | 4 | LSBs of the superframe number in which the coverage-extending cycle should be started. | Mandatory |
| IDCell | 10 | IDCell (SA-Preamble index Idx and subcarrier set index n) to be used by the scheduled HR-MS (If Two-Phase Discovery is used, the HR-BS should assign pre-access SA-Preamble to groups of HR-MS based on their service characteristics.) | Mandatory |
| Number of Preamble-only Superframes (mprep) | 4 | Indicates the number of superframes (starting from superframe with number nstart) in which the scheduled HR-MS should broadcast PA/SA-Preambles. No NCI shall be transmitted during these superframes. | Mandatory |
| Number of Superframes with NCI (mnci ) | 2 | Indicates the number of superframes (starting from superframe with number nstart + mprep) that the scheduled HR-MS transmits NCI. | Mandatory |
| Start RP code information of the S-RCH | 4 |  |  |
| Number of Ranging Opportunities (mrng) | 2 | Indicates the number of ranging opportunities given to outside-of-coverage HR-MS | Mandatory |
| Subframe offset of Ranging Channel | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 | Mandatory |
| Indicator of ranging opportunities per superframe | 1 | 0b0: 1 ranging opportunity per superframe, and located at 2nd frame 0b1: 2 ranging opportunities per superframe, and located at 2nd and 4th frames |  |
| If (Support of WirelessMANOFDMA with FDM-based UL PUSC Zone){ |  |  |  |
| Subframe offset of the RCH | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 |  |
| Start RP code information of the RCH | 4 | Indicates the kns, which is the parameter for start of the RP code group (rns0). rns0(kns) = 16 × kns + 1, kns = 0,1, ...,15 |  |
| Number of RP codes allocated for coverage extension ranging | 2 | Indicates the number of RP codes and the particular codes that can be used for ranging with the forwarding HR-MS. |  |
| }else{ |  |  |  |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation |  |
| Start RP code information of the S-RCH |  | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |  |
| Transmission timing offset of SRCH | 3 | Indicates NRTO, which is the parameter used for the calculation of the sample number, TRTO, which is applied to advance the ranging signal transmission timing relative to the defined uplink transmission timing point based on the frame structure from AMS perspective when AMS conducts initial or handover ranging in a femtocell. TRTO = floor(NRTO × (Tg – 2) × FS)(samples) where NRTO = min( RTD ⁄ (Tg–2) , 7), and RTD is the round trip delay from the femto-ABS to the overlay macro-ABS. The range of values is 0 ≤ NRTO ≤ 7. |  |
| } |  |  |  |
| HR-MS Preamble Timing Advance (tadv) | [TBD] | Timing advanced that should be used by scheduled HR-MS when transmitting PA/SA-Preamble, relative to the beginning of each frame as seen by the scheduled HR-MS. | Optional |
| HR-MS EIRP | 5 | Unsigned integer from 1 to 64 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm | Mandatory |
| HR-MS to HR-MS Feedback Resource Index | 11 | 512 FFT size: 0 in first2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. | Optional |
| HR-MS to HR-BS Report Resource Index | 11 | 512 FFT size: 0 in first2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. | Optional |
| If(Two-Phase Discovery){ |  |  |  |
| Threshold | [TBD] |  |  |
| If(Post-access parameters to be pre-assigned){ |  |  |  |
| Post IDCell | 10 | IDCell (SA-Preamble index Idx and subcarrier set index n) to be used by the scheduled HR-MS after detecting ranging signal above a specified threshold. (The HR-BS should assign post-access SA-Preamble such that they will be unique within physical proximity.) |  |
| Number of Superframes with NCI (mnci ) | 2 | Indicates the number of superframes (starting from superframe with number nstart + mprep) that the scheduled HR-MS transmits NCI. | Mandatory |
| Start RP code information of the S-RCH | 4 |  |  |
| Number of Ranging Opportunities (mrng) | 2 | Indicates the number of ranging opportunities given to outside-of-coverage HR-MS | Mandatory |
| Subframe offset of Ranging Channel | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 | Mandatory |
| } |  |  |  |
| } |  |  |  |

6.2.3.65.37 HR-CEX-RNG-ACK AAI-FN-RNG-ACK Message

A forwarding HR-MS transmits AAI-FN-RNG-ACK to acknowledge successful reception of a ranging preamble transmitted by a new HR-MS. The AAI-FN-RNG-ACK provides ranging status and possible time/frequency/power adjustment for the new HR-MS.

Table 763fn2 – HR-CEX-RNG-ACK AAI-FN-RNG-ACK message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (j = 0; j < N_Received_Codes; j++) { |  |  |  |
| Ranging Preamble Code Index | 2 | Ranging preamble code index received in this ranging opportunity. |  |
| Ranging Status | 2 | Indicate whether ranging preamble code or UL burst is received within acceptable limits by forwarding HR-MS. 0b00 = success 0b01 = continue 0b10 = abort 0b11 = continue with 2nd phase |  |
| If(Ranging Status == 0b00 or 0b01){ |  |  |  |
| Adjustment parameters indication (API) | 3 | Bit 0: Time offset adjustment indication. Bit 1: Power level adjustment indication Bit 2: Frequency offset adjustment indication |  |
| If( API Bit 0==0b1 ) { |  |  |  |
| Timing offset adjustment | 15 | Amount of time required to adjust AMS transmission (in units of 1/Fs). MSB 1 bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1..16384] that corresponds to 0x0000 ~ 0x3FFF, respectively. The AMS shall advance its transmission time if the value is negative (i.e., MSB = 0b1) and delay its transmission time if the value is positive (i.e., MSB = 0b0). |  |
| } |  |  |  |
| If( API Bit 1==0b1 ) { |  |  |  |
| Power level adjustment | 4 | Power level adjustment that expresses the change in power level (in multiples of 1 dB) that the AMS shall apply to its current transmission power for initial ranging. MSB 1 bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 3 bits represent power level correction value of [1..8] that corresponds to 0b000 ~ 0b111, respectively |  |
| } |  |  |  |
| If (API Bit 2==0b1) { |  |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (ƒ) multiplied by the 9-bit number interpreted as a signed integer. MSB 1bit represents the sign of the value. That is, the value is negative(–) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively |  |
| } |  |  |  |
| } |  |  |  |

{Remedy 5: Insert two new sections:}

6.2.3.65.x AAI-FN-RNG-REP

Forwarding HR-MS transmits AAI-FN-RNG-REP message to report to serving HR-BS/RS those ranging signals it can receive from new HR-MS, as part of the coverage extension process.

6.2.3.65.(x+1) AAI-FN-RNG-FLU

Serving HR-BS/RS transmits AAI-FN-RNG-FLU to instruct one or a group of HR-MS to follow-up with the ranging process of a new HR-MS, as part of the coverage extension process.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.