| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Support of Local Forwarding (Rev3-based) |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Shoukang Zheng, Haiguang Wang, Jaya Shankar, Hoang Anh Tuan, Wai Leong Yeow and Joseph Chee Ming Teo Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2252 E-mail: skzheng@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose to support local forwarding for IEEE 802.16n network for more efficient communication |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for Support of Local Forwarding (Rev3-based)

Shoukang Zheng, Haiguang Wang, Jaya Shankar, Hoang Anh Tuan, Wai Leong Yeow and Joseph Chee Ming Teo

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

The SRD in 802.16n requires that HR-Network should allow local forwarding, which allows one HR-MS to communicate to one or more HR-MSs via infrastructure station without going through the backhaul. In this document, we propose the procedures to support local forwarding based on Rev3.

# 2. Proposed protocol

# 3. Text Proposal for IEEE 802.16n AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Insert the following text into line 20 page 58 in the 802.16n AWD)]

6.12.6.1 Detection of LF Opportunity

Local forwarding (LF) opportunity may be detected by HR-infrastructure station or other manner (e.g. ASN-GW).  If there is no backhaul, HR infrastructure station shall detect the LF. In this case, the methods without ASN-GW engaging may be used to determine LF opportunity.

6.12.6.2 LF Setup

When HR-infrastructure station detects the LF opportunity, it may send control message to ASN to get the permission for the forwarding to be done by HR-infrastructure station without going through backhaul. If the ASN detects the opportunity of LF, it communicates with HR-infrastructure station via R6 interface. The details of control messages are out of scope.

If HR-BS/ASN detects the LF opportunity during DSA procedure, data path setup shall be deferred for the DSA-REQ from the source HR-MS. HR-BS will continue the downlink setup through control messages AAI-DSA-REQ/ RSP/ACK between HR-BS and the destination HR-MS. After downlink setup is complete, the uplink setup proceeds to complete after HR-BS sends AAI-DSA-RSP to the source HR-MS and HR-MS confirms by AAI-DSA-ACK.

Alternatively, if the detection is performed to find out the LF opportunity after connection establishment, HR infrastructure station can initiate the similar handshaking process to set up the LF. HR-BS performs LF as described in 16.6.3. HR-BS may communicate with ASN entities to remove data path for the corresponding uplink flow from the source HR-MS and the corresponding downlink flow to the destination HR-MS.

To terminate the LF, when HR infrastructure station receives control message AAI-DSD-REQ from HR-MS (the source and/or the destination) to terminate the connection or it decides to stop LF due to other reasons, it may proceed with the handshaking procedure similar to the setup for LF. HR-BS shall also send AAI-DSD-REQ to the other HR-MS to terminate the connection, following the normal procedure of connection termination.

16. 6.3 Data Traffic Forwarding

When LF is determined and the FIDs for uplink flow from the source to its serving infrastructure station and downlink flow from the serving infrastructure station to the destination HR-MS are assigned after connection setup, HR-infrastructure station shall forward the received data traffic from the source HR-MS to the destination HR-MS locally based on STIDs of the source HR-MS and the destination HR-MS, uplink FID of the source HR-MS and downlink FID of the destination HR-MS.

The data traffic forwarded locally by HR-BS shall follow MAC PDU formats in 6.2.2 and construction and transmission of MPDUs shall follow 6.2.3.

16.6.4 Handover

In handover, there are two cases for LF: case 1) when either the source or the destination is going out of the control of serving infrastructure station that performs LF for it; case 2) when both the source and the destination HR-MSs are going to be under the control of the same infrastructure station that shall perform LF for them.

In case 1, serving infrastructure station has to relinquish the LF since source and destination are not under the same infrastructure station anymore. This results in the connection re-establishment after handover, which is same as the normal procedure without LF. After the connection re-establishment, data flow forwarding shall follow the normal procedure without LF. HR-infrastructure station may update ASN entities on the removal of LF. The details of the messages are out of scope.

In case 2, after handover, the target infrastructure station or its associated ASN shall detect the LF opportunity for the handovering HR-MS. During the connection re-establishment or data flow forwarding, once the opportunity of LF is determined, the target infrastructure station performs LF for the data traffic based on STIDs of source and destination, uplink FID of source and downlink FID of destination as in 6.12.6.3.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0024, “Draft P802.16n Draft AWD,” Oct 2011.