| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Support of Local Forwarding (Rev3-based) |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Shoukang Zheng, Haiguang Wang, Jaya Shankar, Hoang Anh Tuan, Wai Leong Yeow and Joseph Chee Ming Teo Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2252 E-mail: skzheng@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose to support local forwarding for IEEE 802.16n network for more efficient communication |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for Support of Local Forwarding (Rev3-based)

Shoukang Zheng, Haiguang Wang, Jaya Shankar, Hoang Anh Tuan, Wai Leong Yeow and Joseph Chee Ming Teo

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

The SRD in 802.16n requires that HR-Network should allow local forwarding, which allows one HR-MS to communicate to one or more HR-MSs via infrastructure station without going through the backhaul. In this document, we propose the protocol to support local forwarding for unicast flow.

# 2. Proposed protocol

# 3. Text Proposal for IEEE 802.16n AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Insert the following text into line 20 page 58 in the 802.16n AWD)]

6.12.6.1 Detection of LF Opportunity

When local forwarding opportunity (LF) is determined, an HR-MS may communicate with one or more HR-MSs via an HR-infrastructure station without going through the backhaul. HR-infrastructure station may detect the LF opportunity. Otherwise, LF is detected by upper layer, which is outside of this specification. If there is no backhaul, HR infrastructure station (either HR-BS or HR-RS) shall detect the LF.

6.12.6.2 LF Setup and Termination

When HR-infrastructure station detects the LF opportunity, it may send control message to ASN to get permission for the LF to be done by HR-infrastructure station.

An infrastructure station capable of providing local forwarding shall (re)assign and manage the uplink CID for the source HR-MS and downlink CID for destination HR-MS during DSA procedure. Any available CID may be used for the local forwarding (i.e., there are no dedicated CIDs for local forwarding connections). An HR-MS may not be aware of local forwarding but shall follow the same procedure defined in 6.3.

If HR-BS/ASN detects the LF opportunity during DSA procedure, data path setup for the current DSA-REQ shall be deferred. The source HR-MS shall wait for HR-BS setup service flow on the downlink through DSA transaction between HR infrastructure station and the destination HR-MS. After service flow on downlink is setup, the HR infrastructure station shall continue the service flow setup on uplink by sending DSA-RSP to the source HR-MS. The source HR-MS confirms with DSA-ACK after receiving the DSA-RSP.

Alternatively, if LF opportunity is detected after connection establishment, HR infrastructure station can initiate the DSA-REQ to set up the LF. HR infrastructure station performs LF as described in 16.6.3. HR-BS may communicate with ASN entities to remove data path for the corresponding uplink flow from the source HR-MS and the corresponding downlink flow to the destination HR-MS.

When DSD procedure is performed, LF is terminated. If the destination HR-MS terminates the downlink service flow, HR-infrastructure station may setup a data path for the flow from the source HR-MS.

16. 6.3 Data Traffic Forwarding

When local forwarding is adopted for a flow,  the HR-infrastructure station forwards the received data traffic from the source HR-MS to the destination HR-MS based on the CIDs of the flow.

Each MAC PDU shall begin with a fixed-length MAC Header and its format is the same as that are defined in 6.3.2.1. The header may be followed by the Payload of the MAC PDU. If present, the Payload shall consist of zero or more subheaders and zero or more MAC SDUs and/or fragments thereof. The payload information may vary in length, so that an HR-MS’ MAC PDU may represent a variable number of bytes. A MAC PDU

may contain a CRC, as described in 6.3.3.5. MAC PDUs sent on the connection used by

local forwarding shall follow the same format as shown in Figure 20.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048r2, “802.16n System Requirements Document including SARM annex,” July 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16n-11/0024, “Draft P802.16n Draft AWD,” Oct 2011.